3GPP TSG- SA1 Meeting # 107	S1-242142

Maastricht, The Netherlands, 19-23 August 2024	(revision of S1-24xxxx)

# Source:	CATT, China Unicom, China Telecom, KPN, AT&T, AsiaInfo, Novamint, vivo, Honor

# Title:	New WID on Vehicle-Mounted Relays Phase3

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Vehicle-Mounted Relays Phase3

Acronym: VMR_Ph3

Unique identifier:

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | x | x |  |
| No | x |  |  |  |  |
| Don't know |  | x |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
| x | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 840039 | Enhancement for UAVs (EAV) | Rel-17 normative work on UAV, covering the requirements for radio access node on-board UAV (UxNB) |
| 930021 | Stage1 of VMR | Rel-18 Stage1 normative work on Mobile Base Station Relay, covering the requirements of the operation, service continuity, multi-link connectivity, etc. |
| 980018 | Architecture Enhancements for Vehicle-Mounted Relays | Rel-18 Stage2 normative work on VMR, focusing on IAB (Integrated Access and Backhaul) architecture |
| 1020067 | Study on Vehicle Mounted Relays Phase 2 | Rel-19 Stage2 study work on VMR, focusing on MWAB (mobile gNB with wireless access backhaul) architecture |

Dependency on non-3GPP (draft) specification:

3	Justification

Vehicle-mounted relays with wireless backhauling become a promising solution to address the challenges of network deployment and management caused by high densification of access nodes and diversified geographic area (e.g. sparsely populated areas, ocean). It can enable more options to improve cellular coverage and connectivity availability, especially for UEs located inside or near a moving vehicle (e.g. bus, train, UAV, eVTOL, vessel, etc). For example, a vessel-mounted relay using satellite access can enable UEs incapable of satellite access to obtain services from remote home network continuously when the UEs move together with the vessel.

TS 22.125 has highlighted the need to manage and operate radio access node on-board UAV (UxNB) for temporary deployment. And TS 22.261 specifies a series of requirements for Mobile Base Station Relays in configuration, support of multi-link connectivity, mobility, and charging. Accordingly, two types of system architecture (i.e. IAB, MWAB) have been studied in Stage2 individually. However, the use cases related to different types of vehicle-mounted relay’s backhaul links were mentioned in TR 22.839 but not well studied. The requirements about harmonizing the impact of backhaul links with differentiated characteristics (e.g. RAT, QoS, etc.) on the user’s service offering via vehicle-mounted relays are not covered in current 3GPP specification.

When the UEs move together with the vehicle across the areas with different wireless coverage, the vehicular relay will switch access network nodes with different access capabilities (e.g. bandwidth, RAT). The change of backhaul link is assumed to be transparent to served UEs and their serving networks, but in fact the impact on the user experience caused by differentiated QoS of backhaul link exists, and shall be well managed regarding the operator’s policies, e.g. take proper QoS control in UE and its serving network or the relay and its serving network, make UE aware of the impact on user experience such as QoS downgrade or termination for corresponding handling. Moreover, when the vehicular relay can connect to 5GC via more than one backhaul link, e.g. a vessel-mounted relay with one local access and one satellite access, the end-to-end delivery path of UE service will be affected by the selected link of the relay. It’s worthwhile to consider the differentiated characteristics (e.g. QoS, available data networks) of the backhaul links for efficient routing of user traffic with different service needs.

Therefore, it is proposed to enhance 5GS to address additional requirements about QoS management and efficient traffic delivery when vehicle-mounted relays using backhaul links with differentiated characteristics.

4	Objective

The objective of this work item is to specify requirements on the QoS management and efficient traffic delivery when vehicle-mounted relays using backhaul links with differentiated characteristics.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 22.261 | Additional requirements on QoS management and efficient traffic delivery when VMR using backhaul link with differentiated characteristics | TSG#105 |  |

6	Work item Rapporteur(s)

Qing Wan, CATT, wanqing1@cictmobile.com

7	Work item leadership

SA WG1

8	Aspects that involve other WGs

None

9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

CATT

China Unicom

China Telecom

KPN

AT&T

AsiaInfo

Novamint

vivo

Honor