3GPP TSG SA WG 1 Meeting #107 	S1-242169

Maastricht, NL, 19-23 Aug., 2024	(revision of S1-24xxxx)

Source:	OPPO

Title:	Discussion on Rel-18 AIML requirements clean-up

Agenda item:	6.2

Document for:	Discussion

Contact:	Yang Xu
xuyang@oppo.com

Abstract: This contribution provides a proposal clean up to R18 AIML requirements which is not addressed by stage-2 or stage-3.

1. Discussion

The following analysis are provided throughout the all the PIN requirements:

| TS 22.261 (SA1) | TS 23.501, TS23.502, TS23.503, TS23.288 (SA2) TS32.254 (SA5) | Addressed? |
| --- | --- | --- |
| #### 6.40.2.1 Requirements for direct network connection |  |  |
| Based on operator policy, the 5G system shall be able to provide means to allow an authorized third-party to monitor the resource utilisation of the network service that is associated with the third-party. NOTE 1: Resource utilization in the preceding requirement refers to measurements relevant to the UE’s performance such as the data throughput provided to the UE. | ### TS 23.502 clause 5.45 and clause 4.15.6.13 ### Consolidated Data Rate monitoring | Yes |
| Based on operator policy, the 5G system shall be able to provide an indication about a planned change of bitrate, latency, or reliability for a QoS flow to an authorized 3rd party so that the 3rd party AI/ML application is able to adjust the application layer behaviour if time allows. The indication shall provide the anticipated time and location of the change, as well as the target QoS parameters | Not addressed | No Remove it from Rel-18 |
| Based on operator policy, 5G system shall be able to provide means to predict and expose predicted network condition changes (i.e. bitrate, latency, reliability) per UE, to an authorized third party. | TS23.288 Clause 6.9 QoS Sustainability Analytics | Yes |
| Subject to user consent, operator policy and regulatory constraints, the 5G system shall be able to support a mechanism to expose monitoring and status information of an AI-ML session to a 3rd party AI/ML application. NOTE 2: Such mechanism is needed for AI/ML application to determine an in-time transfer of AI/ML model. | Cts23.502 lause 4.15.3.2.3 The AF may subscribe to NEF monitoring events as described in to assist its application AI/ML operations. For example, the AF may subscribe to session inactivity time monitoring event to assist the AI/ML application server in scheduling available UE(s) to participate in the AI/ML operation | Yes |
| 5G system shall be able to provide event alerting to an authorized 3rd party, together with a predicted time of the event (e.g., alerting about traffic congestion or UE moving into/out of a different geographical area). NOTE 3: A 3rd party AI/ML application may use the prediction information to minimize disturbance in the transfer of learning data and AI/ML model data | TS23.503 Clause 6.1.2.7 The AF may request the network to provide a recommended time window for the AI/ML operation using the Planned Data Transfer with QoS (PDTQ) requirements and procedures The requirement is partially addressed, suggested to make some rewording. | Partially Rewording to “5G system shall be able to provide a planed data transfer with QOS to an 3rd party together with a recommended time window. NOTE 3: A 3rd party AI/ML application may use the information to minimize disturbance in the transfer of learning data and AI/ML model data” |
| The 5G system shall be able to expose aggregated QoS parameter values for a group of UEs to an authorized service provider. | TS 23.502 Clause 5.45 and clause 4.15.6.13 The AF may subscribe to QoS Monitoring which may include also Consolidated Data Rate monitoring | Yes |
| The 5G system shall be able to support an authorised 3rd party to change aggregated QoS parameter values associated with a group of UEs, e.g. UEs of a FL group. | TS 23.502 Clause 5.45 and clause 4.15.6.13 The AF may subscribe to QoS Monitoring which may include also Consolidated Data Rate monitoring | Yes |
| Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to expose information (e.g. candidate UEs) to an authorized 3rd party to assist the 3rd party to determine member(s) of a group of UEs (e.g. UEs of a FL group). | TS 23.501 clause 5.46.2 Member UE selection assistance functionality for application operation | Yes |
| #### 6.40.2.1 Requirements for direct network connection |  |  |
| The 5G system shall be able to support collection of charging information for a group of UEs, e.g. UEs of a AI/ML FL group. | TS32.254 Clause 5.1.2 AI/ML support NEF Northbound APIs converged charging encompasses the NEF Northbound APIs specified to support AI/ML-based services: - Member UE selection assistance functionality for application operation; - Planned Data Transfer with QoS (PDTQ) requirements; - Multi-member AF session with required QoS. | Yes |

2. Proposal

SA1 should discuss the proposal above, and agree S1-242168 on a set of requirements to be updated from TS 22.261. Based on the outcome, a correposding CR will be submitted by the rapporteur.