3GPP TSG-SA WG1 Meeting #109 	S1-250669

Athens, Greece, 17-21 February 2025	(revision of S1-250284)

Source:	Huawei

pCR Title:	Update of Use Case 7.5

Draft Spec:	3GPP TR 22.870 v0.1.1

Agenda item:	8.1.3

Document for:	Approval

Contact:	Fangyuan Zhu< zhufangyuan@huawei.com >

Abstract: This document provides some updates for the use case 7.5.

1. Introduction

The aim of this contribution is to update the use case 7.5 by addressing the following Editor’s Notes.

  - Editor’s Note:  the KPI values for this use case is FFS.

2. Reason for Change

The 3GPP TS 22.137 has described the definition of 5G wireless sensing, which enables to get information about characteristics of the environment and/or objects within the environment (e.g., shape, size, orientation, speed, location, distances or relative motion between objects, etc). 3GPP TS 22.137 also has described different performance requirements (e.g., accuracy, resolution, latency, etc.).

5G Wireless sensing: 5GS feature providing capabilities to get information about characteristics of the environment and/or objects within the environment (e.g., shape, size, orientation, speed, location, distances or relative motion between objects, etc) using NR radio frequency signals, which, in some cases, can be extended by information created via previously specified functionalities in EPC and/or E-UTRAN.

The existing KPI values for use cases, such as object detection and tracking in TS 22.137, focus on the positioning and velocity of individual sensing target as whole. The 3GPP R19 does not, however, define whether the sensing service fulfils the performance requirements to determine the richness or characteristics of the target object (e.g. shape, size, type of object / vehicle, etc.). In a growing number of use cases, some of which are given as examples in TS 22.137, such as detection, recognition and tracking for “UAVs, smart home, V2X, factories, railways, public safety, etc.”, sensing results beyond positioning and velocity estimation are required. For example, in the scenario of vehicle detection and tracking, sensing results derived from Figure 1 may satisfy sensing KPIs of positioning and velocity estimation in Rel-19, but these results are irrelevant and insufficient to determine the characteristics of the vehicle, e.g. differentiate whether it is a passenger vehicle or truck.

Figure 1 Detection and Tracking of Passenger vehicle’s Positioning/Velocity in R19

Building on TS 22.137 as a framework, and leveraging the sensing KPI table for consistency, further consideration in SA1 is required on how to compare sensing results of reconstruction with respect to the corresponding ground truth. This is achievable by applying the sensing KPIs simply to components or constituents of the objects, i.e. segments/parts of the object, where the part is comparable to the corresponding spatial resolution.  For example, Figure 2 shows sensing results conceptually representing the closeness of the reconstructed object to the real object of the same sensing target in Rel-20.

Figure 2 Closeness of the reconstructed object to the real object in R20

From the service perspective, therefore, the 3GPP system is expected to offer new/updated KPIs to quantize the reconstruction of a target object. Based on further analysis in Annex and for the sake of consistency/simplicity of ISAC KPIs across use cases, it is proposed that a sensing target in the corresponding KPI table refers to a [segment/part] of the target object to be detected and/or tracked, whereas the size of each [segment/part] is comparable to corresponding spatial resolution. The percentage of missed detection/false alarm represent missed/falsely reconstruction of [segments/parts] of the object statistically. In summary, the closeness of environmental object reconstruction is categorized jointly in the sensing result by the estimation of positioning/velocity accuracy, missed detection and false alarm over [segments/parts] of a target object.

Figure 3: Segments/parts of a target object to be detected and/or tracked

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.1.1.

* * * First Change * * * *

7.5	Use Case on Environment Object Reconstruction

7.5.1	Description

Environmental object reconstruction offers significant potential with great societal and business impact, representing opportunities across wide range of sectors, from smart factories, homes and transportation to components of/entire smart cities and countries. The 3GPP ISAC provides a non-invasive dual-functionality of communication and sensing data collection of environmental features for reconstruction. This sensing data collection is expected to unlock new services and support enhanced performance, deployment utilization, energy, and spectral efficiencies due to the nature of ISAC and also its wide-area coverage and radio-signal availability. For static environmental objects, such as outdoor building/city construction and indoor machinery/wall, 3GPP wireless sensing is effective to provide a wealth of low-cost environmental information for both wide-areas and certain areas. Additionally, for monitoring dynamic targets of interest, such as vehicles, this is essential for enhancing environmental perception, especially in scenarios where non-line-of-sight (NLOS) conditions or poor visibility may limit traditional sensing methods.

The use case of environmental object reconstruction in 3GPP demands finer characterization of surrounding targeted objects based on wireless sensing signal to facilitate industrial innovation. Some applications include:

-	Smart Transportation: The impact of autonomous driving is expected to be significant in terms of safety and comfort, and potentially high efficiencies with respect to traffic, logistics, energy, etc. Such application requires advanced knowledge of moving target detection and its trajectory with detection-to-track association [12], within a multi-object tracking context, including perception of micro-features of the surrounding objects such as micro-Doppler effects, precise classification, and accurate dimensions/orientations, etc. Rough localization of these objects is insufficient for ensuring transport safety and public confidence. Therefore, environmental object reconstruction by the 3GPP wireless sensing is to provide assistance for interpreting the complex traffic scenario and making rapid decisions.

-	Smart City: a smart city can demand survey/digital twinning for a component/system of an entire city, which has motivated a number of smart city initiatives [13] to capture dynamic nature of society. Initially based on cameras, these digital twin models, can be greatly enhanced by environmental object reconstruction enabled by 3GPP wireless sensing

-	Smart Home: there are increasing interests for innovative applications of smart home, e.g. for fall detection, provided with better sensing privacy protection and reliability. Environmental object reconstruction at home is expected for finer sensing information of home objects in order to improve service stability for better consumer experience.

Typical environmental objects to be reconstructed can include the following:

Table 7.5.1-1: Representative dimensions of an environment object

| Object Type | Dimensions |
| --- | --- |
| Building | Approximately ~400 m x ~200 m x ~20 m (L x W x H), static |
| Vehicle | Truck: 13 m x 2.6 m x 3 m (LxWxH), up to 120km/h Passenger vehicle: 5mx2mx1.6m (LxWxH), up to 120km/h |

The 3GPP ISAC is expected to offer more detailed characteristics of an environment object in the spatial domain. The sensing target in the corresponding KPI table refers to a [segment/part] of the target object to be detected and/or tracked, whereas the size of each [segment/part] is comparable to corresponding spatial resolution. The percentage of missed detection/false alarm represent missed/falsely reconstruction of [segments/parts] of the object statistically. In summary, the closeness of environmental object reconstruction is categorized jointly in the sensing result by the estimation of positioning/velocity accuracy, missed detection and false alarm over [segments/parts] of a target object.

Figure 7.5.1-1: Segments/parts of a target object to be detected and/or tracked

7.5.2	Pre-conditions

Map Provider A is a third-party service provider which can render and virtualize detected/tracked surrounding environmental objects within its application, offer real-time 3D virtualization of objects (e.g. via 3D glasses), and display alerts for object-related warnings and information. Examples of alerts include "a car is approaching from the left street corner in 3 seconds".

Good partnership and cooperation are established between Map Provider A and Mobile Operator B in City C. Requested by Map Provider A for sensing service, suitable sensing transmitter and/or sensing receiver deployed in City #C are selected by Mobile Operator B to constantly sense environment objects of City #C including building and vehicle. The sensing signal emitted from sensing transmitter arrives at an environment object whose micro-objects will reflect/diffract the signal to be detected by selected sensing receivers.

NOTE:	For the ease of elaboration, base station or UE is acting as sensing transmitter and/or sensing receiver. Other sensing modes are not excluded and can be useful for environmental object reconstruction.

Charlie is a subscriber of Mobile Operator B and also a subscriber of Map Provider A for real-time 3D virtualisation of his surrounding objects. Charlie would like to navigate unfamiliar city streets while wearing 3D glasses, which display surrounding objects in real-time, for his interest and safety.

7.5.3	Service Flows

Figure 7.5.3-1

1. Charlie is a tourist, who is driving a car to enjoy the view around her in City #C. He would like to navigate unfamiliar city streets while wearing 3D glasses, which display surrounding objects in real-time, for her interest and safety. Map Provider A initiates a sensing request to Mobile Operator B for the latest information of environment reconstruction in the vicinity of Charlie, who is the subscriber of both Map Provider A and Mobile Operator B. The sensing objects, required by Map Provider A, include vehicles, city architecture, etc., around Charlie up to a certain range.

2. Mobile Operator B selects and configures the sensing transmitter and sensing receiver, and any applicable non-3GPP sensors, to facilitate the sensing operations. Both 3GPP and non-3GPP sensing data will be collected, aggregated, and processed by Mobile Operator B's network in accordance with environment reconstruction requirements.

3. Based on environment reconstruction requirements, Mobile Operator B is expected to preform further sensing operations dedicated to each individual object, to provide fine characteristic, in order to determine object type/dimension, monitor micro-motions like car turning, etc. Corresponding sensing operations may require cooperative and dedicated sensing resource allocations across 3GPP system for Charlie. Thereafter sensing results of each object are delivered to Map Provider A.

4. The sensing results generated by Mobile Operator B, with more detailed characteristics of environment objects, are exposed to Map Provider A. This data enables Map Provider A to update Charlie's localized map, reflecting changes such as ongoing construction of university campus, shapes/dimensions of targets, and real-time tracking of movement direction/speed etc.

5. Charlie receives a real-time hazard (e.g. a car is approaching from the left street corner in 3 seconds) display from Map Provider A virtualized in his glass, tailored to Charlie's trajectory. This information is transmitted through Mobile Operator B's network, adhering to pre-defined latency requirements.

6. Luckily Charlie has a chance to visit City# C again. Mobile Operator B keeps detecting and tracking ongoing renovation by sensing for local university campus including new classroom buildings. During his visit, such renovation is completed roughly. By partnering with Mobile Operator B, Map Provider A can integrate and reconstruct the buildings into the real-time 3D map. Charlie decides to pay a visit because of his interest and unique building design.

7.5.4	Post-conditions

User Charlie can safely travel in a new city with enjoyable information associated with his surroundings and necessary travel advice.

7.5.5	Existing features partly or fully covering the use case functionality

3GPP TR 22.837 [9] has described use cases to monitor micro doppler effect by ISAC caused by chest rise/fall during sleeping when detecting the doppler. The sensing results are represented by human respiration rate.

In this use case, 3GPP ISAC is expected to detect and track more comprehensive characteristics of individual environmental object, e.g. for a building, vehicle, robot, etc., with sufficient and accurate sensing information per object type.

7.5.6	Potential New Requirements needed to support the use case

[PR 7.5.6-1] The 6G system shall be able to provide sensing service by detecting and tracking the characteristics of individual environmental object as described in the following table 7.5.6-1.

Table 7.5.6-1

| Scenario | Object Type | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Horizontal [m] | Vertical [m] |  | Horizontal [m/s] | Vertical [m/s] |  | Spatial resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |  |  |  |  |
| Environment object reconstruction |  | Building (Note 1) |  | 95 |  | [0.5-5] (Note 2) | [0.5-5] (Note 2) |  | N/A | N/A |  | [0.5] | N/A |  | [10000-600000] |  | [10-60] |  | [1-5] |  |  | [1-5] |
|  |  | Vehicle (Note 1) |  | 95 |  | 0.5 | 0.5 |  | 1.5 | N/A |  | [0.5] | [0.5] |  | 100 |  | 0.1 |  | 5 |  |  | 5 |

Note 1: a sensing target in above KPI refers to a [segment/part] of the target object to be detected and/or tracked, whereas the size of each part is comparable to corresponding spatial resolution of object reconstruction. The percentage of missed detection/false alarm represent missed/falsely reconstruction of parts of the object statistically.

Note 2: Considering a variety of dimensions, shapes, and functionalities of urban buildings, a range of KPI values are needed to measure and provide the flexiability of reconstruction accuracy.

* * * End of Changes * * * *