3GPP TSG SA WG 1 Meeting #110 	S1-252612

19-23 May 2025, Fukuoka, Japan	(revision of S1-252164)

Source:	Hytera, China Mobile, China Telecom, China Unicom, CSCN

pCR Title:	Update of use case on Seamless Connectivity for 6G-enabled Mission Critical Services

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.8

Document for:	Approval

Contact:	Chuanchuan leng  Chuanchuan.leng2@hytera.com

Abstract: This contribution provides the update of use case enabled by 6G system to provide seamless connectivity for mission critical services supporting disaster response and relief operations.

1. Introduction

As public safety increasingly relies on mobile networks, resilient and always-available connectivity is critical. While 4G and 5G support Mission Critical Services, 6G is needed to close gaps in coverage, availability, and interoperability—especially during large-scale disasters. This use case highlights how 6G can enhance emergency response through seamless network integration, improved service reliability, and support for emerging models like NaaS.

2. Reason for Change

Introduce a new use case on Seamless Connectivity for 6G-enabled Mission Critical Services.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR22.870 v0.2.1.

* * * First Change * * * *

11.x	Use case on Seamless Connectivity for 6G-enabled Mission Critical Services

11.x.1	Description

As public safety communities and society at large grow increasingly reliant on mobile networks, building a resilient foundation for critical communications is essential—connectivity is the lifeline. According to TCCA, mission-critical communications must consistently meet core requirements: coverage, availability, resilience, performance, and scalability, ensuring 24/7 instant and reliable connectivity, particularly during life-saving emergency operations.

Thanks to 3GPP standardisation, Mission Critical Services (MCPTT, MCVideo, MCData) over 4G and 5G are now well-defined. With regulatory support and operator deployments, LTE and 5G have become globally recognised, all-inclusive platforms for delivering mission-critical services. Concurrently, continued hardening of MNO networks is essential to meet public safety-grade (PSG) requirements such as 99.999% availability and robust cybersecurity.

6G is expected to bridge existing gaps and enhance capabilities to support 6G-enabled Mission Critical Services. Its ubiquitous and resilient infrastructure will be essential for search and rescue operations in remote or disaster-affected areas where terrestrial networks are unavailable or damaged, ensuring service availability, reliability and seamless positioning. Deep integration of different network topologies (e.g., NT and NTN) will facilitate seamless transitions and interoperability among agencies across all phases of emergency management, particularly in large-scale disaster responses involving multiple agencies and disciplines coordination.

However, current 6G use cases lack a comprehensive view of 6G-enabled Mission Critical Services. This use case aims to fill that gap by illustrating a disaster response scenario enhanced by the 6G system, strengthening emergency responders’ ability to save lives, protect assets, and reduce environmental impact. Supporting this use case can also reinforce MNOs’ confidence in delivering Networks as a Service (NaaS) with fully shared PSBNs for mission-critical services, while enabling advanced capabilities such as Sensing as a Service (SaaS) model in the 6G era.

Reference and Background:

ITU-R FRAMEWORK FOR IMT-2030[x1]: REVIEW AND FUTURE DIRECTION by NGMN Alliance (Feb 2024) points out that IMT-2030 should not compromise existing services, for example, voice, existing public safety services. Further national/regional regulatory requirements should be delivered with the launch of IMT-2030. Interworking between IMT-2030 and non-IMT systems will need to be embraced, and standards for mobile networks should be global, reﬂecting industry consensus.

At the international level, advancing disaster radiocommunication supporting prevention, mitigation, and relief has become a key priority for the ITU. The Public Protection and Disaster Relief Liaison Rapporteur [x2] (R23-WP5A-C-0182!!MSW-E) and Question ITU-R 209-7/5 [x3] (Use of the Mobile, Amateur, and Amateur-Satellite Services in Support of Disaster Radiocommunications) stress the need for collaborative studies across ITU-R (including WPs 5A and 5D), ITU-T, and ITU-D to address the technical, operational, and procedural aspects of disaster communications.

In Europe, the EU Critical Communication System (EUCCS) is a European Commission-led initiative aimed at developing a secure, interoperable, and resilient broadband communication system for public safety organisations and emergency services across the EU by 2030. Built upon 3GPP-standizised mission-critical services, EUCCS will enable seamless cross-border communication for emergency responders, enhancing Europe’s crisis response and operational mobility.

In China, one of the world’s most disaster-prone nations, has launched a range of initiatives to strengthen its emergency communication infrastructure, including the rollout of a second nationwide Professional Digital Trunking (PDT) network (i.e., e-PDT) dedicated to disaster relief. In December 2024, the Ministry of Industry and Information Technology (MIIT), together with 13 other government agencies, issued guidelines[x4] to further enhance emergency communication capabilities, especially in extreme disaster scenarios. The strategy emphasises inter-agency coordination and infrastructure modernisation, with a focus on integrating satellite systems, PDT networks, private networks, and MNO infrastructure—laying the groundwork for a unified national emergency communication framework.

11.x.2	Pre-conditions

- The UE (e.g., handheld device) is operated by the disaster responders, hosts MCX client and is registered with MCX service in the 6G system.

- The UE supports directly connecting with satellite access.

- The UE supports access to Land Mobile Radio(LMR) networks (e.g., PDT), by integrating LMR technology (e.g., hybrid devices).

- The UE is equipped with radio receivers capable of receiving GNSS signals.

- The UE supports Prose-enabled UE relay or remote UE capability and/or MANET routing, capable of forming a mobile ad-hoc network with other UEs.

Note: The control room solution in the Emergency Operation Centre (EOC), which may be based on 3GPP technologies, non-3GPP technologies, or a combination of both, is out of scope for this use case.

11.x.3	Service Flows

Figure 11.x.3-1 6G-enabled MCX service with seamless connectivity for large-scale disaster response

- The Emergency Operation Center (EOC) receives alerts of an earthquake striking Jiuzhaigou, formulates a response strategy based on situational awareness, and initiates the dispatch resources.

- Upon receiving instruction via LMR networks (e.g., through call-out functionality), Rescue Team A at a nearby station swiftly mobilises, gathers necessary equipment, including handheld devices, and departs for the scene.

- En route, the Incident Commander (IC) remains connected with dispatchers to view comments and accessing critical data (e.g., weather updates) via MCX application across MNO-1 and MNO-2 networks.

- Built-in navigation tools assist first responders in finding optimal routes through traffic, while real-time location tracking in the control room enables dispatchers to monitor all personnel and vehicles, ensuring swift dispatch and enhanced safety.

- As the EOC gains further situational awareness of the disaster’s scale, the response strategy is updated, and additional rescue team B and C are dispatched to the scene through LMR networks.

- Upon arriving in a remote mountain area with no Terrestrial Network (TN) or access routes, the IC’s handheld device automatically connects to satellite (NTN) access, enabling uninterrupted communication with dispatchers (e.g., via private call) for real-time updates.

- Based on the operational strategy, a UAV-based (or HAPS-based) relay platform is deployed.  IC then coordinates on-scene rescue team A to begin victim search operations, while their handheld devices switch (automatically or manually) to off-network, mesh-enabled mode. The established infrastructure-less mobile ad-hoc network, extended by UAV relay node, enhances local connectivity and coverage. IC’s UE and/or UAV, acting as a relay gateway via satellite, enable communication between on-scene responders and dispatchers.

- Meanwhile, as the focal point of communication, the IC continues to coordinate team B and C are en route and connected to the MNO-2 network. Group communication spans satellite, TN and mobile ad-hoc network access, enabling seamless interoperability across agencies, jurisdictions and disciplines.

- Upon arrival, Teams B and C transition (manually and automatically) from TN access (on-network) to mobile ad-hoc network access (off-network), maintaining communication with Team A and IC for coordinated rescue operation.

- Once the crisis is contained, teams coordinate with other agencies, utilities, and local authorities to support organised recovery efforts. After task completion, they return to base, debrief, and provide feedback.

- Throughout all response phases—from call-out to return—reliable positioning ensures continuous personnel tracking and situational visibility, even in scenarios where GNSS signals are compromised.

11.x.4	Post-conditions

Thanks to the support of 6G systems, the MCX service system, and a 6G-enabled MCX hybrid device, first responders can communicate seamlessly across different networks without interruptions, ensuring fast and efficient mission execution.

11.x.5	Existing features partly or fully covering the use case functionality

Clause 6.17.2 of TS 22.280 defines requirements for interworking between MCX service systems. This use case proposes that the 6G system shall continue to support MCX services while ensuring service continuity during UE mobility across different MNO networks, as well as between PLMNs and NPNs. This capability is essential for seamless MCX operations in cross-border coordination, network sharing, and co-construction scenarios—not only in the early stages of 6G deployment but throughout the entire 6G era—to enhance service availability and coverage.

Clause 6.18.3 of TS 22.179 defines requirements for MCPPT interworking with non-3GPP PTT systems, including P25 and TETRA. However, it doesn’t cover MCPTT interworking with PDT in this case, which is particularly important to recently established nationwide e-PDT network in China.

TR 23.700-03 (Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS) Phase 3) by SA2 defines the use of MANET protocol (IETF RFC 7181) as the mobile ad-hoc routing protocol for normative support of multi-hop UE-to-UE relay, where relay UE have a collocated MANET routing functionality. These advancements enable ProSe-capable MCX UE form a mobile Ad-hoc network with mess connectivity, addressing a key requirement for seamless mobility on the scene in this use case. However, the clarification is still required for alignment and consistency across WGs.

In 5G, Clause 6.46.7 of TS 22.261 specifics requirements for 5GS to support relay UE with satellites access. However, support for relay UEs using UAVs and HAPS access are not specified, which is essential for extending communication range and enhancing local connectivity in this use case.

Clause 7.12 and 7.13 of TS 22.280 specify a set of requirements for MCX services streaming for ProSe UE-to-UE relay, UE-to-Network relay and off-network operation, as well as switching to off-network MCX services; and TR 22.887 also studies use cases and requirements for 5G satellite access Phase 4, including mission-critical services. However, they do not address MCX service continuity during UE transitions between TN access and NTN access (e.g., UAVs, HAPS, and satellites).

In 5G, Clause 6.27.2 of TS 22.261 defines requirements for single and hybrid positioning, including MCX UE use of 5G positioning services. However, it does not specify network-assisted GNSS via satellite access in scenarios where terrestrial networks are unavailable in order to improve the resiliency and availability.

11.x.6	Potential New Requirements needed to support the use case

[PR 11.X.6-1]:  The 6G system shall support MCX service continuity during UE mobility  across inter-PLMN (e.g., through indirect network sharing) , as well as between PLMN and NPN (e.g., PSBN), subject to regulatory requirements, inter-operator agreements and capabilities (e.g., agreed QPP parameter settings).

NOTE:   Public safety broadband network (PSBN) is a network generally owned and operated by a public safety network operator and specially designed for public safety users (e.g., police, ambulance, fire service), having their own PLMN identity.

[PR 11.X.6-2]:  The 6G system shall enable MCPTT service interworking with non-3GPP PTT systems that are compliant with GB/T 34991/34992-2017 PDT standards.

Editors’ Note: The detailed requirements for this interworking (e.g., gap analysis based on the current IWF solution) are FFS.

[PR 11.X.6-3]: The 6G system shall minimize interruption to ongoing MCX service during UE mobility across different access (e.g., terrestrial access and satellite access), and/or when MCX services switch between on-network and off-network operations with or without UE relay through UAVs, HAPS, and Satellites access.

NOTE:	The requirement above may include also Pro-enabled access, where relay UEs are equipped with co-located routing functionality, capable of maintaining connectivity in dynamic and infrastructure-less environments.

[PR 11.X.6-4]: The 6G system shall enable an MCX UE to determine its position using hybrid positioning methods, including network-assisted GNSS via satellite access, to improve availability and resiliency, and to convey its position information to the MCX server, triggered by an event, or periodically.

* * * Next Change * * * *

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	QIAN LI et all: "6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies" in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022.

[3]	GSMA: "Going green: benchmarking the energy efficiency of mobile networks (Second edition)", Feb 2023.

[4] 	3GPP TR 32.972: "Study on system and functional aspects of energy efficiency in 5G networks".

[5]	3GPP TR 23.700-66: "Study on Energy Efficiency and Energy Saving".

[6] 	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[7]	5GAA: "5GAA MRP input to 3GPP Stage 1 workshop on IMT 2030 Use Cases".

[8]	5GAA: Technical report "Evolution of vehicular communication systems beyond 5G".

[9] 	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication".

[10]	SAE J3016 202104: "Levels of Driving Automation".

[11]	G. Liu, L. Ma, Y. Xue, et al, "SensCAP: A Systematic Sensing Capability Performance Metric for 6G ISAC," IEEE Internet of Things Journal, vol. 11, no. 18, pp. 29438–29454, 2024.

[12]	Igal Bilik, Oren Longman, Shahar Villeval, and Joseph Tabrikian, "The Rise of Radar for Autonomous Vehicles", IEEE signal processing magazine, Volume 36, Issue 5, September 2019.

[13] 	https://knowledge-base.inspire.ec.europa.eu/publications/technical-guidance-implementation-inspire-dataset-and-service-metadata-based-isots-191392007_en.

[14]	3GPP TS 22.261: "Service requirements for the 5G system".

[15] 	Wang et.al, "Effect of Frame Rate on User Experience, Performance, and Simulator Sickness in Virtual Reality", IEEE Transaction on Visualization and Computer Graphics. 2023 Feb 22:PP. doi: 10.1109/TVCG.2023.3247057.

[16] 	https://en.wikipedia.org/wiki/Adreno.

[17] 	https://www.labtekno.com/snapdragon-8-gen-2-vs-apple-a16-bionic/.

[18] 	Algorithms and tools for GPU-accelerated Computer Generated Holography | Ewireless Research Group (https://www.ewireless.eng.ed.ac.uk/projects).

[19] 	https://phlearn.com/tutorial/how-many-exposures-hdr/.

[20]	https://photographylife.com/compressed-vs-uncompressed-vs-lossless-compressed-raw.

[21]	https://www.dotphoton.com/products/jetraw-core.

[22] 	https://www.lambdatest.com/learning-hub/web-performance-testing.

[23] 	Static 3D triangle mesh compression overview, Marcos Aviles, Grupo de Tratamiento de Imágenes, Universidad Politécnica de Madrid, Spain, 15th IEEE International Conference on Image Processing.

[24] 	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[25]	EUSPA: "Report on Aviation and Drones – User Needs and Requirements", 2023.

[26]	3GPP TR 36.763: "Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)".

[27]	Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond".

[28]	3GPP TS 22.156: "Mobile Metaverse Services".

[29] 	ITU-R Recommendation M.2160-0 (11/2023): "Framework and overall objectives of the future development of IMT for 2030 and beyond" (https://www.itu.int/rec/R-REC-M.2160-0-202311-I/en).

[30] 	3GPP TS 23.502: "Procedures for the 5G System".

[31]	Hexa-X-II, "Deliverable D1.2: 6G Use Cases and Requirements", December 2023.

[32]	https://www.easa.europa.eu/en/what-is-uam.

[33]	https://www.autoflight.com/en/air/.

[34]	"Low altitude economy development white paper (2.0) All digital solutions", International digital economy academy (IDEA), 2023.

[35]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP; Stage 1".

[36]	https://www.unmannedairspace.info/latest-news-and-information/shenzhen-reaches-record-number-of-drone-operations-with-600000-flights-in-2023/.

[37]	"C-V2X Use Cases and Service Level Requirements Volume II", 5GAA White Paper, 2023.

[38]	https://www.flyeye.io/drone-acronym-daa/.

[39]	Report ITU-R M.2516-0, "Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond ". (https://www.itu.int/dms_pub/itu-r/opb/rep/R-REP-M.2516-2022-PDF-E.pdf).

[40]				3GPP TR 33.841: "Study on the support of 256-bit algorithms for 5G".

[41]	3GPP TS 35.246: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 1: algorithm specification".

[42]	3GPP TS 35.247: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 2: implementation test data".

[43]	3GPP TS 35.248: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 3: design conformance test data".

[44]	3GPP TR 22.883: "Feasibility Study on Energy Efficiency as service criteria Phase 2".

[45]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[46]	3GPP TS 38.211: "NR; Physical channels and modulation".

[47]	3GPP TS 36.305: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[48]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[49]	3GPP TS 22.856: "Study on Localized Mobile Metaverse Services".

[50]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[51]	3GPP TS 26.565: "Split Rendering Media Service Enabler".

[52]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[53]	3GPP TS 22.179: “Mission Critical Push to Talk (MCPTT); Stage 1”.

[54]	3GPP TS 22.280: “Mission Critical Services Common Requirements (MCCoRe); Stage 1”..

[55]	3GPP TS 22.281: “Mission Critical (MC) video”.

[56]	3GPP TS 22.282: “Mission Critical (MC) data”.

[57]	3GPP TS 22.262: “Message service within the 5G System (5GS); Stage 1”.

[58]	3GPP TS 22.101: “Service aspects; Service principles”.

[59]	3GPP TS 22.173: “IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1”.

[60]	3GPP TS 22.011: “Service accessibility”.

[61]	3GPP TS 22.071: “Location Services (LCS); Service description; Stage 1”.

[62]	3GPP TS 22.268: “Public Warning System (PWS) requirements”.

[63]	3GPP TS 22.153: “Multimedia priority service”.

[64]	3GPP TS 22.104: “Service requirements for cyber-physical control applications in vertical domains”.

[65]	3GPP TS 22.185: “Service requirements for V2X services”.

[66]	3GPP TS 22.186: “Service requirements for enhanced V2X scenarios”.

[67]	3GPP TS 22.263: “Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA)”.

[68]	3GPP TS 22.115: “Service aspects; Charging and billing”.

[69]	3GPP TS 22.289: “Mobile communication system for railways”.

[70]	3GPP TS 22.468: “Group Communication System Enablers (GCSE)”.

[71]	3GPP TS 22.368: “Service requirements for Machine-Type Communications (MTC); Stage 1”.

[72]	3GPP TR 22.949: “Study on a generalized privacy capability”.

[73]	3GPP TR 33.849: “Study on subscriber privacy impact in 3GPP”.

[74]	3GPP TS 22.278: “Service requirements for the Evolved Packet System (EPS)”.

[75]	ETSI GR SAI 004 (V1.1.1): "Securing Artificial Intelligence (SAI); Problem Statement".

[76]	ETSI GR SAI 005 (V1.1.1): "Securing Artificial Intelligence (SAI); Mitigation Strategy Report".

[77]	3GPP TR 28.915: "Study on management aspects of Network Digital Twin".

[78]	3GPP TS 23.527: “5G System; Restoration procedures”.

[79]	3GPP TR 29.866: “Study on IMS Disaster Prevention and Restoration Enhancement”.

[80]	Enabling Mobile AI Agent in 6G Era: Architecture and Key Technologies; https://dl.acm.org/doi/abs/10.1109/MNET.2024.3422309

[81]	Wang L, Ma C, Feng X, et al. A survey on large language model based autonomous agents[J]. Frontiers of Computer Science, 2024, 18(6): 186345.

[82]	Hong S, Zheng X, Chen J, et al. Metagpt: Meta programming for multi-agent collaborative framework[J]. arXiv preprint arXiv:2308.00352, 2023.[xx]	ISO/IEC 22989: "Information technology — Artificial intelligence — Artificial intelligence concepts and terminology"

[83] 	GSMA Foundry, “Case Study: Delivering Real-Time Translation”,2023, Reference links for https://www.gsma.com/get-involved/gsma-foundry/5g-new-calling/

[84] 	GSMA Foundry, “Case Study: Remote damage assessment ”,2023, Reference links for https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/remote-damage-assessment/

[85]	 GSMA Foundry, “Case Study: Visualised Video-Calling”,2023, Reference links for  https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/visualised-voice-calling/

[86] 	GSMA Foundry, “Case Study: Adding New Value for Voice Call”,2024, Reference links for https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/adding-new-value-to-voice-calls/

[87]	United Nations Sustainable Development Goals (https://sdgs.un.org/goals).

[88]	United Nations Global Digital Compact (https://www.un.org/global-digital-compact/sites/default/files/2024-09/Global%20Digital%20Compact%20-%20English_0.pdf).

[89]	Zhang Z, Wang S, Hong Y, et al. Distributed dynamic map fusion via federated learning for intelligent networked vehicles[J]. arXiv preprint arXiv:2103.03786, 2021

[90]	AECC: Operational Behavior of a High Definition Map Application – White Paper Version 1.0.0

[91]	 https://spectrum.ieee.org/boats-should-be-sleek-but-only-up-to-a-point

[92]	https://www.marketsandmarkets.com/Market-Reports/evtol-aircraft-market-28054110.html

[93]	https://www.fortunebusinessinsights.com/autonomous-vehicle-market-109045

[94] 	5G-ACIA White Paper, “Use Cases and Requirements for Integrated Sensing and Communication (ISAC) in Connected Industries and Automation”

[95]	ETSI GR ISC-001 V1.0.0 group report, “Integrated Sensing And Communications (ISAC); Use Cases and Deployment Scenarios”

[96]	3GPP TR 37.885 Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR.

[97]	3GPP TR 22.865: “Study on satellite access - Phase 3”.

[98]                   	3GPP TR 22.836: "Study on Asset Tracking Use Cases".

[99]	CHINA AIRSPACE CLASSIFICATION AND PILOT UPDATE https://www.icao.int/APAC/Meetings/2024%20ATMSG12/IP09%20China%20Airspace%20Classification%20and%20Pilot%20Update.pdf#search=AIRSPACE%20CLASSIFICATION.

[100]	3GPP TS 38.171: "NR; Requirements for support of Assisted Global Navigation Satellite System (A-GNSS)".

[101] 	https://cloud.tencent.com/developer/article/1654264.

[102] 	ITU G.114: One-way transmission time.

[103]	S4aV200634: "Traces and Configurations for VR2, CG and AR2".

[104]	ITU-T FG NET-2030, "Representative use cases and key network requirements for Network 2030", 2020.

[105]	Ian F. Akyildiz, Hongzhi Guo, "Holographic-type communication: A new challenge for the next decade", ITU Journal on Future and Evolving Technologies, Volume 3 (2022), Issue 2, Pages 421-442.

[106]	X. Xu, Y. Pan, P. P. M. Y. Lwin and X. Liang, "3D holographic display and its data transmission requirement," 2011 International Conference on Information Photonics and Optical Communications, Jurong West, Singapore, 2011, pp. 1-4, doi: 10.1109/IPOC.2011.6122872.

[107]	P. Ekdahl, T. Johansson, A. Maximov, and J. Yang, "A new SNOW stream cipher called SNOW-V", IACR Transactions on Symmetric Cryptology, vol. 2019, no. 3, pp. 1-42, 2019. doi: 10.13154/tosc.v2019.i3.1-42.

[108]	P. Ekdahl, T. Johansson, A. Maximov, and J. Yang, "SNOW-Vi: an extreme performance variant of SNOW-V for lower grade CPUs ", WiSec '21, pp. 261 – 272, 2021. https://doi.org/10.1145/3448300.3467829

[109]	IEC 62056-6-1:2023 Object Identification System (OBIS).

[110]	IEC 62056-6-2:2023 COSEM interface classes.

[111]	IEC 62056-5-3:2023 DLMS/COSEM application layer.

[112]	Hexa-X-II, “Deliverable D1.4, 6G Value, Requirements, and Ecosystem”, April 2025.

[113] 	ITU-T Recommendation Y.3090 (02/22): "Digital twin network - Requirements and architecture" (https://www.itu.int/rec/T-REC-Y.3090-202202-I).

[114]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[115] 	Urban Air Mobility, https://www.airbus.com/en/innovation/low-carbon-aviation/urban-air-mobility, AIRBUS, accessed in May 2024.

[116] 	Urban Air Mobility (UAM) Concept of Operations, https://www.faa.gov/air-taxis/uam_blueprint, Federal Aviation Administration of U.S. Department of Transportation, Aug. 2023.

[117] 	Urban Air Mobility, https://rotorcraft.arc.nasa.gov/Research/Programs/UrbanAirMobility.html, U.S. NASA, 2024.

[118] 	NASA Urban Air Mobility (UAM) Reference Vehicles, https://sacd.larc.nasa.gov/uam-refs/, U.S. NASA, Jan. 2024

[119] 	Future Mobility, https://www.hyundai.com/worldwide/en/newsroom/detail/0000000093.html, Hyundai Motor Company, Jul. 2022.

[120]	LG Uplus, Near-Future Vertical Applications in Korea: K-UAM, https://nextgalliance.org/wp-content/uploads/2024/05/Near-future-Vertical-Applications-in-Korea-K-UAM_6GF_NGA_Joint_Workshop_JKim_final.pdf, Next G Alliance / Korea 6G Forum Joint Workshop

[121]	A. Baltaci, et. al., “A Survey of Wireless Networks for Future Aerial Communications (FACOM),” IEEE Communications Surveys & Tutorial, Vo. 23, No. 4, Q4 2021. https://ieeexplore.ieee.org/document/9508366.

[122]	3GPP TR 22.887: "Feasibility Study on satellite access – Phase 4".

[123]	Next G Alliance Report “6G Applications and Use Cases”: Use Case 4.6 – High-Speed Wireless Connection in Aerial Vehicle for Entertainment Service.

[124]	Topal, O. A., Schupke, D., Björnson, E., & Cavdar, C. ”Optimal Joint Access Point Placement and Resource Allocation for Indoor mmWave Communications”, ICC 2023, Rome, Italy.

[125]	Hofmann, S.; Duhovnikov, S.; Schupke, D., “Massive Data Transfer from and to Aircraft on Ground: Feasibility and Challenges,” IEEE Aerospace and Electronic Systems Magazine, vol. 36, iss. 5, May 2021.

[126] 	NASA. UTM: Air traffic management for low-altitude drones[EB/OL] (2015-10) (https://www.nasa.gov/sites/default/files/atoms/files/utm-factsheet-11-05-15.pdf)

[127] 	SESAR. U-space blueprint[EB/OL]. (2017-06-09). (https://www.sesarju.eu/sites/default/files/documents/reports/U-Space%20Blueprint%20brochure%20final.pdf)

[128] 	AOPA China. Regulations for the operation of light and small UAVs [EB/OL] (2018-04-19). (http://www.aopa.org.cn/usr/1/upload/file/20180419/15241267234030958.pdf)

[129] 	HIROYUKI U. UTM project in Japan[EB/OL]. (2017-06-26)[2025-01-20]. (https://gutma.org/montreal-2017/wp-ontent/uploads/sites/2/2017/07/UTM-Project-in-Japan_METI.pdf).

[130] 	G. Wagner and H. Choset, “A complete multirobot path planning algorithm with performance bounds,” Proc. In IEEE/RSJ International Conference on Intelligent Robots and Systems (IROS), San Francisco, CA, USA, Sept. 2011. Online Link.

[131]	B. Hu, S. Xu, and Z. Cao, “Multi-Robot Path Planning for Each Robot with Several Jobs in a Single Trip,” IFAC-PapersOnline, Volume 53, Issue 5, 2020, Pages 279-284. Online Link.

[132]	S. Lin, A. Liu, J. Wang and X. Kong, “A Review of Path-Planning Approaches for Multiple Mobile Robots,” Section of Automation and Control Systems, MDPI Machines, 10(9), 773, Sept. 2022. Online Link.

[133]	A New Approach to Data Sharing - Open data sharing and collaboration for data, analytics and AI, DataBricks eBook 3rd Ed. Online Link.

[134]	K.-D. Lee, “A Smart Network of Service Robots: Technical Challenges and Design Considerations,” IEEE Communications Magazine, pp. 28-34, vol. 59, Iss. 8, Aug. 2021. Online Link.

[135]	Network-Enabled Robotic and Autonomous Systems, Next G Alliance, May 2023. Online Link.

[136]	3GPP TR 22.916: "Study on Network of Service Robots with Ambient Intelligence; Stage 1".

[137]	3GPP TS 23.548: “5G System Enhancements for Edge Computing; Stage 2”.

[138]	3GPP TS 22.228:  “Service requirements for the Internet Protocol (IP) multimedia core network subsystem (IMS); Stage 1”.

[139]	3GPP TS 28.105: “Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management”.

[140]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”.

[141]	3GPP TS 23.503: “	Policy and charging control framework for the 5G System (5GS); Stage 2”.

[142]	3GPP TS 23.228: “IP Multimedia Subsystem (IMS); Stage 2”.

[143] 	Test Report from Utility Broadband Alliance “2024 UBBA Plugfest Testing Report” (https://www.ubba.com/ubba-resources/2024-ubba-plugfest-testing-report/).

[x1]                   “ITU-R FRAMEWORK FOR IMT-2030”                                                       (https://www.ngmn.org/publications/itu-r-framework-for-imt-2030.html).

[x2]                  “The Public Protection and Disaster Relief Liaison Rapporteur”                              (https://www.itu.int/pub/R-QUE-SG05.209-7-2023)

[x3]                   “Question ITU-R 209-7/5” Use of the Mobile, Amateur, and Amateur-Satellite Services in Support of Disaster Radiocommunications                                                                        (https://www.itu.int/md/R23-WP5A-C-0182/en)

[x4]                   “Guideline of the MIIT and 14 Other Departments on Strengthening Emergency Communication Capacity Construction in Extreme Scenarios” (https://www.gov.cn/zhengce/zhengceku/202501/content_7000295.htm).