3GPP TSG-SA WG1 Meeting #110 	S1-252673

19-23 May 2025, Fukuoka, Japan	(revision of S1-262667)

Source:	Nokia, KPN, Softbank

New use case title:	New use case on Compute Service Discovery in Coordination with the Network

Draft TS/TR:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.9

Document for:	Approval

Contact:	Hideaki Takahashi; hideaki dot takahashi at nokia dot com

Abstract: This contribution proposes a new use case on compute service discovery in coordination with the network to select an optimal application server instance or compute site to serve the UE.

Rev. 2192

- Changed the first digit of sub-clause number to “W”.

- The term “3rd party” is used in PRs.

Rev. 2660

- Fixed the missing update to use the term “3rd party”.

- Compute service instance is replaced with compute service

- Added table number and caption

Rev. 2667

- PR#1 is updated to clarify that computing resources are provided by a 3rd party and located outside 6G NW.

---------- Use Case template ----------

## W.X	Use case on Compute Service Discovery in Coordination with Network

### W.X.1	Description

Edge computing has become a key approach to improving user experience by deploying multiple computing and data storage resources across various locations. This approach aims to achieve benefits such as reduced latency, increased bandwidth efficiency, enhanced security and privacy, and improved reliability. Extended Reality (XR) is a notable application that requires multiple compute resources for edge/cloud rendering, which involves rendering either at a location closer to the user or at a central site in the cloud. Each application server instance can support a certain number of users based on its allocated compute resources. Although these resources can be scaled up to support more users, the overall capacity of the computing site ultimately limits scalability.

The current 5G system (5GS) is capable of providing networking support to Edge Computing services, with a primary focus on achieving or optimizing specific network performance targets, such as latency. In this approach, each user (UE, application client) is allocated computing server instance at the site closest to them. This can result in some compute sites becoming overloaded while others remain underutilized. To optimally allocate or balance the use of computing resources across different sites while ensuring the application's end-to-end performance and user experience, the selection of compute sites or application server instances should be based on both network and compute metrics. Any site or server instance that meets the user's requirements should be considered a candidate. Figure 8.x.1-1 illustrates a conceptual scenario where an optimal compute site is identified using both network and compute metrics.

Figure 8.x.1-1:	Compute Service Discovery by means of network and compute metrics

Beyond the traditional edge computing scenario, three emerging new scenarios can be identified, as illustrated in Figure 8.x.1-1:

1.	The selection of application server instance based on network and compute requirements: Unlike traditional edge computing where a server instance is selected based only on network metrics, this scenario also considers the compute metrics of the application server instance to be selected. For instance, an application server instance can support a certain number of users based on its allocated compute resources.

2.	The on-demand deployment of an application server in a compute site that is selected based on network and compute requirements: Unlike traditionally edge computing where an application server instance operates continuously, this advanced scenario allows application server to be deployed on-demand based on service requests, which would eliminate the need for pre-deployment on every site and save energy by running servers only when needed.

3.	The offloading of a software module from a UE to a compute site that is selected based on network and compute requirements: For instance, in immersive communications, applications running on the UE consist of multiple software modules, e.g., ‘scene manager’ module, ‘object detection’ module, ‘rendering’ module, etc., which can be dynamically offloaded to a compute site depending on available resources. This offloading reduces the UE's resource consumption and allows the software module to run more efficiently on the network's more powerful edge/cloud computing resources, enhancing the QoS.

In the scenarios described above, we can identify the following roles, as depicted in Figure 8.x.1-2:

Figure 8.x.1-2: Distinct roles in compute service coordination

1.	Network Service Provider (NSP), which provides communication services to the end-user (e.g., via access and core networks). The NSP ensures QoS and steering of traffic between the end-users and the target compute sites. Furthermore, the NSP can also measure network metrics between end-user and compute sites. These services are exposed by the NSP to enable compute service coordination;

2.	Computing Service Provider (CoSP), which offers computing resources to run the application servers that serve the end-user. The application servers can have multiple instances deployed at various physical sites of the CoSP. The CoSP is responsible for the lifecycle management of these application servers and the collection of related metrics. These services are exposed by the CoSP to enable compute service coordination. Nowadays, computing technologies are based on orchestration systems for automating the lifecycle management of the server instances. The orchestration systems adopted the industry are rapidly evolving independently from access and core network technologies.

3.	Application Service Provider (ASP), which provides the application servers that will serve the end-user. The ASP interacts with the CoSP to purchase compute resources to run the application servers.

4.	Compute and Network Coordination Service Provider (CNCSP), which is responsible for compute service coordination by selecting optimal compute sites based on network and compute metrics. It therefore provides its services to the ASP, or directly to the end-users (e.g., scenario of offloading a software module from UE to a compute site). The CNCSP may have agreements with multiple CoSPs and multiple NSPs. As shown in Figure 8.x.1-2, this CNCSP would therefore need to interact with CoSP and NSP to acquire the relevant metrics to perform optimal selection. Furthermore, since applications like XR can be deployed across various CoSPs and accessed through different access and core network providers (including non-3GPP systems), the CNCSP needs to evolve independently from specific cloud technologies or access and core network technologies.

NOTE:	These above-mentioned roles represent functional responsibilities and can be performed by a single organisation, such as a Mobile Network Operator. In this case for example, some of these roles could be fulfilled by leveraging the Service Hosting Environment of the MNO.

Given the aforementioned roles, it is crucial to clearly define the role that a 6G network should play. Indeed, considering CoSP, the underlying technologies are advancing independently of access and core network technologies and thus separated from the “6G network”, although they could provide the underlying infrastructure to host “6G network” capabilities. For instance, Kubernetes is nowadays the widely adopted cloud orchestration system in the industry, and is rapidly evolving by worldwide community of contributors under Cloud Native Computing Foundation. Furthermore, the CNCSP must support a variety of access networks used by the users, including non-3GPP systems. CNCSP should also cope with evolving technologies of CoSP. Therefore, the 6G network (access and core network) is provided by the NSP.

Table W.X.1-1:	Potential sustainability impacts of the use case

|  | (the UN SDGs/GDC matching goals of each aspect within 3GPP context) | Potential benefits of the use case (added value) | Potential areas of attention of the use case (risks to be mitigated) |
| --- | --- | --- | --- |
| Environmental sustainability aspects (UN SDGs 12, 13, 14, 15 and indirectly 6, 7 & 11 UN GDC “Develop principles for environmental sustainability of digital technologies”) | Energy resources (UN SDG 7, 11, 12) | - Reducing device energy consumption, by offloading one or more modules to compute sites - Reducing the network energy consumption too by taking metrics of energy consumption into account. - Reducing overall energy consumption by taking energy consumption metrics of compute site (e. g. considering choosing compute site with lower energy consumption or using cleaner energy source) | - success of such use case may trigger more low cost end-user devices and hence as rebound effect generate more energy consumption overall if an additional personal IoT device and not a replacement device of the smartphone for end users |
|  | Material resources (UN SDG 11, 12) | - - Reducing complexity of end-user devices (and thus e-waste) by leveraging network/edge compute capabilities | - - success of such use case may trigger more low-cost end-user devices and hence as rebound effect generate more material extraction to build these devices, as well as globally additional e-waste |
|  | Emissions (UN SDG 6, 7, 11, 12, 13, 14, 15) | - Enabling CO2 emission reduction in the overall system by means of reduced energy consumption, which sources of energy are reduced (network and compute sites, instead of UEs) and can be better managed towards low carbon. | - success of such use case may trigger more low-cost end-user devices and hence as rebound effect generate more ghg emissions |
| Socio-economic sustainability aspects (UN SDGs 2, 3, 4, 5, 8, 9, 10, 11, 16 & 17 and indirectly 12) UN GDC “Closing Digital Divides and Accelerating SDG Progress” & “Expanding Digital Economy Inclusion” & “Fostering an Inclusive, Safe Digital Space”) | Health (UN SDG 3) | - - Offload enable lightweight devices that can thus be worn for a longer period | - effects of massive usage of wearables may have negative impact on health (EMF etc) or safety risks (be isolated from surrounding environment) |
|  | Inclusion & Equality (UN SDGs 11, 10, 4, 5 and indirectly 3, 16 & 17) | - offload enables low-cost end devices more largely affordable by the population - Wearable devices – e.g., handsfree, lightweight – can also be used by a larger population base beyond some disabilities |  |
|  | Trustworthiness (UN SDGs 11 and indirectly 3 & 17) | - Offload to the compute sites may provide various technical alternatives to local UE computing thus increasing the availability and resilience of the system towards service continuity | - privacy of data being offloaded (eg image/video of people’s face) has to be taken into consideration when offloading |
|  | Work & income (UN SDG 8 and indirectly 12) | - offload can enable longer battery time in operations, in particular for remote workers, thus increasing productivity |  |
|  | Infrastructure (UN SDG 9) | - - offload can benefit remote workers for many infrastructure sectors with longer battery times and lightweight, more affordable devices |  |
|  | TCO reduction (UN SDGs 8, 9 and 12) | - offload can benefit several sectors with more affordable devices, thus making some business cases of remote workers commercially viable | - extra cost for investment in end-user devices and subscriptions to access offload capabilities, which should be traded-off with productivity gains |

### W.X.2	Pre-conditions

An ASP has multiple pre-deployed instances of an application server (e.g., split rendering, etc.) at various physical sites of one or more CoSPs.

Subject to CoSP’s policy, the application server instances are always in-operation, or instead, not all of them are always in-operation and some of them are deployed only when a service is requested, i.e., on an on-demand basis.

The CoSP can support of the offload of software modules from the UE to its compute sites.

In CoSP’s platform, while the application server instances are active, their computing metrics (e.g., CPU/memory load of the server instances) are measured and exposed to CNCSP, which is to be used for server instance selection. Additionally, computing metrics of the compute sites (e.g., available CPU/memory of the compute site) are also measured and exposed to CNCSP, which is to be used for on-demand server deployment and software module offload.

The NSP measures UE-independent network metrics to compute sites (e.g., latency between access network and compute sites) and expose them to CNCSP. Additionally, for a given UE, the NSP is capable of measuring UE-specific metrics (e.g., latency between UE and access network) and exposing them to CNCSP.

### W.X.3	Service Flows

For compute service discovery as illustrated in Figure 8.x.1-1;

1.	The consumer (e.g., ASP or the end-user) invokes the discovery of compute service to CNCSP, which can be for the discovery of application server, the deployment of an application server, or the offload of a software module from the UE. The consumer provides the requested network requirements, the requested compute requirements, information about the UE, and information about application server or software module.

2.	Based on the UE information, the CNCSP requests the NSP to collect UE-specific metrics.

3.	Based on the requested network and compute requirements, the collected network and compute metrics (UE-independent and UE-specific), CNCSP selects a matching compute site from the available CoSPs. If specific network or compute requirements are not provided or known, CNCSP selects a target compute site with optimal network and compute metrics.

4.	In case of application server deployment, CNCSP requests the selected compute site to deploy the requested application server. In case of software module offload, CNCSP requests the selected compute site to initiate the offload of software module from the UE.

5.	CNCSP requests the NSP to enforce network requirements by means of QoS enforcement and traffic steering.

6.	CNCSP returns the response the consumer.

### W.X.4	Post-conditions

The application client (e.g., XR application) user can enjoy optimal performance and user experience by means of the metrics of computing and network. The immersive application client or server can relocate and offload one or more software modules in order to retain optimal QoS and user experience over dynamic compute, energy and network characteristics. In addition, the CoSP can save energy consumption by means of the on-demand basis service provisioning and enhance their annual ESG report.

### W.1.5	Existing features partly or fully covering the use case functionality

The 5G system covers features to support edge computing, including Session and Service Continuity, local routing and traffic steering, AF influenced traffic routing and AF requesting QoS for a UE originated and/or terminated traffic flow, as specified in TS 23.548 [137], whereas there is no stage-1 requirement explicitly associated with the edge computing functionality.

The 5G system supports features for the measurement of network metrics within 3GPP domain, as described for QoS Monitoring in clause 5.45 of TS 23.501 [140], and also to a target data network, as described for N6 Delay Measurement and Reporting in clause 5.8.2.23 of TS 23.501 [140] with further details in TS 23.502 [30] and TS 23.548 [137].

### W.1.6	Potential New Requirements needed to support the use case

[PR 8.X.6-1] The 6G network shall be able to determine the network metrics (e.g., latency) between a UE and a set of computing resources provided by a 3rd party and located outside the 6G network.

[PR 8.X.6-2] Subject to operator’s policy and agreement with a 3rd party, the 6G network shall be able to expose the determined network metrics (e.g., latency) to a 3rd party.