3GPP TSG-SA WG1 Meeting #110 	S1-252918

Fukuoka, JP, 19 - 23 May 2025	revision of S1-252118,2555

Title:	Use case on ubiquitous emergency rescue via UAVs

Agenda Item:	8.1.4

Source:	ZTE Corporation, CSCN, China Telecom, China Unicom

Contact:	    xu.ling@zte.com.cn; li.junli@zte.com.cn,

liqt@chinasatnet.com.cn,

xiaxu@chinatelecom.cn

Abstract: This contribution describes a new use case about the ubiquitous emergency rescue via multiple types of UAV through TN and NTNs.

* * * New use case * * * *

8.x	Use case on ubiquitous emergency rescue via UAVs

8.x.1	Description

When emergency situations (such as natural disasters or accidents) occurs, timely and effective rescue is crucial for saving lives and minimizing damage. Traditional rescue operations are affected by various factors, including the severity of a disaster, the availability of transport resources, and the damage to communication infrastructure. By utilizing unmanned aerial vehicles (UAVs) and other low-altitude aircraft supported by NTN, the ubiquitous emergency rescue can overcome limitations posed by adverse factors to minimize casualties and property losses.

The UAVs equipped with intelligent rescue planning system, high-resolution cameras, thermal imaging, and environmental sensors, could gather affected areas environment data in real-time, then deliver to the Emergency Response Agency (ERA). The ERA will design a rescue plan based on these information and dispatch UAVs embedded with 6G RAN to provide temporary communication for rescue team on site. Introducing low-altitude aircraft (e.g., UAV), into emergency rescue can significantly enhance situational awareness, rescue resource management, ultimately leading to more effective and timely responses in crisis situations.

NOTE:	The term 6G RAN does not imply any architectural assumption, e.g. whether 6G CN/RAN is a new or evolved CN/RAN (compared to 5G).

Refer the latest CHINA airspace classification [99], the low altitude area is below 3000m, where the low-altitude aircraft could fly. The communication coverage for altitude less than 3000m is needed accordingly. However, in some area, e.g., forests, mountains, etc. the terrestrial network is not available which great impacts the rescue operation. The Non-Terrestrial Networks (NTNs), supported by Low Earth Orbit (LEO) satellite networks, High-Altitude Platform Stations (HAPS), multi-orbit satellites and other space-based infrastructure, could compensate the coverage blind spots and insufficient coverage of terrestrial network to improve communication service reliability.

Beside NTN based communication service, 6G NTN system will provide computing capabilities in the operator managed Service Hosting Environment, which will reduce the throughput of the feeder link and help intelligent rescue planning system to process data more intelligent and faster.

8.x.2	Pre-conditions

ERA has already applied flight permission for its UAVs from local aviation authority. In order to carry out rescue operations effectively, the rescue teams are equipped with two kinds of UAVs, i.e.Type-1 UAVs (one kind of rescue UAV, equipped with UE, high-resolution cameras, advanced sensors) and Type-2 UAVs( another kind of rescue UAV, embedded with 6G RAN).

NOTE:	The term 6G RAN does not imply any architectural assumption, e.g. whether 6G CN/RAN is a new or evolved CN/RAN (compared to 5G).

Operator B provide 6G TN and NTN service.

The ERA has a contract with Operator B to achieve 6G TN and NTN services.

The rescue team is equipped with Augmented Reality (AR) devices.

8.x.3	Service Flows

Figure 8.x.3-1 Illustration of emergency rescue via UAVs supported by the NTN.

NOTE:	The term 6G RAN below does not imply any architectural assumption, e.g. whether 6G CN/RAN is a new or evolved CN/RAN (compared to 5G).

- When an earthquake occurs, the terrestrial network breaks down. ERA dispatches Type-1 UAV A1, A2 and Type-2 UAV X1 to the disaster area.

- With the help of 6G NTN network for C2 communication, UAV A1, A2 and X1 successfully arrive in the disaster area.

- The UAV X1 working as the temporary 6G RAN, provides 6G coverage for the ground rescue team and low altitude UAV A1, A2, with the assistance of 6G NTN network for the disaster area.

- The UAV X1 could detect ground rescue team moving and UAV A1, A2 flying, and adjust its location to continue providing communication service.

- When the 6G NTN backhaul link quality for UAV X1 degrades, the 6G system transfers UAV X1's communication connection to the nearest LEO satellite, ensuring uninterrupted rescue task operation.

- The UAV A1 and A2 collect disaster area environment data (e.g., thermal imaging/video to locate survivors and environmental monitoring for hazardous conditions, etc.) via their cameras, and deliver these data to ERA through 6G NTN and 6G terrestrial network as the orange line showed in the figure. The data delivery need real time and higher data rate in uplink (e.g., 50Mbit/s) even in complicated geographical environments and long transmission distance.

NOTE 1: The calculation of the uplink data rate needs to comprehensively consider the various data types, including video streams, images, sensor data (such as position, gestures, and depth) of the environmental, etc.  Based on TR 26.925 [24], the typical streaming video for real-time 4K UHD is from 5Mbps to 25 Mbps with 4K resolution (3840×2160). The amount of sensors (including position, posture, gesture, High-resolution depth sensor, etc.) data depends on the data frequency and resolution which in general are 25Mbps. In summary, the Uplink data rate is about 50 Mbps. If it is 8K or 12K, the data rate will be increased more.

- ERA receives disaster area environment data from UAV A1 and A2, analyze and identifies survivor’s location, then direct the rescue team, UAV A1 and A2 to go to the location.

- The rescue team on-site with augmented reality (AR) glasses receives real-time data from UAV A1 and A2 through the communication link provided by UAV X1. These AR glasses can use image recognition technology to identify survivors' faces, heat sources, and dangerous areas, and overlay information in the field of view, providing visual decision support and helping them better understand the situation on site.

- The rescue team receives images from UAV A1 via the 6G RAN on boarding UAV X1, as the red line showed in figure, which require high reliability and low latency transmission in this link. Through the images, rescue team finds survivor in the location area and carry out the rescue as soon as possible.

- Finally, all rescue tasks are successfully completed.

8.x.4	Post-conditions

All the UAVs can be efficiently work for rescue through 6G NTN network. All survivors are timely rescued and treated.

8.x.5	Existing features partly or fully covering the use case functionality

In TS22.261[14], following NTN requirements have been specified, but have not covered RAN on boarding UAV.

The 5G system shall be able to support mobile base station relays using 3GPP satellite NG-RAN (NR satellite access).

The 5G system shall be able to support mobile base station relays accessing to 5GC via NR satellite access and NR terrestrial access simultaneously.

The 5G system shall be able to support service continuity for mobile base station relays using at least one 3GPP satellite NG-RAN.

NOTE 7:	This requirement applies to scenarios where there is a transition between two 3GPP NG-RAN, operated by the same MNO, involving at least one 3GPP satellite NG-RAN.

For a 5G system with satellite access, the following requirements apply:

-	A 5G system with satellite access shall support the use of satellite links between the radio access network and core network, by enhancing the 3GPP system to handle the latencies introduced by satellite backhaul.

-	A 5G system with satellite access shall be able to support meshed connectivity between satellites interconnected with ISLs.

The 5G network can also support multiple wireless backhaul connections (e.g. satellites and/or terrestrial), and efficiently route and/or bundle traffic among them.

In TS22.125[35], following “BS on boarding UAV” requirements are specified, but the NTN has not been considered.

[R-6.4-001] The 5G system shall be able to support UxNBs to provide enhanced and more flexible radio coverage.

[R-6.4-002] The 3GPP system shall be able to provide suitable means to control the operation of the UxNBs (e.g. to start operation, stop operation, replace UxNB etc.).

[R-6.4-003] The 3GPP system shall be able to provide means to minimize power consumption of the UxNBs (e.g. optimizing operation parameter, optimized traffic delivery).

[R-6.4-004] The 3GPP system shall be able to minimize interference e.g. caused by UxNBs changing their positions.

And the KPI is specified in Table 7.1-1 KPIs for services provided to the UAV applications, which did not include reliability KPI.

8.x.6	Potential New Requirements needed to support the use case

NOTE :	The following terms 6G RAN/CN do not imply any architectural assumption, e.g. whether 6G CN/RAN is a new or evolved CN/RAN (compared to 5G).

[PR.8.x.6-1]  The 6G system with satellite access shall support the use of satellite links between the 6G RAN on board of UAV and 6G CN.

[PR.8.X.6-2] Subject operator’s policy, the 6G system with satellite access shall be able to provide suitable means to start and stop the operation of the 6G RAN on board of UAV.

[PR.8.x.6-3]  The 6G system with multiple orbit satellite   (e.g., LEO, MEO, GEO) access shall support to dynamically select suitable satellite backhaul links for 6G RAN on board of UAV.

[PR.8.x.6-4] The 6G system with satellite access shall support communication performance requirements in table 8.x.6-1

Table 8.x.6-1 Performance requirements for ubiquitous emergency rescue via UAVs

| Scenario | user data rate (DL) | user data rate (UL) | Area traffic capacity (DL) | Area traffic capacity (UL) | Communication Latency | Reliability | Overall UE density | Moving speed | Coverage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ubiquitous emergency rescue : the 6G RAN on board of UAV | TBD | TBD | TBD | TBD | TBD | [99.999]% | TBD |  | Altitude: up to 3000 m |
| ubiquitous emergency rescue: UE on boarding UAV | TBD | [50]M bit/s | TBD | TBD | TBD | [99.999]% | TBD | 50 – 100km/h | Altitude: up to 3000 m |
| NOTE: All the values in this table are targeted values and not strict requirements. The UL data rate is assumed video flow 25Mbps [24] + high resolution depth sensor data 25Mbps; The altitude could up to 3000m refer[99] |  |  |  |  |  |  |  |  |  |

* * * End of the First Change * * * *