3GPP TSG-SA WG1 Meeting #111 	S1-253031

Goteborg, Sweeden, Japan, -25-29 August 2025	(revision of S1-25xxxx)

Source:	InterDigital

Use case title:	Trustworthiness AI use cases

Draft TS/TR:	3GPP TR 22.870

Agenda item:	8.1.3

Document for:	Approval

Contact:	Catalina Mladin, Catalina.Mladin@Interdigital.com

## Abstract:

## ---------------- Change 1: -----------------------------------------------

## 6.x	Network-related Trustworthiness assessments for Resource Exposure

6.x.1 	Description

With the advent of advanced technologies such as Generative Artificial Intelligence (Gen AI), Large Language Models (LLMs), and AI-accelerated xPUs, more applications based on AI can utilize wireless network of operators which are now equipped with substantial computational and storage infrastructures that often remain underutilized in the deployed edge data centres. These computation resources may be hosted at edge sites in a distributed manner. It is in the strategic interest of these operators to provide and expose availability of these dormant resources to a third-party in a trustworthy manner. This exposure allows third-parties to leverage the computational power and AI capabilities, as value-add resources, when required. In exchange, operators can monetize these assets by charging nominal fees for offered computing resources (e.g., computing cycles) from the edge data centres at the 6G infrastructure. For instance, applications for enabling AI services may demand extensive computing resources for processing the training data as well as for inferencing (for instance for task offloading, while performing LLM training or inference) and storage utilization by third parties. In addition, operators can monetize their network’s capability to assess the trustworthiness of the value-add resources by exposing selected and abstracted information to the third parties.

This approach not only generates additional revenue streams for network operators but also ensures the optimal utilization of existing infrastructure and capabilities. Consequently, it justifies the need for new requirements in 6G networks to expose and manage these resources and capabilities effectively. By incorporating standardized interfaces, robust security protocols, and advanced resource management frameworks, 6G can facilitate efficient and secure access to these resources. This transformation would position 6G networks as comprehensive platforms for hosting in operator’s edge data centres distributed computing and AI services for third-parties, aligning with the evolving demands of the technology landscape and provide resource as a service. This unique network-aware resource exposure capability is enabled by the 6G system's centralized policy framework.

The 6G system will be built on the cloud, optimized for ubiquitous computing, and natively provide computing services (e.g. in forms of infrastructure services, platform services, and software services) [2].

### 6.x.2	Pre-conditions

Network Operator A is equipped with significant compute, storage, and AI processing resources in deployed edge data centres providing services to UEs hosting applications . Some of these resources are used by NFs, OAM, hosted services, AI processing of network AI agents or supporting UE AI agents etc.

However, resource utilization varies based on the actual utilization by the hosted applications, network load, and during periods of low demand-often at certain times of day or night-up to 40 % of these resources remain idle. These idle resources represent untapped potential, lying unused with no opportunity for additional utilization.

Meanwhile, Operator A's customer, a gaming company called GameCoX, experiences high demand for computational resources during nighttime hours when many gamers are online on both computers and mobile devices. As games become increasingly complex and distributed, companies like GameCoX are integrating LLMs into their platforms to enhance gameplay and to derive user context (e.g. user intent). Offloading some of this processing to external infrastructure would be beneficial, particularly if GameCoX could show that, enabled by UE AI Agents, it has means to determine the idle resources it requests at a competitive rate from Operator A in a precise and efficient manner.

As an existing customer of Operator A, company GameCoX  is authorized to access Operator A’s APIs for requesting access to specific/ additional  resources at favourable rates, based on Operator A’s policies.

When playing on their UEs, users of gaming platform GameCoX consent to hosting AI agents on their UEs that leverage user context (e.g. as user intent) for the purpose of network and compute resource optimization.

### 6.x.3	Service Flows

- A user starts playing game GameCoX on their UE which include a variety of AI user prompts. The user activities result in large computation needs and includes access to LLM as part of the invoked application or Agent Y. Upon the user prompt and based on the AI model, the AI Agent on the UE accesses network (Operator A) and third-party GameCoX information as knowledge sources additional to the user prompt. Based on such knowledge sources and the user’s requirements, the AI Agent determines user-specific information (e.g., communication QoS, offloading compute, compute requirements) for overall resource optimization. The AI Agent Y provides this information to the third-party gaming platform.

2.	The third-party requests access to specific xPU (GPU, CPU etc.) or AI resources via an API provided by Operator A, based in part on the AI Agent output in step 1. The request may include trustworthiness parameters for the requester and/or trustworthiness criteria for the resources requested.

3.	The 6G network verifies the request against defined policies and authorizes access if conditions are met. As part of this step, a trustworthiness assessment of the request may be performed based on policies. The trustworthiness assessment may be based on: user, UE, third-party, location, resources requested, service requirements, reliability requirements, security requirements etc.

4.	The 6G network checks with the policy framework, which in turn may confirm with orchestration system if sufficient resources are available for the expected duration based on the third-party request, by potentially leveraging the available information on the history of network usage and the trustworthiness evaluation. Upon approval, the 6G system allocates the requested resources and establishes secure access for the third-party. This unique network-aware resource exposure capability is enabled by the 6G system's centralized policy framework.

5.	The operator A could also add its own rules and policy to manage the resource allocation with the help of policy framework.

6.	The system continuously monitors resource usage and records data for charging and reporting purposes.

7.	The 3rd party is also allowed to analyse the usage in real time.

### 6.x.4	Post-conditions

1.	Third-party entity completes its tasks using the allocated resources.

2.	The 6G network releases the resources and updates the availability status in real-time.

3.	Usage data is logged, and relevant charges are generated and sent for billing.

4.	Policy function is in control of entire operation.

### 6.x.5	Existing features partly or fully covering the use case functionality

None.

### 6.x.6	Potential new requirements needed to support the use case

[PR 6.x.6-1] Based on operator policy, the 6G system shall provide mechanisms for efficient and secure exposure of the network capability to support Trustworthiness assessment for the purpose of fulfilling service expectations while optimizing infrastructure utilization.

[PR 6.x.6-2] Based on operator policy, the 6G system shall provide suitable APIs for exposure of authorized Trustworthiness assessment enabled by the operator’s network for the purpose of fulfilling service expectations while optimizing infrastructure utilization.

## ---------------- Change 2:  ------------------------------------------

## 6.y	Network-related Trustworthiness assessments for collaborative AI agents

### 6.y.1	Description

AI Agents can perform tasks for or represent e.g. devices, persons, drones, or cars. These AI Agents may be either implemented in a UE or in the network.

AI Agents can be used for many different purposes. In this use case we are including examples of AI Agents interacting with end users on various devices (e.g. car)  and with applications in edge deployments, as well as with other AI Agents. Local/ edge deployments may also enable AI agents to select  other locally relevant AI Agents to collaborate with.

Given the large numbers of AI Agents and their diversity,  the network or deployment enabling their functionality can provide meaningful information for determining  access to resources ( including other AI Agent functionality) and how to establish and maintain secure associations between AI Agents.

Many electric cars have AI Agents with functionality for finding nearby car charging opportunities and optimising the price for charging the car. The AI Agent for the car is provided by the car company and may run on local networks depending on the car location. Some car owners take advantage of the AI Agent not only to determine lowest electricity prices, but also to make a profit by providing energy from the car battery back to the grid when prices are higher and their driving needs low. Conversely, other car owners prefer to minimize risks of being stranded and are less price sensitive.

### 6.y.2	Pre-conditions

A large, multi-day festival attracts many attendees  camping with their cars and caravan. The festival site uses an edge deployment set up for a nearby community and scaled up for the purpose of the event.

The deployment runs an application managing community information and community resources. For example, for power/energy resources it provides information about available electricity from utility company, from homeowners with solar or geothermal resources, about local charging grid, etc. The application can provide ‘spot-price’ electricity information, with prices fluctuating with locally available electricity sources. The edge deployment can run also disaster relief applications and services, offload users’ computation needs, etc. Therefore, the  edge deployment communicates with a variety of AI Agents.

Ann and Beth plan to attend the event in separate cars and caravans. They both plan to minimize the cost of the trip by saving on electricity usage as well as by optimally selling battery capabilities to the grid. For this purpose, each of them lays out a plan for the stay, including some general goals for cooking in the caravan versus taking short trips for food or longer ones for sightseeing.

Among them, there is an energy management application. It allows first responders and volunteer teams to discover,  register  and manage energy sources. After determining unaffected utility power and charging grids, the application allows both energy donors and receivers to register. For example, electric cars unnecessary for other operations may register to connect to active charging grids to provide energy. Conversely, devices necessary for the volunteer teams register for charging.

### 6.y.3	Service Flows

- As attendees arrive to the remote area, many of the attendee car AI Agents, including (Ann’s and Beth’s), discover the community energy application and are authenticated before further communications are enabled.

- Initially, each attendee car AI Agent communicates with the local energy application and it is provided with high-level information of the capabilities of the energy application and of the edge deployment. In turn, the car AI Agents provide capability information in terms of communications, maximum battery charging needs, etc.

- Both Ann’s and Beth’s car AI Agents determine based on the initial information that the electricity prices are likely to particularly high the first night when many cars arrive in need for re-charging. There is the possibility to make a profit if the car can actually provide energy from its battery back to the grid.

- The first evening, Ann’s AI Agent communicates with the edge energy application and indicates it wants to provide additional power to the community. The local energy application requests additional information to determine whether the advertised capabilities are available. The relevant information is provided and the grid connection used by Ann is set up to provide the battery energy to the community. However, the car AI agent routinely brakes communications with the edge deployment, the process gets interrupted often and then aborted before completing.

The first evening Beth follows the same steps and completes providing energy to the community successfully, without communication issues. She gets credited accordingly.

- After another two days, with wind energy being plentiful due to strong winds, both Anna and Beth recharge at a much lower price.

4.	After two days, a tornado blocks the event goers for a few days in the campground, with energy becoming scarce. First responders, volunteers and event goers can all provide energy to the community or request charging, as needed.

Beth’s offers to provide energy to the community are prioritized, matched with first responders with the highest need and/or credited at a high rate. The prioritization done by the energy management platform relies on trustworthiness information retrieved from the edge deployment

Ann’s offers to provide energy to the community are de-prioritised, due to the previously encountered availability issues tracked by the edge deployment. However, she is provided with opportunities to offer individual device charging services, e.g. for first responder’s devices near her car. After the completion of several successful direct charges managed by the edge application, her offers are prioritized the same as Beth’s.

### 6.y.4	Post-conditions

N/A

### 6.y.5	Existing features partly or fully covering the use case functionality

N/A

### 6.y.6	Potential New Requirements needed to support the use case

[PR 6.y.2-1] Based on operator policy, the 6G system shall provide mechanisms for continuous, privacy-preserving assessments of service requests relative to operator and/or third-party Trustworthiness policies  (i.e. objectives to fulfil or exceed service expectations), for the purpose of building system Trust and enhancing the services provided to all service participants.

[PR 6.y.2-2] Based on operator policy, the 6G system shall provide mechanisms for the establishment, confirmation and/or propagation of granular and dynamic relationships based on Trustworthiness.