3GPP TSG SA WG 1 Meeting #111 	S1-253032

25-29 August 2025, Goteborg, Sweden

Source:	Airbus, ESA, Fraunhofer IIS

pCR Title:	Pseudo-CR on “8.9 Use case on hybrid TN and NTN positioning”

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.5

Document for:	Approval

Contact:	José A. del Peral-Rosado, jose_antonio.del_peral_rosado@airbus.com

Abstract: The Editor’s Note in clause 8.9.6 is proposed to be removed based on the justifications proposed to be added in clause 8.9.1. This update of clause 8.9.1 also includes the missing reference to Figure 8.9.1-1.

1. Introduction

In 3GPP TR 22.870, the use case on hybrid TN and NTN positioning includes in clause 8.9.6 an Editor’s Note on the justification of the KPIs and misses a reference to Figure 8.9.1-1 in clause 8.9.1. These topics are identified by TR 22.870 rapporteurs as issue 109 and 110, respectively.

2. Reason for Change

The Editor’s Note in clause 8.9.6 indicates: “it is FFS whether KPIs are needed, and if needed, how the accuracy requirements are justified in relation to the latency and UE speed.”

The KPIs are needed to provide performance requirements to fully define the high-accuracy positioning service with high availability.

The airplane taxiway only targets horizontal accuracy, since the airplane is on ground. As indicated in clause 8.9.1, “navigation requirements depend on the taxiway width of the airport”. The target horizontal accuracy of 1 m at 95% within this use case is derived from category F airports for the taxiway phase. As indicated in [25] of TR 22.870, low-latency requirement is crucial to ensure real-time situational awareness, where typical values of low-latency requirements are below 1 s, including special cases up to 0.1 s. Furthermore, as indicated in clause 8.9.1, the verification mechanism is expected to operate with update rates between 2 Hz and 10 Hz and an availability above 99%, similarly to GNSS state-of-the-art solutions. The airplane taxiway speed of 100 km/h (i.e., between 50 and 60 knots) is justified by the ground aircraft speed in the rapid exit phase as per ICAO recommendations.

The UAV facility monitoring targets horizontal and vertical accuracy below 3 m, aligned with “between 2 and 5 meters for en-route flight” indicated in [25] of TR 22.870. This reference also indicates the importance of low-latency services for Beyond Visual Line of Sight (BVLOS) operations in UAVs, which are applicable to UAV facility monitoring scenarios. The UAV speed of up to 160 km/h (i.e., around 90 knots) is aligned with high-speed UAV operations necessary for monitoring and surveillance, as indicated in [25] of TR 22.870 and in clause 8.9.1 “for surveillance, where the UAVs can achieve speeds up to 160 km/h”.

The proposed changes include: 1) the update of reference [25] (including its link) with the latest version of "Report on Aviation and Drones – User Needs and Requirements" in clause 2; 2) the addition of the missing reference to Figure 8.9.1-1 in clause 8.9.1; 3) the update of the introduction text 1 in clause 8.9.1 by adding the reference [25] to support the justifications of the performance requirements; 4) the removal of Editor’s Note in clause 8.9.6 based on the justifications provided in this document and the updates of clause 8.9.1. These changes are proposed to solve the existing issue 109 and 110 identified by TR 22.870 rapporteurs.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.3.1.

* * * First Change * * * *

# 2	References

(…)

[25]	European Union Agency for the Space Programme (EUSPA): "Report on Aviation and Drones – User Needs and Requirements", 2024. (Report_on_User_Needs_and_Requirements_Aviation.pdf)

* * * Next Change * * * *

## 8.9	Use case on hybrid TN and NTN positioning

### 8.9.1	Description

High-accuracy positioning with high availability is a key feature to enable increased efficiency and automation of operations, such as in mapping and transportation. This is typically achieved thanks to the use of GNSS with precise corrections, which can be provided by 3GPP networks through assistance data. Although GNSS and commercial corrections systems offer reliable services, their positioning solutions can still be affected by different sources of error, such as multipath or interferences. Thus, relative positioning solutions, such as sensors or vision-based navigation, are typically coupled with precise GNSS to enhance the positioning robustness. However, there are very limited mechanisms to verify the absolute positioning accuracy and availability of these precise GNSS solutions, especially when relative positioning solutions are not available or operational.

Dedicated 3GPP terrestrial networks (TN) can be deployed to enable high-accuracy positioning with high availability in enhanced service areas, while ensuring low latency. Nonetheless, the increased cost expenses of such dedicated deployments are not expected in commercial networks. Furthermore, the use of commercial TN deployments may not be sufficient to provide complementary 3GPP positioning for ground users, due to the expected blockage of the line-of-sight (LoS) from distant base stations, i.e., only one or two base stations may be available in LoS conditions. In case of aerial users, the commercial TN deployment may be sufficient to provide 3GPP horizontal positioning, however the similar altitude of the terrestrial base stations results in a poor accuracy in the height estimation. For both ground and aerial users, the deployment of 3GPP NTN can then increase the number of available LoS links and the vertical geometrical diversity, with respect to only using 3GPP TN positioning, as shown in Figure 8.9.1-1.

Figure 8.9.1-1: Hybrid TN and NTN positioning with 3GPP technologies

Hybrid TN and NTN positioning includes the combination of TN and NTN to support 3GPP positioning technologies. Thus, hybrid TN and NTN positioning with 3GPP technologies can enable high-accuracy positioning with high availability and fast convergence time, which could be used as a verification mechanism of non-3GPP positioning technologies, such as GNSS with precise corrections. This high-accuracy positioning service can then fulfil stringent performance requirements from aviation and UAV applications, such as those in [25].Considering GNSS state-of-the-art solutions, the verification mechanism is expected to operate with update rates between 2 Hz and 10 Hz and an availability above 99%.

One example application is the positioning verification of airport surface operations, where the aircraft on ground is required to meet safety navigation requirements in the taxiway. The aircraft follows the planned path based on the on-board navigation systems and visual operations, while the navigation requirements depend on the taxiway width of the airport, including the rapid exit phase with a ground aircraft speed up to 100 km/h. Low-visibility conditions can severely impair these aircraft taxi operations.

Another example application is the positioning verification of UAVs for facility monitoring. The UAVs fly over the installations for inspection (e.g. to capture photos and videos) or for surveillance, where the UAVs can achieve speeds up to 160 km/h.

In both cases, the verification mechanism is expected to operate with update rates between 2 Hz and 10 Hz and an availability above 99%, similarly to GNSS state-of-the-art solutions.

* * * Next Change * * * *

### 8.9.6	Potential New Requirements needed to support the use case

[PR 8.9.6-1]	The 6G system with terrestrial and satellite access shall be able to provide high-accuracy positioning service with high availability, by using 3GPP positioning technologies and fulfilling the following KPIs:

Table 8.9.6-1: Performance requirements for positioning services based on hybrid TN and NTN positioning technologies

| Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |
| Airplane taxiway (note) | [1] m | N/A | [99 %] | [0.1 - 0.5] s | Outdoor | Up to [100] km/h | Airplane mounted |
| UAV facility monitoring | [3] m | [3] m | [99 %] | [0.1 - 0.5] s | Outdoor | Up to [160] km/h | UAV mounted |
| NOTE: Airplane taxiway refers to an airplane on ground during taxi operations. |  |  |  |  |  |  |  |