3GPP TSG SA WG 1 Meeting #111 	S1-253078

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:		OPPO

Title:	Network managed localized communication for verticals

Agenda item:	8.1.8

Document for:	agreement

Contact:	Yang Xu (xuyang@oppo.com)

Abstract: This contribution provides a use case for vertical scenarios The 3GPP network managed localized communication for vertical scenario.

********** Start of Change*********

11.x	Network managed localized communication for verticals

11.x.1	Description

One of new capability is to establish one or more local network which are managed by one Network operator (can be PLMN or NPN), while the 3GPP UEs in the same local network can communicate with each other via localized 6G network functions which are closer to the UE, with authentication and authorization by 6G network functions which can reside in a centralized location and further away from UEs.

Support indirect communication among 3GPP UEs using 6G radio interface and local traffic breakout can help offloading Local services to local networkto reduce load for centrally located CN nodes (Higher Experienced Rate and Connection Density), and local traffic management provides better performance of latency and mobility (Stricter latency and mobility requirement).

The proposed localized communication may have a wider coverage which can be used for communication of 3GPP devices belonging to a user, a family, or used for a Customer Premises Network (CPN) for a factory, residence, office or shop). It may also achieve the same level of transmission latency as direct device connection due to the communication is via a 6G access network while 3GPP UE need not additionally support direct device connection.

11.x.2	Pre-conditions

Factory-A wants to wirelessly connects its devices with a very low latency (e.g 10ms). It has a limited budget to purchase customized UE with direct device connection. It is also difficult for them to deploy a complete local core network including user plane function for LAN.

Mobile operator-B provides the local traffic transmission service, where the 3GPP devices can perform local traffic transmission within the local network coverage area via a 6G access network and limited core network function in the local network.

11.x.3	Service Flows

When Factory-A wants to support communication with very low latency, it applies to Mobile operator-A with their 3GPP devices for local traffic communication. Based on SLA, the certain 3GPP devices are authenticated and authorized by Mobile operator’s core network and perform the traffic transmission with ultra-low latency.

The mobile operator can provide certain QoS guarantee fr the local traffic transmission, which fully satisfied the Factory’s requirement.

11.x.4	Post-conditions

Thanks to the network managed local traffic transmission, the Factory A can realize the local transmission among their devices with ultra-low latency. And it saves much money to purchase a customized devices with direct device connection and no need to deploy the local user plane function.

11.x.5	Existing features partly or fully covering the use case functionality

In 3GPP TS 22.261 clause 3.1 and 6.38, it already has the service requirement of PIN. But PIN is a much smaller scope and it’s tighted to individual cusomters and exclusive for PLMN, such as “CPNs and PINs have in common that in general they are owned, installed and/or (at least partially) configured by a customer of a public network operator”. PIN in 6G can be extended to bigger scale as indicated in the use case.

The network managed localized communication assumes the two 3GPP UEs communicate to each other via 3GPP radi/Uu supported by 6G network

PIN support local inter-PIN element communication using direct network connection only if the PIN elements are in the range to use PIN direct connection, as indicated in TS22.261. clause 6.38.1 “Via a PIN Element with Gateway Capability, PIN Elements have access to the 5G network services and can communicate with PIN Elements that are not within range to use PIN Direct Connection.”

For this network managed loal traffic transmission, 3GPP UE need not additionally support direct device connection.

11.x.6	Potential New Requirements needed to support the use case

Functionality Requirements:

[P.R.5.x.6-1] Subject to operator policy and user consent, the 6G system shall support a user data path trasversing via 6G network for local traffic transmission regardless of whether 3GPP UEs are within the range to use direct connection or not.

[P.R. 5.x.6-2] Subject to operator policy and user consent, the 6G system shall be able to identify the 3GPP UEs in the specific area and support the association between the 3GPP UEs and a User ID for local traffic transmission.

NOTE: the User ID can be an identification of a human user, a family or an enterprise.

[P.R. 5.x.6-3] Subject to operator policy and user consent, the 6G system shall be able to support charging for the localized communication.

[P.R. 5.x.6-4]	Subject to operator policy and user consent, the 6G system shall provide the local traffic transmission between two 3GPP UEs via 6G network with fulfilling the following KPIs:

Table 11.x.6-1  KPI for localized communication between two 3GPP UEs via 6G network

| Scenario | Experienced data rate (note 1) | End-to-end latency (between UEs) | Reliability | Area traffic capacity | Area user density | Area |
| --- | --- | --- | --- | --- | --- | --- |
| Factory | ≤ 1.5 Gbps (Note 1) | [10] ms (Note 1) | 99,99% |  | [100] devices /10000m2 | 10,000 m2 |
| Vehicle | 0,1 to [1] Gbit/s (Note 2) | 10 ms (Note 2) | 99,99% |  | [30] devices/15 m2 (Note 3) | 15 m2 |
| Note 1: Refer to the KPI requirement for direct device connection in TS22.261 Table 7.10.2-1. Note 2: Refer to the performance of Gaming or Interactive Data Exchanging in TS22.261 Table 7.6.1-1 Note:3: an area of a car assums 5m*3m, it may have [30] devices or sensors within a car. |  |  |  |  |  |  |

*******************End of Change*****************