3GPP TSG-SA WG1 Meeting #111 	S1-253117

25-29 August 2025, Goteborg, Sweden	(revision of S1-250xxx)

Source:	China Mobile, ZTE

pCR Title:	Pseudo-CR on update 5.7.5 Network slicing

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Huazhang Lv, lvhuazhang@chinamobile.com

Xiaonan Shi, shixiaonan@chinamobile.com

Dan Wang, wangdanyjy@chinamobile.com

Abstract: This paper provides the updated requirements of network slicing.

1. Introduction

In Fukuoka meeting, several companies that agrees in 6G still needs network slicing. But whether 6G network slicing is the same as 5G or not still has no conclusion. If the 6G slicing is the same as 5G slicing, it means that all of the design should reuse the 5G design.

And from the view of operator, considering that in the early days of 5G, network slicing was regarded as an important technology for increasing revenue for operators. However, from the first discussion of 5G slice standard in 2016 until now, which has been nearly 10 years passed, the development of slice technology has not met expectations, and its contribution to the income growth of operators has been limited. One of the important reasons for this is that the design of the slice mechanism is complex and the implementation on UE side is difficult, resulting in the terminals not supporting some slice features, which has slowed down the commercialization of slice technology.

According to the stage 2 specification of slicing, some of the observations are listed below:

1) Complicated design of slice identifier for UE.

For example, in stage 2 we have too much identifier of slice. In specification, we have S-NSSAI, configure NSSAI, subscription NSSAI and reject NSSAI. This brings higher demand for UE design.

2) URSP rules in UE.

The slice features in 5G has high dependence on UE.

Firstly, before UE registers to network, the configured NSSAI should be configured in USIM. Secondly, when UE triggers registration procedure, the requested NSSAI should be sent to network via NAS message. Thirdly, after registration successfully, the UE performs the PDU session establishment with the help of URSP rules. The UE consumes the URSP rules to select the slice for certain application descriptor or DNN.

But now, from CMCC’s point of view, since that in 2017, the URSP rule is firstly introduced. But till now, the URSP is still not widely supported supported by UE. Where to support URSP rule in UE, on UE OS or UE modem, still unclear. Consider that URSP rule is the key to realize slicing, so this has become the pain points of commercialization of network slicing.

2. Reason for Change

None

3. Conclusions

In 6G, prefer that to have a network slicing that have minimum impact on UE to better promotion of slice deployment and commercialization.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.3.1

* * * First Change * * * *

5.7.5.1	Description

3GPP has introduced network slicing as a mandatory feature for 5G system and indeed this remains one of the few features that operators around the globe are trailing and attempting to monetize for differentiated services beyond traditional mobile broadband. While the 5G system and network slicing have had a learning curve, their adoption is accelerating. Therefore, we consider that network slicing will also need to be supported in 6G system. It is therefore assumed network slicing will remain a component of the 3GPP system as we transit from the 5G system to the 6G system. From operators’ point of view, additional flexibility, automation and efficiency are possibly needed in the 6G network slice design and architecture, to deal with the increased dynamics required by verticals and for the new use cases. For example, in the 5G system, a slice creation/termination requires various network function (NF) configurations and optional rules for route selections depending on the UE platforms, resulting in delay and customized efforts to bring up a slice. The slice related solutions and mechanism should have some flexibility in implementation at the UE, RAN and core network sides, to allow operator to perform fast and smooth commercial deployment. These issues for deployment and potential solutions for the flexibility refer to enhancements in RAN, Core and UE.

* * * Second Change * * * *

5.7.5.2	Potential New Requirements

[PR 5.7.5.2-1] The 6G system should support potential enhancement of network slicing, e.g.:

-	Create slices quickly without much overhead/complexity by leveraging automated operations

-	Scale and manage the network slices efficiently

-	Improve the mechanism to select and access network slice(s)

-	Minimise the UE dependency on network slicing, if any.

* * * End of Change * * * *