3GPP TSG-SA WG1 Meeting #111 	S1-253118

25-29 August 2025, Goteborg, Sweden	(revision of S1-250xxx)

Source:	China Mobile

pCR Title:	Pseudo-CR on update 5.9.2 Data service

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Huazhang Lv, lvhuazhang@chinamobile.com

Xiaonan Shi, shixiaonan@chinamobile.com

Dan Wang, wangdanyjy@chinamobile.com

Abstract: This paper provides the update the requirements of data service.

1. Introduction

This paper proposes to update the requirements of data collection and and consumption for 6G system.

Sometimes that if the 6G system collects the data, such as the sensing data or AI data, for further use or exposure, these data should be stored internal the network.

For example, for sensing cases, some of the sensing data should be stored at least seven days according to the requirements of the local regulations. Also, for AI data, some of the historical AI data can be further used for model training or AI reference. If such data missing, the accuracy of the reference result will be impacted.

And according to the data storage, it should be supported that the data can be distributed or centralized stored. For example, the weather sensing result of a certain city may have high relationship with the location. And these data can be stored locally and requested on-demand.

2. Reason for Change

None.

3. Conclusions

Further update the requirements that related to data storage.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.3.1

* * * First Change * * * *

5.9.2.1	Description

To better serve the users and manage the network, as well as provide non-connectivity service (e.g. LCS), 3GPP system need to conduct collection of data associated with the network and services provided by the 3GPP system. The data collection in the 5GA network for network/service management and non-connectivity service is defined for specific use cases, resulting in varying solutions across different use cases, different layers and different domains. Here is a brief summary of different data collection work done in RAN/SA2/SA4 and corresponding security mechanism in SA3:

-	In RAN domain, data collection from UE is based on MDT (Minimization of drive testing) method as defined in TS 37.320 [237] and TS 38.331 [238]. The user consent of UE location data collection for MDT is defined in TS 32.422 [239].

-	For data collection for AI/ML in core network, the data collection feature permits NWDAF to retrieve data from various sources (e.g. NF such as AMF, SMF, PCF, NSACF, GMLC, and AF, OAM, etc.), as a basis of the computation of network analytics as defined in TS 23.288 [114]. The operator can leverage these network analytics to optimize the management of network operations. For instance, as outlined in clause 6.22 of TS 23.288 [114], the operator can collect network data (such as OAM data and CP signalling) to detect or predict a signalling storm and use the analytics to mitigate or prevent its impact. The data collection is performed via SBI between CN NFs, DCCF (Data Collection Coordination Function) is introduced as logical functionality to coordinate data collection in CN, and ADRF (Analytics Data Repository Function) is defined to store the collected data. The user consent of UE data collection for AI/ML is defined in TS 33.501 [250].

-	For UE positioning, the input data is collected from UE/RAN to CN via CP/UP path as the basis of the computation of UE location as defined in TS 23.273 [240]. The privacy check of UE location collection is also defined in TS 23.273 [240].

-	To collect data and to expose data between CN domain and AF domain, the network exposure mechanism in core network has been defined in clause 5.20, TS 23.501 [140] and clause 4.15, TS 23.502 [30], where the NEF is used as the termination node to isolate both sides. Especially for edge, exposure of UE data is defined in TS 33.558 [241].

-	If the data collection is triggered by OTT server in application layer from UE, the UP path may be used with the assistance of DCAF, as defined in TS 26.531 [242].

The aforementioned mechanisms are defined for different network functions to support data collection in different scenarios, leading to high standardization overhead. There is lack of coordination between these network functions, leading to isolated data collection and duplicated data collection. Meanwhile, the data transmission based on the control plane are not suitable for transmitting large amounts of data, leading to inefficient data transmission. Therefore, the data collection and control mechanism need to be improved in 6G.

Table 5.9.2.1-1 lists several potential use cases for data collection in 6G, and illustrates the corresponding data type, data provider, data consumer and data volumes for each use case. Take AI/ML data as example, there is a large amount of data types defined in 3GPP apart from those non-standardized ones, and the scale of data volume is usually large (from 10k to 100M).

Table 5.9.2.1-1: Heterogeneous 6G System Data

| Use case category | Data types | Data provider | Data Consumer | Data volume |
| --- | --- | --- | --- | --- |
| Sensing data collection/sensing result exposure (e.g. measurement) | Sensing measurement data requires further study and discussion. The potential sensing measurement can be channel estimation results, delay, Doppler, angle, signal strength, etc. | UE, RAN | Sensing Function | Related to use cases: e.g. data rate of measurement is about 10Mbps for objects creating hazards on roads |
| AI/ML data collection / analytics exposure (e.g. AI/ML for air interface, AI/ML for core network) | - For RAN data collection for Air interface, see TR 38.843 [243] - For core network data collection, see the data collection framework in clause 6.2, TS 23.288 [114], and the detailed data type are defined from clause 6.3 to clause 6.21, TS 23.288 [114]. | CN, RAN, UE, AF | CN, RAN, UE | Typically, the internal data collection for AI model training yields tens of thousands of samples. Related to AI model: e.g. the data volume is about 10k bytes to 100M bytes for AI model of beam management |
| Energy Efficiency (EE information exposure to AF) | still under the discussion in SA2 R19 EnergySys WID | CN, OAM | EIF (Energy Information Function) | Related to use cases: e.g. could be similar to data volume for charging |
| Positioning, | See clause 6, TS 23.273 [240] | UE, RAN | LMF, UE | Related to use cases |
| Network Exposure in core network | See clause 5. 20, TS 23.501 [140], and clause 4.15, TS 23.502 [30] | 5GC NF, AF | 5GC NF, AF | Related to use cases |
| … |  | … | … | … |

- All the above use cases for data collection and control share the following commonality：

- The data collection and control mechanisms need to support procedures such as data collection, data storage, data exposure and data deletion.

- The collected data could be requested or consumed by multiple data consumers within 3GPP system to support multiple services; for example, a single spatiotemporal environmental dataset can be concurrently utilized by multiple services, including IoT-enabled pervasive sensing service, latency-sensitive extended reality (XR) applications requiring <10ms motion-to-photon delays, and intelligent transportation systems (ITS) with centimetre-level geolocation precision.

- The upper bond of the collected data volume is very high, e.g. on the order of 100 M byte;

- Latency requirements for data collection and transmission exhibit variations across diverse use cases.

- The collected data transmission path between different entities within 3GPP system can vary widely, encompassing scenarios such as UE to RAN node，UE to CN node, RAN node to CN node, and CN node to CN node;

- All these use cases that involve collecting UE related data shall support security, privacy protection and user consent. In addition, security requirements shall consider different data types.

- All these use cases that involve data exposure shall be subject to operator policy and user consent.

Figure 5.9.2.1-1: Use case specific data mechanisms in 5G  common data mechanism in 6G

As shown in Figure 5.9.2.1-1, use case specific mechanisms are defined for each service scenario in 5G, leading to system complexity and inefficient data collection. As 6G networks integrate advanced capabilities progressively, such as AI, enhanced computing power, multi-dimensional sensing and digital twin network, they will need to process massive amounts of multi-source, heterogeneous data (from diverse formats and data resource) generated by 6G system. Consequently, 6G system shall support efficient data collection and control mechanism capable of addressing both universal requirements and scenario-specific demands across heterogeneous use cases such as AI/ML, sensing, XR service and digital twin network. The efficient data collection and control mechanism shall support various data providers and consumers, which maintains service-agnostic characteristics while embedding extensibility for emerging service.

Also, for some cases that the collected data or generated data should be stored in 6G system with effectively management.

Firstly, data storage in network can provide a unified, efficient and reliable data access interface. Some of the collected data may still be requested by several consumers after the data collection. For example, some consumers may request the original collected raw AIML data in network for further model training and reference. If the collected data is not stored, then the network will request to continuously and repeatedly perform data collection, which will consume a lot of energy. Also, for sensing cases, some of the sensing data (camara data of pedestrains) should be stored at least seven days according to the requirements of the local regulations. So, storage of the collected data may further improve the efficiency, reduce the energy consumption and satisfy the regulation.

Secondly, some of the data will be shared with each NFs in 6G network to keep the consistency. As an example, the PCF and other strategy decision points may collect data from user or AF and decide the policies. The unified storage ensures centralized management and consistency of the policy data, enabling all policy-dependent business Consumers (such as SMF, AMF) to execute based on the latest and unified policy rules.

For data storage, the 6G system can support both distributed data storage and centralized data storage. For example, the weather sensing result of a certain city may have high relationship with the location. And these data can be stored locally and requested on-demand.

In conclusion, the 6G system shall support efficient data collection and control mechanism, which includes the following characteristics:

-	Common data framework that can serve the need of multiple use cases, to improve data collection efficiency and avoid duplication data collection

-	Support data collection from various data providers and data exposure to various data consumers;

-	Support both centralized and distributed data storage and support efficient retrieval the collected data.

-	Support user consent for data collection; and

-	Support data exposure subject to operator policy and user consent.

* * * Second Change * * * *

5.9.2.2	Potential New Requirements

Editor’s Note: user consent in this section is FFS.

[PR 5.9.2.2-1] The 6G system shall support mechanisms for 6G System Data collection and consumption minimizing the impact to 6G services.

[PR 5.9.2.2-2] Subject to operator’s policy, regulation and user consent, the 6G system shall support processing of 6G System Data.

NOTE: examples of data processing are use case dependant, e.g. data fusion, data anonymization and data analysis.

[PR 5.9.2.2-3] Subject to user consent, regulation and operator's policy, the 6G system shall support secure means to expose 6G System Data to authorized trusted third-party, authorized network function or authorized UE.

[PR 5.9.2.2-4] The 6G system shall support security and privacy protection of the 6G System Data.

[PR 5.9.2.2-5] The 6G system shall be able to provide charging and accounting mechanisms for 6G System Data.

[PR 5.9.2.2-x] The 6G system shall support storage of 6G System Data.

[PR 5.9.2.2-x] The 6G system shall support the efficient retrieval of the stored 6G System Data.

[PR 5.9.2.2-x] The 6G system shall support both centralized and distributed data storage.

* * * End of Change * * * *