3GPP TSG SA WG 1 Meeting #111 	S1-253142

25-29 August 2025, Goteborg, Sweden

Source:	IIT Bombay

pCR Title:	New use case on 6G network offering information as a service

Draft Spec:	3GPP TR 22.870 V0.3.1

Agenda item:	8.1.9 - Other Use Cases

Document for:	Approval

Contact:	Manjesh K. Hanawal (mhanawal@iitb.ac.in)

Abstract: This document proposes a new use case on 6G network offering information-as-a-service. Cellular network operators collect data about UEs like connectivity and location. This data could be used to extract relevant information for different industry verticals / public services. For example, a city planning authority could leverage this information for relevant insights on human migration patterns and urban expansion. The proposed use case focuses on extracting and exposing such information for external use.

1. Introduction

This pCR proposes a new usecase in 6G networks providing information-as-a-service for third party applications.

2. Reason for Change

NA

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 V0.3.1.

* * * First Change * * * *

## x.1	Use case on network offering information-as-a-service

### x.1.1	Description

Consider a city’s planning authority aiming to deploy city tour buses for tourists. To determine the appropriate number of buses, the authority must first estimate how many tourists visit the city. These visitors may arrive via flights, buses, trains, cars, ships, or other modes of transport. The key question is: How can the planning authority accurately gather data on the total number of incoming tourists? Consider another example of disaster management. During any natural disaster, it is important to plan the evacuation and maintain the supplies to survivors. To plan rescue activities, the disaster management team needs to know the number of people in the affected area. How can the team get a good estimate of the number of people in the affected regions?

Moreover, it is not just the event authority that requires such data. Several other municipal departments—such as the police, public health, tourism, and public works—also rely on up-to-date city-level information to plan and execute their functions effectively.

Interestingly, cellular networks inherently possess valuable insights that can help address these needs. Through routine operations, mobile networks continuously collect service logs, location updates, mobility patterns, and traffic statistics from users' devices. This data, if appropriately processed and aggregated (this includes anonymization), could provide accurate and timely estimates of the number of people arriving in specific locations, possibly their modes of travel, and movement trends across the city.

The data available in the network may not readily reveal the requested information. It may require processing of historical data. Additionally, the processing requires provisioning of computational and storage resources. For example, to estiamte the number of tourists visiting a city for an annual event, all the historical and current roaming data needs to processed to extract the numbers.

In this context, this use case presents the requirement for supporting “information as a service” in the 6G network which includes extraction, exposure and computing of the data to derive relevant information.

### x.1.2	Pre-conditions

Operator A provides information service to third party applications like city planning authority X. This service is provided to users via an operator portal. The third party X is authorized with the operator A and registered for this information service with the network.

### x.1.3	Service Flows

Figure x: Service flows and examples for “Information as a service”

Figure x shows the service flows and examples for “Information as a service” support in the 6G network. Following is the description for service flows:

- Third party city planning authority X uses the operator portal to submit a query—such as, “How many people have arrived in the city?”

- This query is received by the Network AI agent. The network AI agent converts it into queries that the other relevant 6G network functions can understand.

- The network then retrieves large volume of relevant data. It processes this data to extract meaningful insights—for example, the total number of users currently visiting (roaming users) the city. And sends this information to the Network AI agent.

- The Network AI agent converts this information into simple response format that third party can understand and send it to the third party as response to the input query.

### x.1.4	Post-conditions

The planning authority organizes and deploys the appropriate number of buses.

### x.1.5	Existing features partly or fully covering the use case functionality

None

### x.1.6	Potential New Requirements needed to support the use case

[PR x.1.6-1] Subject to regulation and operator(s) policy, the 6G network shall support a mechanism to extract relevant information using historical operational data and network logged events, based on intent of a third-party application.

[PR x.1.6-2] Subject to regulation and operator(s) policy, the 6G network shall support exposure of the extracted information to third party application.

[PR x.1.6-3] Subject to operator(s) policy, the 6G network shall provide the necessary computing and storage resources for information extraction.