3GPP TSG-SA WG1 Meeting #111 	S1-253149

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:	ZTE, China Telecom， China Mobile

New use case title:	New use case on independent 6G Local Network for Factory

Draft TS/TR:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.8

Document for:	Approval

Contact:	XuLing xu.ling@zte.com.cn,

XiaXu,  xiaxu@chinatelecom.cn,

Abstract: <provide a short description of the content>

This contribution proposes a use case about independent 6G local network which could be automatically and quickly constructed to continue supporting factory operation when the external network is failure.

11.x	Use case on Independent 6G Local Network for Factory

11.x.1	Description

In smart factories, 6G networks are critical communication infrastructures that support real-time communication and collaboration among various types of devices, robots, sensors in the factory, local platform and cloud-based platforms. In the extreme case of a 6G network failure e.g. the connection between the RAN and CN is broken, the production and  management of the factory may be seriously affected. In this case, 6G network in factories need to quickly build a 6G local network to ensure production continuity.

Once a 6G network failure is detected, the 6G local network will be automatically and quickly constructed based on the existing 6G infrastructure in the factory. The 6G local network will maintain basic operation of production processes, ensuring that automated equipment, robots, sensors, etc. can continue to work together. The capability of the 6G local network depends on the network functions of the existing 6G infrastructure in the factory.  It supports local data delivery, processing and after the external 6G network is restored, it will resume either. Providing local monitoring and control functions is another demand to ensure production safety and efficiency.

11.x.2	Pre-conditions

Operator TT provides “Smart Factory network+” service for its factory customer through its 6G network which provide independent 6G local network with communication and data services when the network has some failure.

Operator TT has deployed and configured and network components e.g. base station, multiple local data processing nodes, “Smart Factory network+” local platform etc. in the factory.

Manufactory MM has subscribed the “Smart Factory network+” service from operator TT.

11.x.3	Service Flows

The “Smart Factory network+” local platform continuously monitors the 6G network. Suddenly, there is something wrong with the 6G network, the base station in the factory can’t connect with the core network deployed remotely.

The “Smart Factory network+” local platform sends the fault alert to the factory's production scheduling system which is deployed in the factory by Manufactory MM, and whole network components in the factory.

The whole network components include local service hosting environment, base stations, automatically construct a 6G local network to provide communication service for the factory's equipment, robots, and sensors, with no further communicate with external network.

Through the new constructed 6G local network, the factory's production scheduling system demands the factory's equipment, Automation devices (e.g., robotic arms, CNC equipment, AGVs, etc.), and sensors etc. continuously perform scheduled production tasks.

Since external networks cannot be connected, local service hosting environment takes over more local processing tasks. They continue to receive data from devices and sensors and assist the factory's production scheduling system to process all types of real-time data, such as production progress, equipment status, material scheduling, fault detection, etc. ensuring that production efficiency is not compromised.

Due to the failure of the external 6G network, the local production data is unable to be uploaded to the cloud or remote servers in real time. Production data from all devices, sensors and automation systems are transmitted over the local 6G network and cached in the factory's production scheduling system. Once the 6G external network is restored, the cached data is to ensure data automatically synchronized to the cloud consistency.

The “Smart Factory network+” local platform continuously monitors the local 6G network.

When the 6G external network is restored, the local 6G network will re-establish connectivity with the external network. The factory's production scheduling system will synchronize the cached data to the cloud to ensure data consistency. All devices resume communication with the cloud, enabling external data analysis and remote monitoring.

11.x.4	Post-conditions

After confirming that the external network is operating stably, the local 6G network connects with core network in remote, and continues to provide efficient communication support for the factory.

11.x.5	Existing features partly or fully covering the use case functionality

In TS22.261 section 6.41 Providing access to local services, multiple requirements about local service are defined. For example,

Subject to regulatory requirements and localized service agreements, the 5G system shall allow a home network operator to automatically negotiate policies with the hosting network for allowing the home network’s subscribers to connect at a specific occasion, e.g., time and location, for their home network services.

Subject to the automatic localized services agreements between the hosting network operator and home network operator, for UE with only home network subscription and with authorization to access hosting networks the 5G system shall support:

-	access to the hosting network and use home network services or selected localized services via the hosting network,

-	seamless service continuity for home network services or selected localized services when moving between two hosting networks or a host network and the home network.

The 5G system shall support a mechanism to enable configuration of a network that provides access to localized services such that the services can be limited in terms of their spatial extent (in terms of a particular topology, for example a single cell), as specified by a service provider of localized services.

The 5G system shall support a mechanism to enable configuration of a network that provides access to localized services such that the services can be limited in terms of the resources or capacity available, to correspond to requirements that apply only to the locality of service delivery, as specified by a service provider of localized services.

The 5G system shall enable the home network to allow a UE to automatically select a hosting network for accessing localized services when specified conditions (e.g., predefined time, location) are fulfilled.

The 5G system shall be able to prevent a UE to re-access the hosting network after the localized services were terminated if the authorization for the localized services is no longer valid (e.g., can be based on certain conditions such as time or location of the user).

The 5G system shall provide mechanisms to mitigate user plane and control plane overload caused by a high number of UEs returning from a temporary local access of a hosting network to their home network in a very short period of time.

The 5G system shall provide mechanisms to minimize the impact on the UEs communication e.g., to prevent user plane and control plane outages when returning to a home network together with other high number of UEs in a very short period of time, after terminating their temporary local access to a hosting network.

But, the automatically establishing local network and detection the home network restore and switch off the local 6G network are missed.

11.x.6	Potential New Requirements needed to support the use case

[PR.11.x.6-1] Subject to operator’s policy, the 6G network shall support to automatically establish local 6G network for communication among the UEs and local service hosting environment in local area, when 6G network failure is detected.

[PR.11.x.6-2] Subject to operator’s policy, the local 6G network shall be able to support detection of 6G network restore.

[PR.11.x.6-3] Subject to operator’s policy and agreement with the 3rd party, the 6G system shall be able to autonomously switch off local 6G network with minimum service interruption, when the 6G network is restored.