3GPP TSG SA WG 1 Meeting #111 	S1-253156

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:	FirstNet, MITRE

pCR Title:	Pseudo-CR enhancement of use case 8.12 Use case on HAPS-based rapid deployable network for public safety

Draft Spec:	3GPP TR 22.870, v0.3.1

Agenda item:	8.1.5

Document for:	Approval

Contact:	Mark Lipford, mark.lipford@firstnet.gov

…

Abstract: This PCR proposes updates to clause 8.12 of the TR 22.870 “Study on 6G Use Cases and Service Requirements, in order to support part of the service flow.

1. Introduction

This pCR introduces a new requirement for supporting UE-HAPS-UE communication

2. Reason for Change

To add a new requirement for UE-HAPS-UE communications when there is a base station onboard the HAPS.

3. Conclusions

n/a

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870, v0.3.1.

* * * First Change * * * *

## 8.12	Use case on HAPS-based rapid deployable network for public safety and disaster response

### 8.12.1	Description

This use case focuses on leveraging HAPS, specifically HIBS (base station on board a HAPS), to rapidly establish or restore communication services in areas affected by natural disasters (earthquakes, floods, hurricanes, wildfires) or other emergencies where terrestrial infrastructure is damaged, destroyed, or overloaded. HAPS platforms, potentially pre-positioned or rapidly flown into the affected zone, can provide wide-area coverage (e.g. 50-100 km radius per platform) for first responders, affected populations (Direct-to-Device). Its stratospheric position makes it immune to ground-level events. Multiple HAPS can be deployed to cover larger areas or provide redundancy.

### 8.12.2	Pre-conditions

•	A disaster or emergency event has occurred, disrupting terrestrial communications.

•	HAPS platform(s) are available for deployment or are already airborne near the affected region.

•	Authorized spectrum is available for emergency use.

•	Emergency response agencies and targeted user groups are equipped with compatible devices (standard smartphones or dedicated terminals).

•	A mechanism exists to establish backhaul connectivity for the HAPS (e.g. via satellite link (HTS), a connection to an intact ground gateway outside the disaster zone (HTG), or through another nearby HAPS (HTH)).

•	Coordination with relevant authorities (e.g. aviation, emergency management) is established.

### 8.12.3	Service Flows

- Upon notification/authorization, HAPS platform(s) are deployed/re-tasked to the target area.

- HAPS activates communication payload, establishing service links over the designated area.

- Backhaul connectivity (HTS, HTG and/or HTH) is established.

- Authorized UEs (first responders, public safety devices, potentially public UEs) register and connect to the network via HAPS.

- Voice, data, and potentially location services are provided. HAPS should prioritize traffic for emergency services.

- Network monitoring and management are performed remotely.

HAPS

HAPS

Satellite

Disinterred area

Core NW

HTS

HTH

HTG

Gateway

Direct access

Figure 8.12.3-1: HAPS-based rapid deployable network

### 8.12.4	Post-conditions

•	Essential communication services are established/restored in the affected area via HAPS.

•	First responders can communicate effectively.

•	Affected populations can access emergency information or contact help.

•	Network status is monitored, and operational logs are maintained.

### 8.12.5	Existing features partly or fully covering the use case functionality

•	Public Safety features (e.g. priority access [63], MCPTT [53]/MCData [56]/MCVideo [55]).

•	Location Services (LCS) [61].

•	Basic connectivity, QoS framework.

•	Mobility Management (Handover, Cell Reselection), especially in case the TN network is just partially or locally damaged, some level of mobility between TN and temporary HAPS coverage is expected.

•	Multi-connectivity/redundancy features with regard to multi-link backhaul connectivity.

### 8.12.6	Potential New Requirements needed to support the use case

[PR 8.12.6-1]: The 6G system shall enable deployment, configuration, service activation, and management for base station onboard HAPS platforms in emergency scenarios.

[PR 8.12.6-2]: The 6G system with base stations onboard HAPS platforms shall support UE-HAPS-UE communications without the use of core network.

* * * End of Changes * * * *