3GPP TSG SA WG 1 Meeting #111 	S1-253157

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:	FirstNet, DISA, AT&T

pCR Title:	Pseudo-CR on Use Case on Spectrum Sensing to Support Disaster Recovery

Draft Spec:	3GPP TS / TR 22.870, v0.3.1

Agenda item:	8.1.9

Document for:	Approval

Contact:	Mark Lipford, mark.lipford@firstnet.gov

Abstract: This pCR proposes “Use Case on Spectrum Sensing to Support Disaster Recovery” in TR 22.870 “Study on 6G Use Cases and Service Requirements”.

1. Introduction

This is a use case for spectrum sensing. After reviewing the existing use cases documented in the TR we determined there was not an existing use case to incorporate the specific requirement.

2. Reason for Change

This CR proposes to add spectrum sensing of 3GPP systems to support disaster recovery. We are currently proposing it to be added to the section W on “Other use cases” in the TR with the understanding that the use case may be moved to more appropriate section as part of editorial exercise at a later stage.

3. Conclusions

n/a

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870, v0.3.1.

* * * First Change * * * *

## W.X Use case on spectrum sensing to support disaster recovery

### W.x.1 Description

The cellular infrastructure and power infrastructure may be destroyed in the event of natural disasters including:

- Hurricane

- Tornado

- Flood

- Winter Storm

- Earthquake

- Wildfire

- Tsunami

In these types of situations, first responders gather at the scene to conduct disaster recovery operations. They need cellular communications services to enable them to support mission-critical communications, share important information about the environment, coordinate response operations, and conduct damage assessments, using 6G devices, which are the only common communication devices available. The first responders may deploy their own temporary networks (e.g. portable base stations) as new entrants’ private networks.

However, due to the urgency of conducting rescue missions and rapid deployment of equipment, it is likely that the new entrants’ private networks have not done proper spectrum coordination and planning with incumbent public networks in the area. As a result, there may be a need to support spectrum coexistence of available bands between private networks and public networks during disaster recovery operations.

The following provides assumptions.

- The disaster recovery services are provided by the new entrants’ private networks. The private networks supporting disaster recovery operations are for first responders only. The private networks to support disaster recovery operations are not for regular public subscribers.

- The new entrants’ private networks considered are standalone private networks with their own RAN and core infrastructures.

- The incumbent public providers may also deploy their own cell on wheels (COWs) and / or drones to restore cellular coverage as soon as possible. After the cellular coverage has been restored as well as the cellular capacity is sufficient to support both regular subscribers and first responders, it is possible that first responders can join public networks as subscribers or special priority can be provisioned for first responders (e.g., operators providing public safety network slices) to conduct disaster recovery operations. However, this scenario is not considered since no spectrum coexistence is required.

- It is possible that incumbent public networks also conduct their own disaster recovery operations; however, they are not considered as part of the use case since no spectrum coexistence is required.

- The scenarios for spectrum existence considered are only between new entrants’ private networks and incumbent public networks when first responders conduct disaster recovery operations and deploy their own private networks. The scenarios for spectrum coexistence between new entrants’ private networks and incumbent private networks are out of scope.

- The actual spectrum bands used by the new entrants’ private networks during disaster recovery operations are out of scope.

- The spectrum adaptability capabilities of the RAN equipment deployed in the new entrants’ private networks are out of scope since they are implementation specific. For example, some RANs could be based on software defined radios that can adapt to any spectrum band, but other RANs may support only one or a few specific spectrum bands.

- The scenario where Disaster Roaming capabilities are supported in the area as defined in clause 6.31.2.3 of [14] is not considered since no spectrum coexistence is required.

The following explains different sub-scenarios about the degrees of damage occurring at the public networks during natural disasters situations and their implications to spectrum coexistence.

- Sub-scenario 1: If the assets (including RANs) of all existing public networks get damaged or compromised, when a new entrant private network enters the area, there is no need to invoke spectrum coexistence. However, spectrum sensing can be used to determine what is still operating in public networks.

- Sub-scenario 2: If 80% of existing public networks get damaged while 20% of the public networks survive, a new entrant private network needs to invoke spectrum coexistence with the 20% of the public networks that are still operational.

- Sub-scenario 3: Continue from Sub-scenario 2. While 80% of the incumbent public networks repair their networks and bring themselves back online, a new entrant private network needs to invoke spectrum coexistence with 20% of operational public networks as well as those RANs back online.

In sub-scenarios 2 and 3, there will be co-channel interference among networks using the same spectrum during disaster recovery operations. The private networks need to de-conflict the spectrum contention during disaster recovery operations.

Spectrum sensing of available 3GPP networks in the affected area can be used to determine what is still operating in public networks as well as to enable spectrum coexistence between multiple temporary networks (e.g. portable base stations) conducting disaster relief and recovery operations in the affected area, as well as spectrum coexistence between temporary networks and existing public networks providing coverage in the affected area.

### W.x.2	Pre-conditions

The base stations of public networks have been damaged or destroyed during earthquake which created a major failure of the cellular networks where partial cellular services are not available in the affected area. First responders arrive at the scene to conduct disaster recovery operations in the affected area.

### W.x.3	Service Flows

1. First responders deploy temporary 3GPP networks (formed with portable base stations).

2. First responders enable spectrum sensing services on temporary 3GPP networks.

3. The Spectrum sensing service is used to generate spectrum information of available 3GPP networks in the affected area.

### W.x.4	Post-conditions

The generated spectrum information of available 3GPP networks determines what is still operating in public networks. The generated spectrum availability of available 3GPP networks enables multiple temporary 3GPP networks to coexist with each other using the same spectrum in the affected area. The generated spectrum availability of available 3GPP networks enables coexistence of temporary 3GPP networks with other existing 3GPP public networks in the affected area where the existing 3GPP public networks are in the process of bringing themselves back online.

### W.x.5	Existing features partly or fully covering the use case functionality

None

### W.x.6	Potential New Requirements needed to support the use case

[PR 7.x.6-x] Based on operator policy, regional and/or national regulations, the 6G system shall provide secure mechanisms to collect spectrum sensing results of 3GPP networks to determine spectrum availability in the desired geographical area.

* * * End of Changes * * * *