3GPP TSG SA WG 1 Meeting #111 	S1-253158

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:	T-Mobile USA

pCR Title:	Pseudo-CR on Enhanced Network Service Awareness in 6G

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: Provides potential new requirements for an enhanced service-aware RAN.

1. Introduction

6G applications and services, especially those with multiple data streams of mixed media types will present challenges to a network using traditional static or coarse-grained QoS mechanisms as the differing media within may require different treatment to optimize the subscriber’s QoE.  6G networks will face the challenge of efficiently managing the diverse application traffic with varying service requirements (e.g., latency, reliability, bandwidth).

Existing network QoS mechanisms lack real-time insight into the specific service characteristics and quality expectations of individual application traffic sub-flows. This limitation prevents the network from optimizing resource allocation and traffic differentiation in a manner aligned with end-user experience. The absence of dynamic, application-originated feedback on traffic requirements and quality of service degrades the network's ability to make informed decisions, especially during congestion scenarios.

To address this, the 6G Network needs mechanism(s) to identify the service characteristics (e.g., latency, reliability, throughput, and (relative) priority) of individual media components within a specific service or application not just at the initialization of the traffic, but periodically to reflect application/service characteristic changes (e.g. needing more or less latency).  The 6G Network could also provide information regarding network capabilities to a service or application, enabling the service or application to adjust (e.g. change a codec or lower video resolution) to changing network conditions.

2. Reason for Change

Current networks lack the flexibility to efficiently support applications that generate multiple concurrent traffic types with varying service-level agreement (SLA) requirements. Existing network capabilities do not provide mechanisms to dynamically adjust to changing service or application characteristics or to varying propagation conditions at a level of granularity that takes into consideration multiple media types per service or application in order to optimize the use of network resources and provide the best QoE to the subscriber as possible in that moment in time.

3. Proposal

It is proposed to agree the following new text to 3GPP TR 22.870v0.3.1 (clause 5.9).

* * * First Change * * *

### 5.9.x 	Enhanced Network Service Awareness

#### 5.9.x.1	Description

The 5G system provides capabilities (e.g. QoS, network slicing) that enable a network operator to offer differentiated services to their customers.  These service awareness capabilities provide the network with the means to identify applications, services, or flows, service characteristics (e.g. latency sensitivity, bandwidth needs, security requirements), and the ability to dynamically adapt network settings to meet the needed/requested service requirements.  This is a key capability to comply with SLAs.

6G services or applications (e.g. immersive communications or integrated sensing and communication services) may include multiple data streams (e.g. video, voice, text, sensor data) that exhibit varied traffic patterns and each component may have significantly different QoS requirements (e.g. latency, jitter, reliability, and throughput). The diversity and dynamism of such traffic patterns challenge traditional static or coarse-grained network management approaches.

Static slicing or coarse per-application (e.g. eMBB, URLLC) QoS management does not account for intra-application diversity, leading to suboptimal performance or over-provisioning of network resources. The 6G system needs to enrich handling intra-application diversity, used in 5G XRM services, for example, to provide expanded service awareness capabilities to enable the RAN to differentiate traffic types in a more granualar way and apply the most appropriate radio and scheduling policies accordingly.

The 6G system is also expected to increase user security and privacy, and these requirements should not be viewed in opposition to the requirements for increased network capabilities (e.g. slicing, traffic steering, QoS mapping) intended to improve network optimization, such as enhanced service awareness by the RAN. New mechanisms may be needed to satisfy both the security/privacy requirements while simultaneously enhancing service awareness.

The use of AI/ML is expected to improve the 6G system’s ability to enhancing service awareness, enabling (near) real-time detection of changes in service usage and assisting in proactive service assurance, dynamic handover optimizations, and spectrum reallocation based on detected service behaviour.

#### 5.9.x.2	Potential New Requirements

[PR-5.9.x.2-1] Based on operator policy, the 6G system shall support the ability for services, including 3rd party services, to exchange the more granular service characteristics for each media flow component of its service/application with the RAN and the RAN to exchange updated network information (e.g. changes to radio environment or resource availability) with the service/application, if it is capable of receiving this information, in order for the service/application to optimize the QoE of the user.

[PR-5.9.x.2-2] Based on operator policy, the 6G network shall support mechanisms for the RAN to dynamically adjust and optimize radio resources based on the service characteristics provided by the service or application.

[PR-5.9.x.2-3] The 6G network shall enhance the RAN resource management and QoS support capabilities to enable AI/ML-driven optimization in the RAN to support differentiated service treatment based on the service characteristics provided by a service or application.

[PR-5.9.x.2-4] The 6G system shall support user security, privacy and policy enforcement so authorised services and traffic get differentiated treatment as requested in the service characteristics.

[PR-5.9.x.2-5] The 6G system shall provide appropriate charging support for differentiated services per media component (e.g. to meet SLA requirements).

* * * End of Change * * *