3GPP TSG SA WG 1 Meeting #111 	S1-253178

25-29 August 2025, Goteborg, Sweden

Source:	CEWiT

pCR Title:	Pseudo-CR on Use case on enhancement of RAN exposure service

Draft Spec:	3GPP TR 22.870v0.3.1

Agenda item:	8.1.2

Document for:	Agreement

Contact:	Ankita Lachhwani

ankita.lachhwani@cewit.org.in

Abstract: This contribution proposes enhancement to the RAN exposure service which is currently supported in 5G

* * * First Change * * * *

## 5.7 	6G enhancements of legacy/existing services and capabilities

Editor’s Note:  Text to be provided.  This clause to address potential (new) requirements for new (6G) aspects of existing (5G and earlier) services or capabilities.

### 5.7.X	Use case on enhancement on RAN Exposure service

#### 5..7.X.1	Description

The exposure of network information has been a cornerstone for enabling applications to fetch data from the 5G system. This not only allows operators to monetize network insights but also empowers applications to take proactive actions, thereby improving service availability and quality.

In 5G, the core network exposes information related to RAN such as Cell ID and helps applications make informed decisions. However, for 6G, this level of exposure may not be sufficient. New types of information beyond the current set will need to be made available from 6G RAN.

For instance, in 5GS, when the IMS (IP Multimedia Subsystem) initiates an audio or video call, it requests QoS resource allocation to the Core Network. This involves a series of service requests across multiple 5G entities, including the RAN, UE and user plane functionality of core network. If the RAN rejects the QoS setup based on its prevailing resource capabilities, all previously executed steps must be rolled back resulting in inefficiency in the network.

To optimize this in 6G, it is crucial that the IMS or any third-party application is made aware in advance of the likelihood of a successful QoS setup at 6G RAN. This requires the 6G RAN to expose its capability or readiness to fulfil the QoS request. Such foresight would significantly reduce unnecessary signalling and improve response times.

Moreover, in certain scenarios, applications may tune their services to specific UEs based on current radio conditions of the UEs exposed by 6G RAN.

Other scenarios could be,

- Predictive Network State Sharing

  - Provide predictive insights to third parties based on AI/ML models, not just real-time changes.

  - Example: Forecasted RAN signal degradation, cell congestion, or expected handovers.

  - Advantages: Enables third parties to proactively reconfigure services before degradation occurs.

- Semantic Information Sharing

  - Move beyond raw data (e.g., “RSRP = -95 dBm”) to semantically meaningful descriptors.

  - Example:

    - “Network conditions suitable for 8K video streaming.”

    - “Cell coverage expected to degrade in 20s due to weather.”

  - Advantages: Reduces complexity for third parties and enables smarter application behaviour.

Such RAN exposure capabilities will empower the 6G system to support specific customizations, optimized resource usage and support emerging use cases like XR, IoT, and NTN. The 6G system should ensure exposed data is non-sensitive information.

By enabling the RAN exposure in 6G system to authorized third-party applications, we move toward a new era of proactive and adaptive service delivery, enhancing user experience and network efficiency. This opens the door for a new class of intelligent, network-context-aware applications.

#### 5.7.X.2	Existing features partly or fully covering the use case functionality

Table 5.7.X.2-1: Gap analysis for this use case

| Specifications and clauses | Examples of existing Requirements | Gap Analysis |
| --- | --- | --- |
| 22.261 [14] Clause 6.10.2 | The 5G system shall provide a means by which an MNO can inform authorised 3rd parties of changes in the - RAT type that is serving a UE. - cell ID. - RAN quality of signal information. - assigned frequency band. This information listed above shall be provided with a suitable frequency via OAM and/or 5G core network. NOTE 6: The information aids the third party user to take proactive actions so that it can achieve high service availability in delivery of its services. | These parameters exposed are limited and do not cater to extensive needs of the applications such as UE specific radio conditions and/or change in conditions and predictive network state sharing of 6G RAN |

#### 5.7.X.3	Potential New Requirements needed to support the use case

[PR 5.7.X.3-1] Subject to regulation, operator(s) policy and user consent, the 6G system shall provide mechanisms to support 6G RAN exposure such as real time radio resource capabilities, UE specific radio conditions and/or change in radio conditions and predictive network state sharing of 6G RAN to an authorized third party application.

NOTE	The 6G system should ensure exposed data is non-sensitive information.