3GPP TSG-SA WG1 Meeting #111

Goteborg, Sweden, 25-29 Aug 2025

S1-253182

Title:	Use case on intent-based orchestration of services in service hosting environment for 6G.

Agenda Item:	8.1.9

Source:	NEC

Contact:                       Hiroshi Dempo, hiroshidempo@nec.com

Deepak Rohilla, deepak.rohilla@india.nec.com

Abstract:

This use case highlights the need for intent-based orchestration in 6G networks operating across distributed telco cloud environments. It focuses on enabling AI applications to interpret high-level service intents and autonomously manage service lifecycles across multiple domains such as RAN, core and edge. The orchestration system dynamically allocates and adjusts resources in real time, ensuring service continuity.

* * * Start of 1st change * * * *

# W	Other Use Cases

Editor's Note: If there are any use cases that does not fall into any sections in this TR, these use cases could be placed in this clause, subject to further discussion. How this clause will be used is FFS.

## W.x	Use case on intent-based orchestration of services in service hosting environment for 6G.

### W.x.1		Description

As 6G networks expand across diverse infrastructure domain and 6G vision emphasizes automation, resilience, and service agility. such as RAN, core, and edge. There is a growing need for intelligent orchestration that can manage services seamlessly and autonomously. This use case focuses on enabling AI-driven, intent-based orchestration that interprets high-level service goals and dynamically allocates resources. The orchestration system leverages AI agent applications and standardized APIs to ensure service continuity, and efficient resource utilization.

In 6G, service hosting environments are envisioned as intelligent, distributed platforms that span edge, cloud, and device layers, enabling seamless deployment and management of services. AI applications play a central role in managing service lifecycle from interpreting user intents and dynamically composing services to real-time monitoring, adaptation, and optimization. Unlike 5G, where intent-based orchestration is emerging as rule based or semi-automated, 6G makes it foundational, allowing services to be autonomously tailored to user needs and environmental context. This marks a shift toward autonomous intent-driven service. To support this, the 6G system should provide mechanisms to expose standardized interfaces for AI Applications to manage service lifecycles and perform autonomous decision-making. These capabilities are essential for delivering flexible, resilient, and scalable services in complex environments, especially during high-demand or failure scenarios.

The orchestration system must dynamically allocate, scale, and migrate services based on real-time network conditions, user demands, and resource availability.

### W.x.2 	Precondition

A large-scale sports event is scheduled in a metropolitan area. The operator expects a surge in demand for low-latency video streaming and AR-based navigation services. The 6G network is deployed across RAN, edge, and core. AI-based orchestration agents are active and authorized to manage service deployment and optimization.

### W.x.3 	Service Flows

- An operator defines an intent: “Ensure low-latency immersive media services in Stadium Zone A during the event.”

- The AI orchestration agent receives the intent and queries the digital twin to simulate optimal deployment strategies.

- Based on real-time analytics and policy, the agent selects edge locations near the stadium for service placement.

- The orchestration system instantiates required network functions (e.g., UPF, MEC apps) using standardized APIs.

- During the event, the system monitors KPIs such as latency and throughput. If congestion is detected, it dynamically scales services or migrates workloads to nearby zones.

- In case of a failure in one edge zone, the system re-evaluates the digital twin and re-orchestrates services.

- After the event, the system deallocates resources and updates the digital twin with performance data for future optimization.

### W.x.4	Post-condition

The immersive media services were delivered with minimal latency and no service disruption. The orchestration system autonomously managed resource allocation and adaptation across domains, ensuring efficient use of infrastructure.

### W.x.5	Existing features partly or fully covering the use case functionality

Not applicable

### W.x.6	Potential New Requirements needed to support the use case

[PR W.x.6-1] Subject to operator’s policy, the 6G system shall support intent-based orchestration of services in service hosting environment.

[PR W.x.6-2] Subject to operator’s policy, the 6G system shall provide standardized APIs to allow AI applications to manage service lifecycle and orchestration.

[PR W.x.6-3] Subject to operator’s policy, the 6G system shall support dynamic workload placement and migration based on policies and resource availability.

* * * End of 1st change * * * *