3GPP TSG SA WG 1 Meeting #111 	 S1-253188

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:	CATT

pCR Title:	Update 5.9.6 UC on 6G Local Area Networks

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes to capture additional potential requirements, which are identified from the clause 5.9.6 use case on 6G local area networks.

1. Introduction

The use case on 6G local area network and PR1 were agreed in SA1#110 meeting. Regarding the description of the use case, the requirements related to the following aspects need to be captured.

- What 3GPP services can be provided by which network when both local area network and PLMN coexist according to the Scenario2 and Scenario3.

- How to maintain the service when the service provider is changed between PLMN and local area network caused by the user’s mobility regarding Scenario3.

This contribution intends to introduce the relevant requirements.

2. Reason for Change

1. Editorial change for the missing figure reference and detailed illustration about the relation between the figure and the use case.

2. Introduce two additional requirements.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.870 v0.3.1

* * * First Change * * * *

5.9.6	Use case on 6G Local Area Networks

5.9.6.1	Description

Local mobile communication networks have emerged as a viable and attractive deployment model for vertical scenarios like factories or campuses since the 4G era. Characterized by scenario-specific requirements, limited coverage and well-defined user groups, these local networks have empowered diverse local stakeholders to build and manage their customized networks - whether private or public - for distinct business needs [210]. The 5G era continues the exploration of expanding vertical markets beyond consumers for new revenue through more standardized technical enablers such as network slicing, non-public networks (NPN), 5G-LAN type services. However, the current realization remains constrained by a holistic “one-system-ﬁts-all” philosophy, which gradually leads to an increase in the complexity of 5G systems. Thus, it is difficult for the network tenders e.g. private service providers to select the best option and also impossible for new and smaller players to bring in their innovations [211]. The mismatch between technical complexity (coupled with high investment and operational cost) and fragmented demands of diverse verticals has caused 5G local mobile communication networks to lag behind initial market expectations. The 6G system is expected to address the above issues through simplifying network, natively supporting deployment options with smaller granularity and modular solution tailored for vertical use cases.

6G, envisioned as a general-purpose technology platform integrating communication with sensing, positioning, AI, and computing capabilities via multiple accesses, will unlock a vast array of new use cases as mentioned in [39]. The transition from dedicated communication systems to versatile and intelligent infrastructure will fundamentally reshape the business models and associated design paradigms of local area networks in,

-	Business value evolution from connectivity to capabilities as service offerings: for example, besides the connectivity, the local area networks are promising to play an important role in AI-driven application (e.g. collaborative robots of smart factories, intelligent hospital) such as providing on-demand computing and sensing capabilities for edge AI, or coordination of the communication and computing for edge-cloud collaborative AI. Thus, those local area networks need to be able to close to the end devices as much as possible for optimized latency and relatively better security protection in some cases while it is able to support interoperability with remote devices or platform in some cases.

-	Growing divergence in the demands of local stakeholders: 6G communication performance requirements will become increasingly diverse across the scenarios like immersive communication, ubiquitous connectivity, massive communication and intelligent application in each dimension such as data rate, latency, reliability, availability, etc. The integration of non-connectivity capabilities such as sensing, computing, AI will gradually unlock new use cases and further accelerates the diversification of their service expectations. Thus, the differentiated and changing demands on network services of various local stakeholders will intensify challenges in network complexity and flexibility, which must be addressed in the early stage of system design to enable the customized, adaptable and agile deployment with optimized cost.

This following use case illustrates how 6G local area networks serve corresponding local users with simplified and flexible deployment.

5.9.6.2	Pre-conditions

Network Operator OpX has deployed PLMN in the country A and supports the subscribers to access the PLMN via multiple accesses such as terrestrial access, satellite access as Figure 5.9.6.3-1 shows.

OpX is also capable of offering customized LAN service, therefore

-	The Manufacturer MfX has signed SLA with OpX to deploy private local area network (e.g. LAN#1) for communication service, AI services and computing services.

-	The event organizer has signed SLA with OpX to deploy local area network (e.g. LAN#2) during the event period. The offered network services need to be activated on demand.

-	The dock operator has signed SLA with OpX to deploy public local area network (e.g. LAN#3) for basic communication services (e.g. voice, limited-data-rate data ) and sensing services. All the users entering this area can get network services with the authorization of the dock operator.

Alice and Bob have the subscription with OpX for communication services.

5.9.6.3	Service Flows

Figure 5.9.6.3-1: Differentiated 6G Local Area Network

Scenario1: Private local area network in cost-efficient way

Regarding the request of smart manufacture plant, OpX deploys a local area network (LAN#1) with dedicated functionalities to support low latency communications and edge AI services in cost-efficient way.

The AMRs and industrial robots can execute various tasks in form of groups under the monitoring of central control platform. The collaborative robots can be authorized to communicate within individual groups but not allowed to communicate to the members of other groups without access permission. When a new production line is introduced with strict requirements on deterministic latency, LAN#1 will autonomously initiate the upgrading with provisioning new functionalities and/or policies once the robot of new production line accesses for the service or based on OAM’s request.

Scenario2: Temporary local area network

A sports event will be held in the local stadium, which has a capacity of more than 50,000 audiences. At the moment, there will be around hundreds of thousands of audience watching the live remotely via VR/AR devices, small screens (e.g. smartphones) or large screens. The entire event will be digitally operated and managed– from monitoring athletes’ conditions to collecting and disseminating event information.

OpX deploys a local area network (LAN#2) based on existing base stations and computing resources of PLMN and non-3GPP access with configured validity period (e.g. duration of the event). The staff were authorized to interact with each other within individual work groups via LAN#2 while the audiences (e.g. Alice) within the stadium were authorized to connect to LAN#2 to get latest event information, watch holographic presence and communicate with remote friends (e.g. Bob). The users outside the stadium can be only allowed to  access PLMN via the base stations as usual, but cannot access LAN#2. Meanwhile, the remote audience (e.g. Bob) enjoy the live in immersive experience as if they attend the event in person. When the event finishes, the network resources of LAN#2 will be automatically returned to PLMN without any impact on other services.

Scenario3: Service Continuity between PLMN and local area networks

OpX deploys a local area network (LAN#3) on cargo dock with dedicated functionalities for high reliable communication, massive IoT communication and 3GPP sensing service. When a truck is authorized to enter this area for landing with goods, it is also authorized to access LAN#3 to report the information to the local authority platform as well as remote application platform simultaneously. When it leaves this area, the truck is not allowed to access LAN#3 so continue reporting sensor data to remote application platform through PLMN.

Bob, the fleet manager on truck is authorized to answer the voice call from Alice via LAN#3 during the truck is unloading the goods. Meanwhile, based on the dock operator’s policies, his UE is not allowed to  connect to the live of the sports event via LAN#3 so still through PLMN during the truck enters in and out of this area.

The stadium mentioned in Scenario2 is not far from the dock. When the truck finishes the unloading, Bob is navigated to the stadium following Alice’s instruction in the ongoing voice call via PLMN until meet each other in the stadium, and watches the rest of the sport event in live via LAN#2.

5.9.6.4	Post-conditions

Thanks to 6G local area networks, the smart manufacture plant and the cargo dock can operator efficiently in an orderly manner while the staff and the audiences relevant to the sports events are happy with network services.

Alice and Bob can communicate successfully via local area network and PLMN.

5.9.6.5	Existing features partly or fully covering the use case functionality

TS 22.261 [14] introduces several features to serve the diverse needs of regional/local area networks, such as

(1) 5G LAN

TS 22.261 [14] has defined 5G LAN-type service and 5G LAN-virtual network as below,

5G LAN-type service: a service over the 5G system offering private communication using IP and/or non-IP type communications.

5G LAN-virtual network: a virtual network capable of supporting 5G LAN-type service.

And clause 6.26.1 describes the characteristics of 5G LAN as

There are multiple market segments in the realm of residential, office, enterprise and factory, where 5G will need to provide services with similar functionalities to Local Area Networks (LANs) and VPN’s but improved with 5G capabilities (e.g. high performance, long distance access, mobility and security).

5G LAN-VN can be scaled up/down, created/removed and managed by the network operator on demand. Also, the UEs of 5G LAN-VN can be dynamically added or removed.

However, 5G LAN-VNs heavily rely on the robustness and resilience of PLMN infrastructure. Any failure or disruption of PLMN could have a widespread impact on the users of local area network - particularly those with stringent requirements for service isolation and data security such as industrial enterprise. Moreover, the sharing functionalities hinder the potential simplification by some-level customization or function tailor.

(2) Network Slice

TS 22.261 [14] has defined network slice as below,

network slice: a set of network functions and corresponding resources necessary to provide the required telecommunication services and network capabilities.

And clause 6.1.1 describes the characteristics of network slice as

Network slicing allows the operator to provide customised networks.

A network slice can provide the functionality of a complete network, including radio access network functions, core network functions (e.g. potentially from different vendors) and IMS functions. One network can support one or several network slices.

However, a network slice is a logical network that will share the similar infrastructure and network functionalities with other network slices. Once an instance of network slice is created and deployed, the offered capabilities and services cannot be changed. Also, the granularity of network slice is still too big for smaller operators to deploy for limited service demands of specific vertical needs.

(3) NPN

TS 22.261 [14] has defined non-public network as below,

non-public network: a network that is intended for non-public use.

And clause 6.25.1 indicates that

Non-public networks are intended for the sole use of a private entity such as an enterprise, and can be deployed in a variety of configurations, utilising both virtual and physical elements. Specifically, they can be deployed as completely standalone networks, they can be hosted by a PLMN, or they can be offered as a slice of a PLMN.

Standalone NPN can provide specific services and good security to dedicated users via customized functionalities and physical isolation. But when SNPN users intend to access to PLMN or other SNPNs, they still faces challenges in the authentication (using the isolated credentials) and service continuity (due to inconsistent policies, QoS), which has not been well solved in 5G.

5.9.6.6	Potential New Requirements needed to support the use case

[PR 5.9.6.6-1] Subject to operator’s policy and agreement with 3rd party, the 6G network shall support a mechanism to start and stop offering certain network service(s) in a local area network adapting to the demand of e.g. the users, 3rd party or the network operator.

[PR 5.9.6.6-2] Subject to operator policies, and an agreement between the PLMN operator and authorized 3rd party, the 6G network shall support a mechanism to authorize PLMN’s subscribers to access to a subscribed service provided by authorized 3rd party via a local area network (deployed by PLMN operator) e.g. considering serving geographic area, time duration, etc.

[PR 5.9.6.6-3] Subject to operator policies, and an agreement between the PLMN operator and authorized 3rd party, the 6G network shall support a mechanism to maintain service continuity of a subscribed service for the PLMN’s subscribers when the ongoing service is changed across a PLMN or a local area network (deployed by PLMN operator).

* * * End of Changes * * * *