3GPP TSG-SA WG1 Meeting #111 	S1-253206

Goteborg, Sweden, 25-29 August 2025

Title:	Update data collection use case 5.9

Agenda Item:	8.1.2

Source:	Nokia

Contact:	Laurent-Walter Goix

Abstract: This contribution proposes to update the use case and its requirements, as well as clarifying the definition of the “6G System Data”. Part of the description has been simplified eg:

- The notion of data “types” is not mentioned earlier and can be seen as a solution-oriented approach which is not needed for now

- Normative wording not to be used in the conclusion of the descriptive text, and the same meaning is already listed in the 7/8 bullets

2 new requireemtns were added to stress the need to optimize UE/network energy consumption when collectign and processign data.

---------- Use Case template ----------

* * * First Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

6G Computing Service: A service provided by 6G network utilizing computing resources in Service Hosting Environment, which can be used by a subscriber (via UE)/3rd party.

6G System Data: the data that is generated or controlled by the 6G system.

NOTE 1: 6G System Data is different from traditional user traffic data which is application level data being transmitted through the 3GPP system for user related services.

NOTE 1a:	6G System Data can include 3rd party data which is imported into the 6G system for internal use (e.g. grid energy mix or price imported into OAM)

Editor’s Note: The definition of 6G System Data is FFS.

6G Wireless sensing: 6G system feature providing capabilities to get information about characteristics of the environment and/or objects within the environment (e.g. shape, size, orientation, speed, location, distances or relative motion between objects, etc) using radio frequency signals.

NOTE 2: 	The 6G Wireless sensing service can use data acquired with either NR-based radio signals, non-3GPP radio signals, or a combination.

Al Agent: an automated intelligent entity capable of e.g interacting with its environment, acquiring contextual information, reasoning, self-learning, decision-making, executing tasks (autonomously or in collaboration with other Al Agents) to achieve a specific goal.

Editor’s Note: This definition could be updated as the study goes on to align with the potential new understand of AI agent.

AI service: an 6G service (e.g. AI model inference) to support AI-related activities, which is provided by 6G core network to the 6G user, considering the required quality of the service (e.g. inference accuracy, latency).

NOTE 3:	The above term does not imply any architectural assumption, e.g. whether 6G CN is a new or evolved CN (compared to 5G).

Editor’s Note:	The definition of AI service is FFS.

Digital Twin: A real-time representation of physical assets in a digital world.

NOTE 4: This definition was taken from ITU-T Recommendation Y.3090 [113].

Energy Supply: The delivery of electricity to a physical location. This is typically realized by placing two or more wires coming from a DSO at a geographical location and connecting those wires to a metering device.

NOTE 5:	This definition was taken from TS 28.318 [232].

Intelligent Communication Assistant: The virtual intelligent communication assistant locates in operator network and interacts with the users through voice, video, text, gestures or other modalities. The assistant can be customized for each particular user by accessing user data stored in the network, with user’s consent. It can provide various communication services and support individual users based on user’s intention and requirement utilizing AI capability. One subscriber can have one or more Intelligent Communication Assistants.

Intent: Expectations including requirements, goals and constraints without specifying how to achieve them. [147]

NOTE 6: Intent can be used for 6G services as well as OAM.

Editor’s note: NOTE 6 is FFS.

Editor’s note: this definition is FFS for further enhancement along the study goes on. If more detail regarding Intent is necessary to support the use cases in this TR it may be introduced in an Annex.

Maximum slice energy credit limit: a policy establishing an upper bound on the aggregate quantity of energy consumption by the 6G system to provide services for a specific slice, e.g. in kilowatt hours.

Network Digital Twin: virtual replica of (part of) a mobile network to emulate (or simulate) the behaviour of the actual network.

Editor’s Note: it is FFS to update this definition.

Network Federation: Refers to the interoperability of two or more 6G networks, enabling them to share resources and services, to achieve shared objectives. Federated 6G networks maintain their autonomy but coordinate to share resources, or services, ensuring mutual benefits without compromising individual operational control or data privacy.

NOTE 7: 	Network federation is currently defined in TS 28.538 [257], TS 23.558 [52] and allows MNOs to share edge computing resources.

non-3GPP sensing station: a device capable of emitting and/or receiving radio signals that can result in acquisition of sensing data.

Personal Data: any information relating to a user or subscriber that can be used to, either directly or indirectly, identify that user or subscriber, or to distinguish that user or subscriber from others.

Sensing target density: total number of objects to be sensed per geographic area. It is a measure of how many objects the 3GPP system can detect, identify and/or track within a target sensing area.

* * * Second Change * * * *

### 5.9.2	Efficient data collection and consumption for 6G system

#### 5.9.2.1	Description

To better serve the users and manage the network, as well as provide non-connectivity service (e.g. LCS), 3GPP system need to conduct collection of data associated with the network and services provided by the 3GPP system. The data collection in the 5GA network for network/service management and non-connectivity service is defined for specific use cases, resulting in varying solutions across different use cases, different layers and different domains. Here is a brief summary of different data collection work done in RAN/SA2/SA4 and corresponding security mechanism in SA3:

-	In RAN domain, data collection from UE is based on MDT (Minimization of drive testing) method as defined in TS 37.320 [237] and TS 38.331 [238]. The user consent of UE location data collection for MDT is defined in TS 32.422 [239].

-	For data collection for AI/ML in core network, the data collection feature permits NWDAF to retrieve data from various sources (e.g. NF such as AMF, SMF, PCF, NSACF, GMLC, and AF, OAM, etc.), as a basis of the computation of network analytics as defined in TS 23.288 [114]. The operator can leverage these network analytics to optimize the management of network operations. For instance, as outlined in clause 6.22 of TS 23.288 [114], the operator can collect network data (such as OAM data and CP signalling) to detect or predict a signalling storm and use the analytics to mitigate or prevent its impact. The data collection is performed via SBI between CN NFs, DCCF (Data Collection Coordination Function) is introduced as logical functionality to coordinate data collection in CN, and ADRF (Analytics Data Repository Function) is defined to store the collected data. The user consent of UE data collection for AI/ML is defined in TS 33.501 [250].

-	For UE positioning, the input data is collected from UE/RAN to CN via CP/UP path as the basis of the computation of UE location as defined in TS 23.273 [240]. The privacy check of UE location collection is also defined in TS 23.273 [240].

-	To collect data and to expose data between CN domain and AF domain, the network exposure mechanism in core network has been defined in clause 5.20, TS 23.501 [140] and clause 4.15, TS 23.502 [30], where the NEF is used as the termination node to isolate both sides. Especially for edge, exposure of UE data is defined in TS 33.558 [241].

-	If the data collection is triggered by OTT server in application layer from UE, the UP path may be used with the assistance of DCAF, as defined in TS 26.531 [242].

The aforementioned mechanisms are defined for different network functions to support data collection in different scenarios, leading to high standardization overhead. There is lack of coordination between these network functions, leading to isolated data collection and duplicated data collection. Meanwhile, the data transmission based on the control plane are not suitable for transmitting large amounts of data, leading to inefficient data transmission. Therefore, the data collection and control mechanism need to be improved in 6G.

Table 5.9.2.1-1 lists several potential use cases for data collection in 6G, and illustrates the corresponding data type, data provider, data consumer and data volumes for each use case. Take AI/ML data as example, there is a large amount of data types defined in 3GPP apart from those non-standardized ones, and the scale of data volume is usually large (from 10k to 100M).

Table 5.9.2.1-1: Heterogeneous 6G System Data

| Use case category | Data types | Data provider | Data Consumer | Data volume |
| --- | --- | --- | --- | --- |
| Sensing data collection/sensing result exposure (e.g. measurement) | Sensing measurement data requires further study and discussion. The potential sensing measurement can be channel estimation results, delay, Doppler, angle, signal strength, etc. | UE, RAN | Sensing Function | Related to use cases: e.g. data rate of measurement is about 10Mbps for objects creating hazards on roads |
| AI/ML data collection / analytics exposure (e.g. AI/ML for air interface, AI/ML for core network) | - For RAN data collection for Air interface, see TR 38.843 [243] - For core network data collection, see the data collection framework in clause 6.2, TS 23.288 [114], and the detailed data type are defined from clause 6.3 to clause 6.21, TS 23.288 [114]. | CN, RAN, UE, AF | CN, RAN, UE | Typically, the internal data collection for AI model training yields tens of thousands of samples. Related to AI model: e.g. the data volume is about 10k bytes to 100M bytes for AI model of beam management |
| Energy Efficiency (EE information exposure to AF) | still under the discussion in SA2 R19 EnergySys WID | CN, OAM | EIF (Energy Information Function) | Related to use cases: e.g. could be similar to data volume for charging |
| Positioning, | See clause 6, TS 23.273 [240] | UE, RAN | LMF, UE | Related to use cases |
| Network Exposure in core network | See clause 5. 20, TS 23.501 [140], and clause 4.15, TS 23.502 [30] | 5GC NF, AF | 5GC NF, AF | Related to use cases |
| … |  | … | … | … |

All the above use cases for data collection and control share the following commonality：

- The data collection and control mechanisms need to support procedures such as data collection, data storage, data exposure and data deletion.

- The collected data could be requested or consumed by multiple data consumers within 3GPP system to support multiple services; for example, a single spatiotemporal environmental dataset can be concurrently utilized by multiple services, including IoT-enabled pervasive sensing service, latency-sensitive extended reality (XR) applications requiring <10ms motion-to-photon delays, and intelligent transportation systems (ITS) with centimetre-level geolocation precision.

- The upper bond of the collected data volume is very high, e.g. on the order of 100 M byte;

- Latency requirements for data collection and transmission exhibit variations across diverse use cases.

- The collected data transmission path between different entities within 3GPP system can vary widely, encompassing scenarios such as UE to RAN node，UE to CN node, RAN node to CN node, and CN node to CN node;

- UE related data collection  needs to consider security, privacy protection, user consent and UE energy consumption.

- Data exposure needs to consider operator policy and user consent.

- Collecting, processing and consuming data can consume a lot of energy if not properly optimized to the needs.

Figure 5.9.2.1-1: Use case specific data mechanisms in 5G è common data mechanisms in 6G

As shown in Figure 5.9.2.1-1, use case specific mechanisms are defined for each service scenario in 5G, leading to system complexity and inefficient data collection. As 6G networks integrate advanced capabilities progressively, such as AI, enhanced computing power, multi-dimensional sensing and digital twin network, they will need to process massive amounts of multi-source, heterogeneous data (from diverse formats and data resource) generated by 6G system. Consequently, 6G system needs to support efficient data collection and control mechanisms capable of addressing both universal requirements and scenario-specific demands across heterogeneous use cases such as AI/ML, sensing, XR service and network digital twin via various data providers and consumers.

#### 5.9.2.2	Potential New Requirements

Editor’s Note: user consent in this section is FFS.

[PR 5.9.2.2-1] The 6G system shall support mechanisms for 6G System Data collection and consumption minimizing the impact to 6G services.

[PR 5.9.2.2-2] Subject to operator’s policy, regulation and user consent, the 6G system shall support processing of 6G System Data.

NOTE:	examples of data processing are use case dependant, e.g. data fusion, data anonymization and data analysis.

[PR 5.9.2.2-3] Subject to user consent, regulation and operator's policy, the 6G system shall support secure means to provide authorized access to 6G System Data towards third-party, network function or UE.

[PR 5.9.2.2-4] The 6G system shall support security and privacy protection of the 6G System Data.

[PR 5.9.2.2-5] The 6G system shall be able to provide charging and accounting mechanisms for 6G System Data.

[PR 5.9.2.2-6] The 6G network shall support mechanisms to optimize network energy consumption when collecting, processing and consuming data.

[PR 5.9.2.2-7] The 6G system shall support mechanisms to optimize UE energy consumption when collecting data from UE.