3GPP TSG SA WG 1 Meeting #111 	S1-253210

Goteborg, Sweden, 25 -29 August 2025	(revision of S1-25xxxx)

Source:	ESA, Fraunhofer, Airbus, TNO, Thales, Novamint

Title:	New use case on high-altitude UAV supervision

Draft Spec:	3GPP TR 22.870 V0.3.1

Agenda item:	8.1.4 Integrated Sensing and Communication (FS_6G_REQ)

Document for:	Discussion/Approval

Contact:	relja.djapic@tno.nl

Abstract: This document proposes to address integrated sensing and communication from space using NTN and introduces a new use case on high-altitude UAV supervision.

1. Introduction

Integrated Sensing and Communication (ISAC) is an important element of 6G as reflected in Chapter 7 of TR 22.870. However, the use cases so far explicitly only refer to the use of terrestrial networks (TN) for ISAC. As the field of ISAC from space is rapidly emerging ([X1], [X2]) it becomes apparent that also non-terrestrial network (NTN) can support integrated sensing application.

The following table highlights the ISAC use cases currently in TR 22.870 V0.3.1 that would benefit from NTN contributions in particular (but not restricted to) remote areas:

| ID | Use case on | Remarks |
| --- | --- | --- |
| 7.1 | coordination of search and rescue missions in large disaster areas | The satellite assets of a NTN Satellites can provide significant contributions to coordination in disaster situations in particular in remote areas and if the operation of the TN is impaired by the disaster (e.g. loss of terrestrial base stations, …) |
| 7.3 | high-resolution topographical maps | NTN can provide additional sensor layers to improve accuracy of topographical maps |
| 7.6 | road digitalization | NTN ISAC can complement by provisioning suitable traffic flow data in remote areas with a scarcity of terrestrial base stations. |
| 7.9 | advanced modern city transportation system | Analogously to 7.6 NTN can complement TN ISAC services for providing additional data to Intelligent Transportation Systems |
| 7.10 | stored sensing data handling | NTN can provide additional sensing data upon request especially in barely accessible remote regions |
| 7.14 | infrastructure collapse monitoring | Infrastructure monitoring and change detection is an estalbished Earth Observation use cas, NTN can contribute to pre-event monitoring through wide area surveillance and automated change detection |
| 7.15 | multi-sensor fusion based sensing for UAV takeoff and landing | NTN ISAC results can be fused into the results from other (terrestrial) sensors to improve accuracy |
| 7.17 | safe & economic UAV transport | UAV transport is in particular viable in rural regions where TN coverage can be sparse. NTN coverage can complement with sensing data for safe UAV operation even in these areas |
| 7.20 | autonomous driving based on network-assisted sensing | Sensing data obtained by the 3GPP network – both TN and NTN – together with the vehicle’s onboard sensors can very advantageous for the safe operation of the autonomous vehicle |

The use case 7.4 on  the low-altitude UAV supervision is relying on ISAC with terrestrial base stations, which are limited in reach. Utilizing NTN UAV supervision can also be performed at high altitudes.

Consequently, this contribution proposes a new use case supervision of UAVs in high-altitudes, based on ISAC from space, i.e., based on the usage of  NTN domain.

2. Reason for Change

Ensure that the capabilities of Integrated Sensing and Communications from space, i.e. by NTN, will be supported by 6G networks.

3. Conclusions

Add a new use case on Integrated Sensing and Communications from space (NTN) in TR 22.870.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 V0.3.1.

* * * 1st Change * * * *

# 2	References

Add the following references to Clause 2:

[X1]	M. Corici et al. "Repurposable payloads: concept, design, and initial results," 41st International Communications Satellite Systems Conference (ICSSC 2024), Seattle, USA, 2024, pp. 120-124, doi: 10.1049/icp.2024.4622.

[X2]	J. Kim et al, “Integrated Sensing and Communication from Space – Satellite Payload Hardware Aspects”, Proceedings of 2025 IEEE Radar Conference, Krakow, Poland, 2025,

* * * 2nd Change * * * *

## 7.x	Use case on high-altitude UAV supervision

### 7.x.1	Description

With the development of Uncrewed Aerial Vehicles (UAV) technologies, light and small civilian UAVs have played a great role in aerial photography, agriculture, mapping and other fields. Various commercial UAV applications are now becoming a reality. Some of these UAVs operate at high altitudes often in the same airspace as commercial aircraft. This may produce a series of safety control problems, e.g. UAV illegal intrusion and UAV collision. In addition, surveillance of commercial air traffic in remote areas (oceans, mountains, deserts) remains a challenge and suitable data supplementary to data provided by existing ATC/ATM (air traffic control/ air traffic management) systems will be welcome by the relevant service providers and flight safety authorities. The integrated sensing capabilities of 6G can support the collection of suitable data for air traffic and UAV management. Additional details in particular on the detection, supervision and collision avoidance of UAV can be found in section 7.4.1 and its application to high-altitude UAVs and commercial aviation can easily be inferred.

### 7.x.2	Pre-conditions

Satellite Network Operator NN provides 6G sensing service in high-altitude airspace for air traffic management and UAV flight assistance service, including flight trajectory tracing, collision prediction, illegal UAV intrusion detection and etc. NN can make use of NTN satellites to sense the airspace within their coverage area and report the sensing information (including tracked UAV and aircraft as well as the environment around them) to the respective authorities and service suppliers such as – ATM, USS (Uncrewed Aerial System Service Supplier)/UTM.

The Company MM uses the USS/UTM to supervise the high-altitude UAVs..

The USS/UMT uses 6G sensing service provided by the 6G Satellite Network Operator NN to track the flight trajectory and also for potential UAV illegal airspace intrusion detection and UAV collision prediction.

Air Navigation service providers (ANSPs) use the 6G sensing service provided by the 6G Network Operator NN as an auxiliary source of data for providing ATM/ATC services

The UAV Operator/UTM provides specific details to the 6G Network Operator NN, including the characteristics of the UAV that will be tracked, along with details about the time and location for flight tracing. This information includes regulated flight paths as well as potential areas where the UAV might temporarily deviate from its route.

ANSPs may provide specific details to the 6G Network Operator NN about air traffic environment, flight paths and other characteristics.

The 6G Satellite Network Operator NN can realize 6G data collection, 6G data transmission, 6G data processing, 6G data storage within the network, and provide sensing results to the ATM/USS/UMT/UAV.

### 7.x.3	Service Flows

For high-altitude UAV supervision:

1.	The Company MM requests 6G sensing service for high-altitude UAV from the USS/UTM.

2.	The USS/UTM transmits the request to the 6G Satellite Network Operator NN.

3.	The 6G Satellite Network Operator NN utilizes its 6G NTN assets to track the high-altitude UAV of Company MM constantly collecting sensing data of the UAVs location.

4.	The 6G Network aggregates, forwards and transmits the data generated by the base stations, and processes the data to obtain the sensing results.

5.	The 6G Network exposes the sensing result to the USS/UTM. The USS/UTM could trigger to send warning and control messages to the UAV and if necessary to take corrective action and redirect the UAV.

For supplementary ATM data provision:

1.	When the scheduled time for tracking begins, the 6G Network Operator activates the aircraft trajectory tracing service within the designated area until the tracking session ends.

2.	Using radio sensing, a network of 6G TN and NTN base stations and connected devices (UEs) detect aircraft and continuously gather data on its position and movement, such as distance, velocity and angle. These metrics, also known as 3GPP sensing data, are sent to a 6G processing unit for real-time analysis.

3.	During the flight, if the aircraft leaves the coverage range of one TN or NTN base station and enters a new coverage zone, the 6G system could let the old base station stop radio sensing, and switch to a new base station for sensing until it is out of coverage. This transition is based on the aircraft's estimated position and velocity, which the 6G processing unit calculates. The network can automatically adjust the sensing operations at base stations depending on this data or based on a pre-defined time frame. In certain cases, sensing handover may be triggered to maintain continuous coverage. For instance, if the current base station's connection weakens or if another nearby base station can offer better coverage for the aircraft, the system proactively shifts the sensing function to this new station to ensure uninterrupted tracking.

4.	The 6G processing unit can aggregate sensing data from multiple sources, including RANs and UEs, to estimate aircraft's location and velocity. Similar approach can also be applied to multiple aircraft, in the case there are multiple aircraft service in the area. This real-time information is then transmitted to the ANSP, who monitors the aircraft's trajectory.

5.	If the respective aircraft deviate from prescribed routes, the ANSP receives alerts, allowing them to take corrective action and redirect the aircraft as necessary. This is especially valid for remote airspaces such as over oceans.

### 7.x.4	Post-conditions

For high-altitude UAV supervision: UAV#1 efficiently and safely follows the tracked flight route to its destination; any off-route movements are detected and can be quickly corrected. Thanks to the wide-area and constant sensing capability of the 6G NTN satellites, and the efficient data transmission, processing and storage by the 6G core network, the safe supervision of the high-altitude UAV of Company MM is improved.

For supplementary ATM data provision: The supplementary ATM data are included in the ATM activities of the ANSPs to provide safe and efficient air traffic control and to detect and correct any deviations in a timely manner.

### 7.x.5	Existing features partly or fully covering the use case functionality

In TS 22.137 [6], there are requirements specified for 5G system on integration of sensing and communication.

Based on operator’s policies, operator’s control and regulation, the 5G system shall be able to collect 3GPP sensing data from sensing receivers for processing.

Subject to user consent, regulation, and operator’s policy, the 5G system should support the joint processing of the 3GPP sensing data and non-3GPP sensing data to derive a combined sensing result.

Subject to operator’s policy, the 5G network shall provide secure means for a trusted third-party to request 5G wireless sensing service based on specific parameters (e.g., refresh rate, period of time, sensing KPIs, geographical location) and to receive the corresponding sensing results.

### 7.x.6	Potential New Requirements needed to support the use case

[PR 7.x.6-1] Subject to operator policy, the 6G network shall enable the base station to send sensing measurement data to the core network, and enable the core network to aggregate, collect, process, and store sensing measurement data from base stations.

[PR 7.x.6-2] The 6G system should provide mechanisms to ensure sensing service is able to be provided with a given sensing system capacity.

NOTE 6:	The term 'sensing system capacity' is the maximum number of targets that can be detected per unit area given sensing QoS requirements per target, which include localization accuracy and sensing service latency [11].

[PR 7.x.6-3] The 6G system shall be able to provide sensing with following KPIs as derived from UAV/Air Traffic Management requirements (to be confirmed). The achievable sensing performance of the 6G system has to apply to varying size of high-altitude UAVs and aircraft, however, in contrast to low-altitude UAVs they will have lengths/wing spans of least 10 meters:

Table 7.x1.6-1:	Performance requirements of sensing results for high-altitude UAV supervision and ATM support

| Scenario |  | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate (Hz) | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s] |  |  |  |  |
| High-altitude UAV supervision | UAV intrusion detection | Outdoor | ≥95 | ≤10 | ≤10 | N/A | N/A | [10] | [≥5] | [≤1000] | [≤1] | [≤5] | [≤5] |
|  | UAV trajectory tracking | Outdoor | ≥90 | 1-2 | 1-2 | 3-5 | 3-5 | N/A | N/A | 100~1000 | ≤1 | ≤5 | ≤5 |
| ATM supplementary data provision |  | Outdoor | ≥95 | ≤10 | ≤10 | 3-5 | 3-5 | N/A | N/A | < 1500 | 1 | ≤1 | ≤1 |