3GPP TSG-SA WG1 Meeting #111 	S1-253254

Goteborg, Sweden, 25-29 August 2025

Title:	New Use case on Energy-aware Network API fulfilment considering UE involvement preference

Draft TS/TR:	3GPP TR 22.870 v.0.3.1

Agenda item:	8.1.2

Document for:	Approval

Source:	Nokia

Contact:	Laurent-Walter Goix

Abstract: This contribution proposes to introduce a new use case and requirements related to energy-aware Network APIs. Specifically, it introduces some evaluation of the amount of energy/carbon related to a specific API request, so it can be provided by the provider and requested as criteria by the consumer. In addition, a UE involvment preference is defined to temporarily avoid to involve the UE when fulfilling some network API request which could drain the device battery.

---------- Use Case template ----------

FIRST CHANGE (new text)

### 5.8.x	Use case on Energy-aware Network API fulfilment considering UE involvement preference

#### 5.8.x.1	Description

5G provides several network services to consumers, like location services, and one of means is through Network APIs.  Several new services such as sensing, high precision positioning, and AI-based services are envisioned in 6G.  These services may be energy intensive or require UE involvement.

When Network APIs are invoked by service consumers (e.g. AF, applications residing in the UE or enterprises/industrial applications), data from UE and different network domains (RAN, Core, OAM) may be required in order to fulfil the request (e.g. Location API might be fulfilled by various methods such as DL-TDOA; which involves data from UE while UL-AoA might not involve data from UE). These have a direct impact on the energy consumption of the network and UE, especially over long periods of time for e.g. tracking services.

In addition, the network nowadays decides on its own whether to involve the UE or not based on the specific API (e.g. location API depending upon the positioning method used), on the consumer’s request parameters or on the producer’s internal logic.  UE involvement may increase UE energy consumption which may not be the user preference when e.g. the UE battery is low, due to privacy concerns, roaming situation or other scenarios.

#### 5.8.x.2	Pre-conditions

John can set a preference to minimize the solicitation of its UE by network services e.g.  based on its battery level or roaming status.

#### 5.8.x.3	Service Flows

- John, based on its UE energy level e.g. low battery, indicated to the network its UE involvement preference settings for network API services.

- John’s UE requests the energy information (e.g. level of consumption, efficiency, CO2e etc) of the network API service that it requests

- The network considers the UE involvement preference to select the best method for fulfilling the service and returns the energy information (e.g. level of consumption, efficiency, etc) of the network and UE for fulfilling this service. The level of energy consumption or carbon emission caused in the network can be abstracted (e.g. low, medium, high).

- If some energy related criteria are provided in the request from the UE (e.g. the UE does not want to use a “high” energy consuming service), the specified energy related criteria are considered by the network when selecting the method for fulfilling the service.

#### 5.8.x.4	Post-conditions

The  network takes into account John’s preference in UE involvement preference settings for network API services as well as energy criteria to fulfil network services requested by John’s UE.

#### 5.8.x.5	Existing features partly or fully covering the use cases functionality

Network APIs offer the possibility to program the network and services to meet the needs of the end consumers.  These APIs are being standardized by e.g. 3GPP via NEF/AEF, operational APIs by TMForum [r3] and service APIs by CAMARA [r1].  Such network services may be requested via Intents too.

CAMARA is an open-source project within Linux Foundation to define, develop and test simplified APIs that allow to access network capabilities. CAMARA works in close collaboration with the GSMA Operator Platform Group [r2] to align API requirements and publish API definitions. In addition, billable concepts/events in CAMARA API from GSMA Open Gateway Business Stream are defined.

#### 5.8.x.6	Potential New Requirements needed to support the use case

[PR 5.8.x.6-1] Subject to operator’s policy, user consent and regulatory requirements, the 6G system shall provide means to temporarily exclude UE involvement when fulfilling network API services.

NOTE 1:	A user can prefer to not involve its UE in API service fulfilment of the network due to e.g., low battery, privacy concerns, roaming status etc. Based on this preference, the network determines the method of network API service fulfilment, which can result degraded or not achievable.

[PR 5.8.x.6-2] Subject to operator’s policy and regulatory requirements, the 6G system shall provide means for a network API service to indicate to the service consumer a prediction or estimation of the energy consumed and/or carbon emission equivalent generated by a service request.

NOTE 2:	The service consumer can utilize such energy related information to adjust the service characteristics it requests through network APIs.

[PR 5.8.x.6-3] Subject to operator’s policy and regulatory requirements, the 6G system shall provide means for a network API service to receive consumer’s energy related preferences or criteria as part of the network API request (e.g. maximum energy or CO2e) and consider these preferences or criteria for the fulfilment of the service.

SECOND CHANGE (add the following references to clause 2)

[r1] CAMARA:  CAMARA API

[r2] GSMA: " GSMA Open gateway API”  (GSMA Open gateway API)

[r3] TMForum: “TMForum API directory” (TMForum API directory)

END OF CHANGES