3GPP TSG-SA WG1 Meeting #111 	S1-253260

Goteborg, Sweden, 25-29 August 2025	(revision of S1-25xxxx)

Title:	ATSSS Enhancements

Agenda Item:	8.1.2 (System and Operation Aspects)

Source:	Lenovo

Contact:	Apostolis Salkintzis (y1026c at lenovo.com)

Abstract: This contribution proposes an enhancement of the multi-access connectivity capabilities for 6G systems. It addresses the limitations of the existing 5G ATSSS and defines new requirements for the 6G system to adopt a more efficient traffic steering strategy.

NOTE:    	This contribution has been partially supported by the HORIZON-JU-SNS-2023 EU project ENVELOPE (GA Number 101139048).

## 5.7 	6G enhancements of legacy/existing services and capabilities

Editor’s Note:  Text to be provided.  This clause to address potential (new) requirements for new (6G) aspects of existing (5G and earlier) services or capabilities.

* * * First Change (ALL NEW TEXT) * * *

## 5.7.x	Access Traffic Steering, Switching, and Splitting

### 5.7.x.1	Description

The 5G System supports the Access Traffic Steering, Switching, and Splitting (ATSSS) feature, which enables the UE and the network to establish a multi-access PDU session, using one 3GPP access and one non-3GPP access, and to distribute the traffic across these accesses using one or more steering modes, e.g., "Smallest Delay", "Load Balancing", "Redundant", etc. The multi-access rules created by the network for the multi-access PDU session are used to statically assign a traffic flow to a steering mode. For example:

-	A multi-access rule may assign the traffic of App-X to the "Smallest Delay" steering mode, meaning that this traffic should be sent to the access (3GPP or non-3GPP) with the smallest delay.

-	Another multi-access rule may assign the traffic of App-y to the "Load Balancing" steering mode, meaning that this traffic should be split across the 3GPP and the non-3GPP accesses.

However, this static assignment of a traffic flow to a single steering mode is a limitation for advanced 6G services where the performance requirements of an application can change dynamically during a session. Consider for example the following scenarios:

-	The traffic of an Extended Reality (XR) application can fluctuate dramatically on a small timescale. For example, a user turning their head quickly requires immediate transmission of new head-pose data (low data volume, extreme latency sensitivity) to trigger the rendering of a new viewpoint. The subsequent delivery of the rendered video frames requires high throughput but may have a slightly more relaxed latency budget. In this case, a static steering rule for the traffic of this application is incapable of optimizing for these rapidly changing, multi-modal traffic flows.

-	Similarly, an interactive video conference application may require low latency and low jitter during a conversational phase. However, during the same session, the user may begin sharing a high-resolution, full-motion video, which requires high throughput. Thus, mapping the traffic of this application to a single steering mode cannot provide optimal performance.

From the above, it becomes clear that there is a need for more flexible traffic steering strategies in 6G, which can adapt to the dynamic application requirements and optimize the efficiency of multi-access connectivity. A fixed steering mode cannot provide this level of adaptability, leading to bad user experience.

### 5.7.x.2	Potential New Requirements

[PR 5.7.x.2-1] The 6G system shall support multi-access connectivity between 3GPP and non-3GPP accesses based on the Access Traffic Steering, Switching, and Splitting (ATSSS) feature defined in 5G.

[PR 5.7.x.2-2] The 6G system shall be able to identify and distribute the different traffic flows of an application across multiple accesses, enabling each traffic flow to be handled according to its specific performance requirements.

NOTE 1:	For example, an XR application could have a head-tracking traffic flow that requires ultra-low latency and a rendered video traffic flow that requires high throughput. These traffic flows are dynamically created during the application runtime.

* * * End of Change * * *