3GPP TSG SA WG 1 Meeting #111 	S1-253277

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:	Huawei, HiSilicon, Rakuten

pCR Title:	Update of existing Use Case (clause 5.5.7) on privacy protection of data exposure

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Shuang ZHANG, zhang.shuang2 AT Huawei DOT com

1. Introduction

User privacy is an integral part of the 3GPP system, with requirements on the protection for the communication content (e.g. content of emails, web pages), as well as any identities and UE location, from being exposed to unauthorised parties.

The 3GPP system includes a wide variety of data, such as sensing data, AI-related data, network operation data, positioning data and so on. The data can be collected, generated shared with or exposed to 3rd parties for certain purposes (e.g., collaborative data utilization to enhance business opportunities). Data exposure should protect user privacy to comply with privacy regulatory requirements.

2. Reason for Change

In TR 22.870 v.031, use case in clause 5.5.7 on privacy protection of data exposure addresses that the 6G system shall provide user privacy protection, location privacy, identity protection for UEs accessing 6G network for 6G new services (e.g. communication, sensing, AI inferencing).

Historical evidence shows different uptake patterns of previous generations of 3GPP technologies/networks. Even for a particular generation (e.g. 2G, 3G, 4G, 5G), the beginning of the adoption and commercialization varied significantly across different regions, countries and among operators, which is presumably resulted from differences in market demand, operator investment capital and their market strategies, etc. 6G is expected to not only provide enhanced communication service but also many novel beyond-connectivity services. This is considered a factor further affecting the extent of the range in which 6G adoption is going to vary among different operators.

However, as an individual subscriber, he/she may want/need to enjoy 6G services not yet provided by the home network operator. To satisfy such needs constitutes a valuable opportunity for operators with varied 6G deployment to collaborate in the initial phase of 6G adoption, if they can enable their subscribers to consume 6G services provided by any of the partner operators.

3. Conclusions

Add the missing information that clarifies the aspects related to the use case to support the proposed new potential requirement.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.3.1.

* * * First Change * * * *

### 5.5.7	Use Case on privacy protection of data exposure

#### 5.5.7.1	Description

In the 5G era, proposals have been made to support user data communication tailored to various use cases such as enhanced Mobile Broadband (eMBB), ultra-reliable low-latency communications (URLLC), and massive Internet of Things (mIoT).

Looking ahead to the 6G era, it is anticipated that there will be an increasing demand for data collection beyond just user data communication. Use cases involving artificial intelligence (AI), and network digital twins (NDT) are expected to drive this demand. It is crucial to consider the efficiency of data collection processes, as the volume of data collected can significantly escalate based on the number of data points and the frequency of collection. Such an increase may lead to congestion in internal communications within the network operator. Moreover, collected data can be utilized for internal purposes, and the data shared with third parties should be processed to ensure the protection of user privacy to follow the operator’s policies and regulatory requirements, thereby facilitating collaborative data utilization to enhance business opportunities. In light of these considerations, there is a pressing need for frameworks that enhance the efficiency of data collection while addressing critical aspects of privacy through information disclosure practices, such as the abstraction and anonymization of data within mobile network systems.

As 6G system is expected to provide more novel services beyond connectivity as well as a plethora of new capabilities, the data collected and exposed is more versatile and could be different in nature, volume, formats, etc. When the data is provided to authorized 3rd parties via 6G core network, the privacy of the user/subscriber identities and UE’s identifiers should be protected.

Particularly aspects about data exposure:

Our experience in previous generations of 3GPP technologies shows their different uptake patterns. Even for a particular generation (e.g. 2G, 3G, 4G, 5G), the beginning of the network deployment and service commercialization varied significantly across different regions, countries and among operators. This is presumably resulted from differences in market demand, operator investment capital and their market strategies, etc. The perspective that 6G is provide multiple beyond-connectivity services and enhanced communication service is considered a factor further affecting the extent of the range in which 6G adoption is going to vary among different operators.

However, as an individual subscriber who has subscription with a particular operator, he/she may want/need to enjoy quite a few new 3GPP/6G services, and it may well be the case that some are not yet deployed by the home network operator. To bridge that gap, the 3GPP-provided capabilities to satisfy such user needs constitute a valuable opportunity for operators with varied 6G deployment to collaborate as partners (e.g. via service agreement) specially in the initial phase of 6G adoption, by enabling subscribers to consume 6G services provided by any of the partner operators. The 6G services can be pre-defined in the service agreement. According to service agreements between the home operator and the partner operators, a subscriber is aware of 6G services from the partner operators. The subscriber can consume these 6G services provided by different partner operators after the subscriber is authorized to access these 6G services e.g. by/via the home operator. For example, subscriber Alain of home network operator A can consume sensing from A’s partner B’s PLMN_B, and Alain can consume AI service from A’s partner C’s PLMN_C, potentially at different point in time/location.

Then in the context of 6G system data collection and consumption where operators monetize 6G System Data, such data related to Alain consuming these services from various partner 3GPP networks will ensue. When such 6G System Data is exposed to a 3rd party, it is in the subscriber/user Alain’s interest that 6G systems provide mechanisms to ensure his privacy is protected as much as possible.

NOTE: 	For example, Alain consumed sensing service at location L1 and around time T1, the sensing service-related data in PLMN_B may be exposed to data consumer Diana. Also, Alain consumed AI inference service at location L2 around time T1, the AI service-related data in PLMN_C may also be exposed to the same data consumer.

In summary, the goal is to preserve user’s confidentiality protecting against link-ability and trackability attack. In case the privacy sensitive information about UE’s identifiers may be leaked to undesired parties, the privacy of the user/subscriber can be protected. This may suggest the need for the subscriber’s UE to access partner operators’ various PLMNs with different temporary identifiers, to avoid partner operators or 3rd party (e.g. data consumer Diana) to fathom out when and from which PLMNs the very user/subscriber Alain has accessed a given service.

* * * Second Change * * * *

#### 5.5.7.2	Existing features partly or fully covering the use cases functionality

There are requirements for network capability exposure that are partially related, as indicated in clause 6.10.2 on TS 22.261 [14]. However, the requirement is for a security logging information of UEs.

Based on operator policy, the 5G network shall expose a suitable API to provide the security logging information of UEs, for example, the active 3GPP security mechanisms (e.g. data privacy, authentication, integrity protection) to an authorized third-party.

The 5G system shall be able to:

-	provide a third-party with secure access to APIs (e.g. triggered by an application that is visible to the 5G system), by authenticating and authorizing both the third-party and the UE using the third-party's service.

Subject to regulatory requirements and based on operator policy, the 5G system shall provide a mechanism to support confidentiality to prevent exposure of data exchanged between the 5G network and a third party service provider. Subject to regulatory requirements and based on operator policy, the 5G system shall provide a mechanism to support data integrity verification service to assure the integrity of the data exchanged between the 5G network and a third-party service provider.

The 5G system shall provide suitable means to allow use of a trusted and authorized third-party provided integrity protection mechanism for data exchanged between an authorized UE served by a private slice and a core network entity in that private slice.

The 5G system shall provide suitable means to allow use of a trusted and authorized third-party provided integrity protection mechanism for data exchanged between an authorized UE served by a non-public network and a core network entity in that non-public network.

In TS 22.261 clause 8.5 includes the requirements of identity management. The 5G system shall provide a mechanism for an operator to allow access from a UE using a temporary identifier (e.g. 5G-GUTI) that hides its subscriber identity. The 5G system shall provide a mechanism for an operator to allow access from a UE connected in an indirect network connection using a temporary identifier that hides its subscriber identity. The HPLMN shall be able to associate a temporary identifier to a UE's subscriber identity. The existing requirements does not cover when a UE accesses one or more multiple beyond-connectivity services provided/shared by other PLMN(s), how to prevent/protect against link-ability and trackability when different data such as 6G System Data related to a certain user is exposed to a 3rd party (e.g. using different/multiple identifiers tht hide subscriber identity).

TS 22.261 clause 6.21 includes the requirements for Indirect Network Sharing. Indirect Network Sharing is a type of NG-RAN Sharing in which the communication between the Shared NG-RAN and the Participating Operator’s core network is routed through the Hosting NG-RAN Operator’s core network. The existing Indirect Network Sharing doesn’t cover that a UE only with a subscription to HPLMN can access one or multiple beyond connectivity service shared by other HPLMN’s (non-roaming) partner PLMN(s).

TS 22.261 clause 6.25.2, subject to an agreement between the operators and service providers, operator policies and the regional or national regulatory requirements, the 5G system shall enable a UE, with multiple subscriptions, to simultaneously access multiple non-public networks and corresponding services, via those NPNs or via a different network (PLMN or NPN). In this use case, the subscriber only subscribes with his/her home operator, and does not have multiple subscriptions with multiple different PLMNs for accessing their new services.

TS 22.261 clause 6.1 includes the requirements for Network slicing. According to the TS 22.26, a network slice can provide the functionality of a complete network, including radio access network functions, core network functions (e.g. potentially from different vendors) and IMS functions. In addition, the following requirement requires UE to access more than one VPLMN simultaneously if the network slices are not available in a single VPLMN:

- For UEs that have the ability to obtain service from more than one VPLMN simultaneously, the following requirements apply:

-	When a roaming UE with a single PLMN subscription requires simultaneous access to multiple network slices and the network slices are not available in a single VPLMN, the 5G system shall enable the UE to:

-	be registered to more than one VPLMNs simultaneously; and

-	use network slices from more than one VPLMN simultaneously

In TS 33.501, clause 6.3.1.3, the transmission of the subscriber identities (SUPI) and security context is permitted between 5G core network entities of different serving network domains (e.g. during roaming, between HPLMN CN and VPLMN). In our scenario, subscriber identity (SUPI) is not permitted to expose to any serving network domains (e.g. incl. non-roaming partner PLMNs).

* * * Third Change * * * *

#### 5.5.7.3	Potential New Requirements needed to support the use case

[PR 5.5.7.3-1]    Subject to operator policy and regulatory requirements, the 6G system shall support privacy protection for any information exposure to a 3rd party.

[PR 5.5.7.3-2] Subject to national or regional regulatory requirement, the 6G system shall provide user privacy protection, location privacy, identity protection for UEs accessing 6G network for services (e.g. communication, sensing, AI inferencing), and for the corresponding information exposure to an authorized 3rd party.

[PR 5.5.7.3-3] The 6G system shall be able to protect UE’s subscriber identities from attacks.

[PR-5.3.6.3-4] Subject to agreement among operator, national and regional regulation, the 6G system shall provide a mechanism to protect privacy of the user and UE’s identities (incl. avoiding link-ability and trackability), when the UE accesses home provider’s various partners’ 6G networks for consuming their 6G services (e.g. possibly with different UE identifiers), and/or during UE/user-related data exposure (e.g. 6G System Data) to an authorized 3rd party.

* * * End of Change * * * *