3GPP TSG SA WG 1 Meeting #111 	S1-253282

25–29 August 2025, Göteborg, Sweden

pCR Title: Pseudo CR on Adaptive Group Management and Task Offloading in V2X Platooning for 6G system

Draft Spec:	3GPP TR 22.870v0.3.1

Agenda item:	8.1.3

Document for:	Approval

Source:	TCS

Contact: Garima Mishra (garima.mishra2@tcs.com), Hemant Kumar Rath (hemant.rath@tcs.com) and Muskan Rana (muskan.rana@tcs.com)

Abstract: This use case proposes a dynamic group management and QoS prediction for group mobility scenarios in vehicular platoons. By leveraging MEC-based analytics, the system predicts QoS in non-stationary radio environments to support proactive task offloading and collision risk mitigation. Stationarity checks guide optimal platoon formation, maintaining safe inter-vehicle distances and enhancing traffic efficiency while reducing service interruptions.

* * * First Change * * * *

## 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	QIAN LI et all: "6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies" in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022. (6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies | IEEE Journals & Magazine | IEEE Xplore).

[3]	GSMA: "Going green: benchmarking the energy efficiency of mobile networks (Second edition)", Feb 2023. (Going green: benchmarking the energy efficiency of mobile networks (second edition)).

[4] 	3GPP TR 32.972: "Study on system and functional aspects of energy efficiency in 5G networks".

[5]	3GPP TR 23.700-66: "Study on Energy Efficiency and Energy Saving".

[6] 	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[7]	5G Automotive Association (5GAA): "5GAA MRP input to 3GPP Stage 1 workshop on IMT 2030 Use Cases".

[8]	5G Automotive Association (5GAA): Technical report "Evolution of vehicular communication systems beyond 5G", August 2023. (Evolution of Vehicular Communication Systems Beyond 5G - 5GAA).

[9] 	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication".

[10]	SAE J3016 202104: "Levels of Driving Automation". (J3016_202104: Taxonomy and Definitions for Terms Related to Driving Automation Systems for On-Road Motor Vehicles - Recommended Practice).

[11]	G. Liu, L. Ma, Y. Xue, et al, "SensCAP: A Systematic Sensing Capability Performance Metric for 6G ISAC," IEEE Internet of Things Journal, vol. 11, no. 18, pp. 29438–29454, 2024. (SensCAP: A Systematic Sensing Capability Performance Metric for 6G ISAC | IEEE Journals & Magazine | IEEE Xplore).

[12]	Igal Bilik, Oren Longman, Shahar Villeval, and Joseph Tabrikian, "The Rise of Radar for Autonomous Vehicles", IEEE signal processing magazine, Volume 36, Issue 5, September 2019. (The Rise of Radar for Autonomous Vehicles.pdf).

[13] 	https://knowledge-base.inspire.ec.europa.eu/publications/technical-guidance-implementation-inspire-dataset-and-service-metadata-based-isots-191392007_en.

[14]	3GPP TS 22.261: "Service requirements for the 5G system".

[15] 	Wang et.al, "Effect of Frame Rate on User Experience, Performance, and Simulator Sickness in Virtual Reality", IEEE Transaction on Visualization and Computer Graphics. 2023 Feb 22:PP. doi: 10.1109/TVCG.2023.3247057. (Effect of Frame Rate on User Experience, Performance, and Simulator Sickness in Virtual Reality | IEEE Journals & Magazine | IEEE Xplore).

[16] 	https://en.wikipedia.org/wiki/Adreno.

[17] 	https://www.labtekno.com/snapdragon-8-gen-2-vs-apple-a16-bionic/.

[18] 	Algorithms and tools for GPU-accelerated Computer Generated Holography | Ewireless Research Group (https://www.ewireless.eng.ed.ac.uk/projects).

[19] 	https://phlearn.com/tutorial/how-many-exposures-hdr/.

[20]	https://photographylife.com/compressed-vs-uncompressed-vs-lossless-compressed-raw.

[21]	https://www.dotphoton.com/products/jetraw-core.

[22] 	https://www.lambdatest.com/learning-hub/web-performance-testing.

[23] 	Marcos Aviles, et al., Static 3D triangle mesh compression overview, Grupo de Tratamiento de Imágenes, Universidad Politécnica de Madrid, Spain, 15th IEEE International Conference on Image Processing. (Static 3D triangle mesh compression overview | IEEE Conference Publication | IEEE Xplore).

[24] 	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[25]	European Union Agency for the Space Programme (EUSPA): "Report on Aviation and Drones – User Needs and Requirements", 2023. (Report_on_User_Needs_and_Requirements_Aviation.pdf)

[26]	3GPP TR 36.763: "Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)".

[27]	Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond". (https://www.itu.int/dms_pubrec/itu-r/rec/m/R-REC-M.2160-0-202311-I!!PDF-E.pdf).

[28]	3GPP TS 22.156: "Mobile Metaverse Services".

[29] 	United Nations General Assembly. (1987). Report of the world commission on environment and development: Our common future. https://digitallibrary.un.org/record/139811.

[30] 	3GPP TS 23.502: "Procedures for the 5G System".

[31]	Hexa-X-II, "Deliverable D1.2: 6G Use Cases and Requirements", December 2023. (Hexa-X-II D1.2).

[32]	https://www.easa.europa.eu/en/what-is-uam.

[33]	https://www.autoflight.com/en/air/.

[34]	"Low altitude economy development white paper (2.0) All digital solutions", International digital economy academy (IDEA), 2023.

[35]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP; Stage 1".

[36]	https://www.unmannedairspace.info/latest-news-and-information/shenzhen-reaches-record-number-of-drone-operations-with-600000-flights-in-2023/.

[37]	5G Automotive Association (5GAA): "C-V2X Use Cases and Service Level Requirements Volume II", 2023. (C-V2X Use Cases and Service Level Requirements Volume II - 5GAA).

[38]	https://www.flyeye.io/drone-acronym-daa/.

[39]	Report ITU-R M.2516-0, "Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond ". (https://www.itu.int/dms_pub/itu-r/opb/rep/R-REP-M.2516-2022-PDF-E.pdf).

[40]				3GPP TR 33.841: "Study on the support of 256-bit algorithms for 5G".

[41]	3GPP TS 35.246: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 1: algorithm specification".

[42]	3GPP TS 35.247: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 2: implementation test data".

[43]	3GPP TS 35.248: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 3: design conformance test data".

[44]	3GPP TR 22.883: "Feasibility Study on Energy Efficiency as service criteria Phase 2".

[45]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[46]	3GPP TS 38.211: "NR; Physical channels and modulation".

[47]	3GPP TS 36.305: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[48]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[49]	3GPP TS 22.856: "Study on Localized Mobile Metaverse Services".

[50]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[51]	3GPP TS 26.565: "Split Rendering Media Service Enabler".

[52]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[53]	3GPP TS 22.179: “Mission Critical Push to Talk (MCPTT); Stage 1”.

[54]	3GPP TS 22.280: “Mission Critical Services Common Requirements (MCCoRe); Stage 1”..

[55]	3GPP TS 22.281: “Mission Critical (MC) video”.

[56]	3GPP TS 22.282: “Mission Critical (MC) data”.

[57]	3GPP TS 22.262: “Message service within the 5G System (5GS); Stage 1”.

[58]	3GPP TS 22.101: “Service aspects; Service principles”.

[59]	3GPP TS 22.173: “IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1”.

[60]	3GPP TS 22.011: “Service accessibility”.

[61]	3GPP TS 22.071: “Location Services (LCS); Service description; Stage 1”.

[62]	3GPP TS 22.268: “Public Warning System (PWS) requirements”.

[63]	3GPP TS 22.153: “Multimedia priority service”.

[64]	3GPP TS 22.104: “Service requirements for cyber-physical control applications in vertical domains”.

[65]	3GPP TS 22.185: “Service requirements for V2X services”.

[66]	3GPP TS 22.186: “Service requirements for enhanced V2X scenarios”.

[67]	3GPP TS 22.263: “Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA)”.

[68]	3GPP TS 22.115: “Service aspects; Charging and billing”.

[69]	3GPP TS 22.289: “Mobile communication system for railways”.

[70]	3GPP TS 22.468: “Group Communication System Enablers (GCSE)”.

[71]	3GPP TS 22.368: “Service requirements for Machine-Type Communications (MTC); Stage 1”.

[72]	3GPP TR 22.949: “Study on a generalized privacy capability”.

[73]	3GPP TR 33.849: “Study on subscriber privacy impact in 3GPP”.

[74]	3GPP TS 22.278: “Service requirements for the Evolved Packet System (EPS)”.

[75]	ETSI GR SAI 004 (V1.1.1): "Securing Artificial Intelligence (SAI); Problem Statement".

[76]	ETSI GR SAI 005 (V1.1.1): "Securing Artificial Intelligence (SAI); Mitigation Strategy Report".

[77]	3GPP TR 28.915: "Study on management aspects of Network Digital Twin".

[78]	3GPP TS 23.527: “5G System; Restoration procedures”.

[79]	3GPP TR 29.866: “Study on IMS Disaster Prevention and Restoration Enhancement”.

[80]	Enabling Mobile AI Agent in 6G Era: Architecture and Key Technologies; https://dl.acm.org/doi/abs/10.1109/MNET.2024.3422309.

[81]	Wang L, Ma C, Feng X, et al. A survey on large language model based autonomous agents[J]. Frontiers of Computer Science, 2024, 18(6): 186345. ([2308.11432] A Survey on Large Language Model based Autonomous Agents).

[82]	Hong S, Zheng X, Chen J, et al. Metagpt: Meta programming for multi-agent collaborative framework[J]. arXiv preprint arXiv:2308.00352, 2023. ([2308.00352] MetaGPT: Meta Programming for A Multi-Agent Collaborative Framework).

[xx]	ISO/IEC 22989:2022, "Information technology — Artificial intelligence — Artificial intelligence concepts and terminology".

[83] 	GSMA Foundry, “Case Study: Delivering Real-Time Translation”,2023, 5G New Calling - GSMA Foundry .

[84] 	GSMA Foundry, “Case Study: Remote damage assessment ”, 2023, Remote Damage Assessment - GSMA Foundry.

[85]	GSMA Foundry, “Case Study: Visualised Video-Calling”,2023, Visualised Voice-Calling - GSMA Foundry.

[86] 	GSMA Foundry, “Case Study: Adding New Value for Voice Call”,2024, Reference links for https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/adding-new-value-to-voice-calls/.

[87]	United Nations Sustainable Development Goals (https://sdgs.un.org/goals).

[88]	United Nations Global Digital Compact (https://www.un.org/global-digital-compact/sites/default/files/2024-09/Global%20Digital%20Compact%20-%20English_0.pdf).

[89]	Zhang Z, Wang S, Hong Y, et al. Distributed dynamic map fusion via federated learning for intelligent networked vehicles[J]. arXiv preprint arXiv:2103.03786, 2021 ([2103.03786] Distributed Dynamic Map Fusion via Federated Learning for Intelligent Networked Vehicles).

[90]	Automotive Edge Computing Consortium (AECC): Operational Behavior of a High Definition Map Application – White Paper Version 1.0.0, May 2020, (Description of the Operational Behavior of an HD Map Application).

[91]	 https://spectrum.ieee.org/boats-should-be-sleek-but-only-up-to-a-point.

[92]	https://www.marketsandmarkets.com/Market-Reports/evtol-aircraft-market-28054110.html (eVTOL Aircraft Market Size, Share, Industry Report, Revenue Trends and Growth Drivers).

[93]	https://www.fortunebusinessinsights.com/autonomous-vehicle-market-109045 (Autonomous Vehicle Market Size, Share, Trends | Report [2030]).

[94] 	5G Alliance for Connected Industries and Automation (5G-ACIA): White Paper, “Use Cases and Requirements for Integrated Sensing and Communication (ISAC) in Connected Industries and Automation” (Use Cases and Requirements for Integrated Sensing and Communication (ISAC) in Connected Industries and Automation - 5G-ACIA).

[95]	ETSI GR ISC-001 V1.0.0 group report, “Integrated Sensing And Communications (ISAC); Use Cases and Deployment Scenarios”

[96]	3GPP TR 37.885 Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR.

[97]	3GPP TR 22.865: “Study on satellite access - Phase 3”.

[98]                   	3GPP TR 22.836: "Study on Asset Tracking Use Cases".

[99]	CHINA AIRSPACE CLASSIFICATION AND PILOT UPDATE https://www.icao.int/APAC/Meetings/2024%20ATMSG12/IP09%20China%20Airspace%20Classification%20and%20Pilot%20Update.pdf#search=AIRSPACE%20CLASSIFICATION.

[100]	3GPP TS 38.171: "NR; Requirements for support of Assisted Global Navigation Satellite System (A-GNSS)".

[101] 	https://cloud.tencent.com/developer/article/1654264.

[102] 	ITU-T Recommendation G.114 (05/2003): “One-way transmission time” (G.114 : One-way transmission time).

[103]	S4aV200634: "Traces and Configurations for VR2, CG and AR2".

[104]	ITU-T FG NET-2030, "Representative use cases and key network requirements for Network 2030", 2020. (Network 2030 - Representative use cases and key network requirements for Network 2030 - ITU).

[105]	Ian F. Akyildiz, Hongzhi Guo, "Holographic-type communication: A new challenge for the next decade", ITU Journal on Future and Evolving Technologies, Volume 3 (2022), Issue 2, Pages 421-442. (Holographic-type communication: A new challenge for the next decade).

[106]	X. Xu, Y. Pan, P. P. M. Y. Lwin and X. Liang, "3D holographic display and its data transmission requirement," 2011 International Conference on Information Photonics and Optical Communications, Jurong West, Singapore, 2011, pp. 1-4, doi: 10.1109/IPOC.2011.6122872. (3D holographic display and its data transmission requirement | IEEE Conference Publication | IEEE Xplore).

[107]	P. Ekdahl, T. Johansson, A. Maximov, and J. Yang, "A new SNOW stream cipher called SNOW-V", IACR Transactions on Symmetric Cryptology, vol. 2019, no. 3, pp. 1-42, 2019. doi: 10.13154/tosc.v2019.i3.1-42. (A new SNOW stream cipher called SNOW-V | IACR Transactions on Symmetric Cryptology).

[108]	P. Ekdahl, T. Johansson, A. Maximov, and J. Yang, "SNOW-Vi: an extreme performance variant of SNOW-V for lower grade CPUs ", WiSec '21, pp. 261 – 272, 2021. https://doi.org/10.1145/3448300.3467829

[109]	IEC 62056-6-1:2023 Object Identification System (OBIS).

[110]	IEC 62056-6-2:2023 COSEM interface classes.

[111]	IEC 62056-5-3:2023 DLMS/COSEM application layer.

[112]	Hexa-X-II, “Deliverable D1.4, 6G Value, Requirements, and Ecosystem”, April 2025. (D1.4-final.pdf).

[113] 	ITU-T Recommendation Y.3090 (02/22): "Digital twin network - Requirements and architecture" (https://www.itu.int/rec/T-REC-Y.3090-202202-I).

[114]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[115] 	Urban Air Mobility, https://www.airbus.com/en/innovation/low-carbon-aviation/urban-air-mobility, AIRBUS, accessed in May 2024.

[116] 	Urban Air Mobility (UAM) Concept of Operations, https://www.faa.gov/air-taxis/uam_blueprint, Federal Aviation Administration of U.S. Department of Transportation, Aug. 2023.

[117] 	Urban Air Mobility, https://rotorcraft.arc.nasa.gov/Research/Programs/UrbanAirMobility.html , U.S. NASA, 2024.

[118] 	NASA Urban Air Mobility (UAM) Reference Vehicles, https://sacd.larc.nasa.gov/uam-refs/, U.S. NASA, Jan. 2024

[119] 	Future Mobility, https://www.hyundai.com/worldwide/en/newsroom/detail/0000000093.html, Hyundai Motor Company, July 2022.

[120]	LG Uplus, Near-Future Vertical Applications in Korea: K-UAM, https://nextgalliance.org/wp-content/uploads/2024/05/Near-future-Vertical-Applications-in-Korea-K-UAM_6GF_NGA_Joint_Workshop_JKim_final.pdf, Next G Alliance / Korea 6G Forum Joint Workshop

[121]	A. Baltaci, et. al., “A Survey of Wireless Networks for Future Aerial Communications (FACOM),” IEEE Communications Surveys & Tutorial, Vo. 23, No. 4, Q4 2021. https://ieeexplore.ieee.org/document/9508366.

[122]	3GPP TR 22.887: "Feasibility Study on satellite access – Phase 4".

[123]	Next G Alliance Report “6G Applications and Use Cases”: Use Case 4.6 – High-Speed Wireless Connection in Aerial Vehicle for Entertainment Service. (https://nextgalliance.org/6g-library/).

[124]	Topal, O. A., Schupke, D., Björnson, E., & Cavdar, C., “Optimal Joint Access Point Placement and Resource Allocation for Indoor mmWave Communications”, ICC 2023, Rome, Italy. (Optimal Joint Access Point Placement and Resource Allocation for Indoor mmWave Communications | IEEE Conference Publication | IEEE Xplore”.

[125]	Hofmann, S.; Duhovnikov, S.; Schupke, D., “Massive Data Transfer from and to Aircraft on Ground: Feasibility and Challenges,” IEEE Aerospace and Electronic Systems Magazine, vol. 36, iss. 5, May 2021. (Massive Data Transfer From and to Aircraft on Ground: Feasibility and Challenges | IEEE Journals & Magazine | IEEE Xplore).

[126] 	NASA. UTM: Air traffic management for low-altitude drones[EB/OL] (2015-10) (https://www.nasa.gov/sites/default/files/atoms/files/utm-factsheet-11-05-15.pdf)

[127] 	SESAR. U-space blueprint[EB/OL]. (2017-06-09). (https://www.sesarju.eu/sites/default/files/documents/reports/U-Space%20Blueprint%20brochure%20final.pdf)

[128] 	AOPA China. Regulations for the operation of light and small UAVs [EB/OL] (2018-04-19). (http://www.aopa.org.cn/usr/1/upload/file/20180419/15241267234030958.pdf)

[129] 	HIROYUKI U. UTM project in Japan[EB/OL]. (2017-06-26)[2025-01-20]. (https://gutma.org/montreal-2017/wp-ontent/uploads/sites/2/2017/07/UTM-Project-in-Japan_METI.pdf).

[130] 	G. Wagner and H. Choset, “A complete multirobot path planning algorithm with performance bounds,” Proc. In IEEE/RSJ International Conference on Intelligent Robots and Systems (IROS), San Francisco, CA, USA, Sept. 2011. M*: A complete multirobot path planning algorithm with performance bounds | IEEE Conference Publication | IEEE Xplore.

[131]	B. Hu, S. Xu, and Z. Cao, “Multi-Robot Path Planning for Each Robot with Several Jobs in a Single Trip,” IFAC-PapersOnline, Volume 53, Issue 5, 2020, Pages 279-284. Multi-Robot Path Planning for Each Robot with Several Jobs in a Single Trip - ScienceDirect.

[132]	S. Lin, A. Liu, J. Wang and X. Kong, “A Review of Path-Planning Approaches for Multiple Mobile Robots,” Section of Automation and Control Systems, MDPI Machines, 10(9), 773, Sept. 2022. A Review of Path-Planning Approaches for Multiple Mobile Robots.

[133]	A New Approach to Data Sharing - Open data sharing and collaboration for data, analytics and AI, DataBricks eBook 3rd Ed. A New Approach to Data Sharing | Databricks.

[134]	K.-D. Lee, “A Smart Network of Service Robots: Technical Challenges and Design Considerations,” IEEE Communications Magazine, pp. 28-34, vol. 59, Iss. 8, Aug. 2021. A Smart Network of Service Robots: Technical Challenges and Design Considerations | IEEE Journals & Magazine | IEEE Xplore.

[135]	Next G Alliance, “Network-Enabled Robotic and Autonomous Systems”, May 2023. (https://nextgalliance.org/6g-library/).

[136]	3GPP TR 22.916: "Study on Network of Service Robots with Ambient Intelligence; Stage 1".

[137]	3GPP TS 23.548: “5G System Enhancements for Edge Computing; Stage 2”.

[138]	3GPP TS 22.228: “Service requirements for the Internet Protocol (IP) multimedia core network subsystem (IMS); Stage 1”.

[139]	3GPP TS 28.105: “Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management”.

[140]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”.

[141]	3GPP TS 23.503: “	Policy and charging control framework for the 5G System (5GS); Stage 2”.

[142]	3GPP TS 23.228: “IP Multimedia Subsystem (IMS); Stage 2”.

[143] 	Test Report from Utility Broadband Alliance “2024 UBBA Plugfest Testing Report” (https://www.ubba.com/ubba-resources/2024-ubba-plugfest-testing-report/).

[144]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[145]	GSMA NG.129: "IMS Data Channel White Paper".

[146]	Bajpai, Divya Jyoti, and Manjesh Kumar Hanawal. "Distributed Inference on Mobile Edge and Cloud: An Early Exit based Clustering Approach." arXiv preprint arXiv:2410.05338 (to appear in IEEE ICC 2025) ([2410.05338] Distributed Inference on Mobile Edge and Cloud: An Early Exit based Clustering Approach).

[147]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks".

[148]	Levels of AI Agents: from Rules to Large Language Models, Yu Huang, https://arxiv.org/abs/2405.06643, Oct 2024.

[149]	Morris, Meredith Ringel, et al. "Position: Levels of AGI for operationalizing progress on the path to AGI." Forty-first International Conference on Machine Learning. 2024. ([2311.02462] Levels of AGI for Operationalizing Progress on the Path to AGI).

[150]	3GPP TS 24.604: “Communication Diversion (CDIV) using IP Multimedia (IM) Core Network (CN) subsystem”.

[151] 	B5GPC’s Views on IMT2030 use cases in SA1 Workshop On IMT2030 Use Cases.

[152] 	GSMA Foundry, “5G New Calling”,  5G New Calling - GSMA Foundry.

[153] 	GSMA Foundry, “Delivering Real-Time Translation”, https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/delivering-real-time-translation/.

[154] 	https://www.naka.syntphony.com/generative-ai-immersive-experiences/.

155] 	https://arxiv.org/html/2404.01713v2.

[156] 	https://www.emarketer.com/content/older-adults-prefer-phone-calls-digital-customer-service.

[157] 	Tan G, Lee Y H, Zhan T, et al. Foveated imaging for near-eye displays[J]. Optics express, 2018, 26(19): 25076-25085.

[158] 	White Paper for 5G Cloud VR Service Experience Standards, 2019.06

[159]	eXtended Reality for NR, https://www.3gpp.org/technologies/xr-nr.

[160] 	Chroma subsampling https://en.wikipedia.org/wiki/Chroma_subsampling#4:1:1.

[161]	Requirements of 6G+ "Future TV" video Application Scenarios | Future Mobile Forum (https://www.g6gconference.com/upload/file/20240420171358871090.rar)

[162]	Cheng Y, Zhang Z, Li H, et al. GRACE: Loss-Resilient Real-Time Video through Neural Codecs[C]//21st USENIX Symposium on Networked System Design and Implementation (NSDI 24). 2024: 509-531. ((GRACE: Loss-Resilient Real-Time Video through Neural Codecs | USENIX).

[163]	Immersive virtual reality (immersive VR). https://www.techtarget.com/whatis/definition/immersive-virtual-reality-immersive-VR.

[164]	3GPP TR 22.847: “Study on supporting tactile and multi-modality communication services; Stage 1”.

[165]	3GPP TR 26.854: “Study on Haptics in 5G Media Services”.

[166]	3GPP TR 26.998: “Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices”.

[167]	3GPP TR 26.806: “Study on Tethering AR Glasses – Architectures, QoS and Media Aspects”.

[168]	C. Dong, H. Liang, X. Xu, S. Han, B. Wang and P. Zhang, "Semantic Communication System Based on Semantic Slice Models Propagation," in IEEE Journal on Selected Areas in Communications, vol. 41, no. 1, pp. 202-213, Jan. 2023, doi: 10.1109/JSAC.2022.3221948. (Semantic Communication System Based on Semantic Slice Models Propagation | IEEE Journals & Magazine | IEEE Xplore).

[169]	Xuemin S., Jie Gao, “Toward Immersive Communications in 6G”, Frontiers of Computer Science, Volume 4 (2022), 11 January 2023 ([2303.05283v1] Toward Immersive Communications in 6G).

[170]	Kougioumtzidis, G., Poulkov, V.,” QoE Assessment Aspects for Virtual Reality and Holographic Telepresence Applications”. In: Future Access Enablers for Ubiquitous and Intelligent Infrastructures (pp.171-180).

[171]	A. Clemm, M. T. Vega, “Toward truly immersive holographic-type communication: Challenges and solutions,” IEEE Communications Magazine, vol. 58, no. 1, pp93-99, 2020. (Toward Truly Immersive Holographic-Type Communication: Challenges and Solutions | IEEE Journals & Magazine | IEEE Xplore).

[172]	dupe of [106] Xu, Xuewu & Pan, Yuechao & Lwin, Phyu & Liang, Xinan. (2011), “3D holographic display and its data transmission requirement”, 10.1109/IPOC.2011.6122872. (3D holographic display and its data transmission requirement | IEEE Conference Publication | IEEE Xplore).

[173]	H. Ban, S. Choi, J. Y. Cha, Y. Kim and H. Y. Kim, "NHVC: Neural Holographic Video Compression with Scalable Architecture," 2024 IEEE Conference Virtual Reality and 3D User Interfaces (VR), Orlando, FL, USA, 2024, pp. 969-978, doi: 10.1109/VR58804.2024.00116

[174] 	Duanmu Z, Liu W, Li Z, Chen D, Wang Z, Wang Y, Gao W., “Assessing the quality-of-experience of adaptive bitrate video streaming”. arXiv preprint arXiv:2008.08804. 2020 Aug 20. ([2008.08804] Assessing the Quality-of-Experience of Adaptive Bitrate Video Streaming).

[175] 	Amiri A, Paymard P, Andres-Maldonado P, Paris S, Pedersen KI, Kolding T., “Application Awareness for Extended Reality Services: 5G-Advanced and Beyond”, IEEE Communications Magazine. 2024 Aug 12;62(8):38-44. (Application Awareness for Extended Reality Services: 5G-Advanced and Beyond | IEEE Communications Magazine).

[176]	Doherty Threshold | Laws of UX. https://lawsofux.com/doherty-threshold/

[177]	Hello GPT-4o | OpenAI. https://openai.com/index/hello-gpt-4o/

[178]	I. Burgetová, O. Ryšavý and P. Matoušek, "Towards Identification of Network Applications in Encrypted Traffic," 2024 8th Cyber Security in Networking Conference (CSNet), Paris, France, 2024, pp. 213-221, doi: 10.1109/CSNet64211.2024.10851738.(Towards Identification of Network Applications in Encrypted Traffic | IEEE Conference Publication | IEEE Xplore).

[179]	3GPP TS 38.212 V18.6.0: " Multiplexing and channel coding".

[180]	https://www.cnet.com/tech/computing/ai-brains-in-a-humanoid-robot-meet-figure-02/.

[181]	https://humanbenchmark.com/tests/reactiontime.

[182]	https://std.samr.gov.cn/hb/search/stdHBDetailed?id=2001448F0D0D32F5E06397BE0A0AAD9C.

[183]	https://www.nextrongroup.com/news/web-news-industry/humanoid-robots-industry-news.

[184]	Dupe to [162]    Cheng Y, Zhang Z, Li H, et al. GRACE: Loss-Resilient Real-Time Video through Neural Codecs[C]//21st USENIX Symposium on Networked System Design and Implementation (NSDI 24). 2024: 509-531. (GRACE: Loss-Resilient Real-Time Video through Neural Codecs | USENIX).

[185]	Xi, Zhiheng, Wenxiang Chen, Xin Guo, Wei He, Yiwen Ding, Boyang Hong, Ming Zhang et al. "The rise and potential of large language model based agents: A survey," 2023,  [2309.07864] The Rise and Potential of Large Language Model Based Agents: A Survey.

[186]	Hoffmann, Jordan, Sebastian Borgeaud, Arthur Mensch, Elena Buchatskaya, Trevor Cai, Eliza Rutherford, Diego de Las Casas et al. "Training compute-optimal large language models.", 2022,  ([2203.15556] Training Compute-Optimal Large Language Models).

[187]	A. Singh, "A Survey of AI Text-to-Image and AI Text-to-Video Generators," 2023 4th International Conference on Artificial Intelligence, Robotics and Control (AIRC), Cairo, Egypt, 2023, pp. 32-36, doi: 10.1109/AIRC57904.2023.10303174. ([2311.06329] A Survey of AI Text-to-Image and AI Text-to-Video Generators).

[188]	X. Shi, Q. Li, D. Wang and L. Lu, "Mobile Computing Force Network (MCFN): Computing and Network Convergence Supporting Integrated Communication Service", 2022 International Conference on Service Science (ICSS), Zhuhai, China, 2022, pp. 131-136, doi: 10.1109/ICSS55994.2022.00028. (Mobile Computing Force Network (MCFN): Computing and Network Convergence Supporting Integrated Communication Service | IEEE Conference Publication | IEEE Xplore).

[189]	"5G-A/6G New Computing Plane for Coordination of Networking and Computing", ( https://cmri.chinamobile.com/thinktank/origin/file/viewer?id=3801004&moduleType=7&subModuleType=71&pos=).

[190]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

[191]		P. D. L. Williams et al., “Experimental Study of the Radar Cross Section of Maritime Targets”, Electronic Circuits and Systems, Volume 2, No 4, July 1978, Experimental study of the radar cross-section of maritime targets | IEE Journal on Electronic Circuits and Systems.

[192]		IMO A.1046(27): "Revised report on the study of a worldwide radionavigation system". (1046(27) Worldwide radionavigation system - Netherlands Regulatory Framework (NeRF) – Maritime).

[193]		ES-TRIN:"European Standard laying down Technical Requirements for Inland Navigation vessels". (ES-TRIN 2025/1 - European Standard laying down Technical Requirements for Inland Navigation vessels)

[194] 		Meijun Zhou et al., “Risk analysis of road networks under the influence of landslides by considering landslide susceptibility and road vulnerability: A case study”, Natural Hazards Research,Volume 4, Issue 3,2024, Pages 387-400,ISSN 2666-5921. https://doi.org/10.1016/j.nhres.2023.09.013.

[195]		Behnam Khorrami et al. (2021), “Land deformation and sinkhole occurrence in response to the fluctuations of groundwater storage: an integrated assessment of GRACE gravity measurements, ICESat/ICESat-2 altimetry data, and hydrologic models”, GIScience & Remote Sensing, 58:8, 1518-1542, https://doi.org/10.1080/15481603.2021.2000349

[196]		M. Parise et al. (2018), “Sinkhole clusters after heavy rainstorms. Journal of Cave and Karst Studies”, v. 80, no. 1, p. 28-38. DOI: 10.4311/2017ES0105 ((PDF) Sinkhole clusters after heavy rainstorms).

[197]		Euronews (2024), “Surge in 'Turkey's granary' sinkholes imperils agriculture”, 6 September, https://www.euronews.com/2024/09/06/surge-in-sinkholes-in-turkeys-granary-endangers-agriculture

[198]	Future of Freight: The Use of Drones in Logistics,
https://dhl-freight-connections.com/en/solutions/future-of-freight-the-use-of-drones-in-logistics/.

[199]	Yasin, Jawad N., et al. "Unmanned aerial vehicles (uavs): Collision avoidance systems and approaches." IEEE access 8 (2020): 105139-105155. (Unmanned Aerial Vehicles (UAVs): Collision Avoidance Systems and Approaches | IEEE Journals & Magazine | IEEE Xplore).

[200]	Chen, Wen, et al. "Enhancing GNSS Positioning in Urban Canyon Areas via a Modified Design Matrix Approach." IEEE Internet of Things Journal (2023). (Enhancing GNSS Positioning in Urban Canyon Areas via a Modified Design Matrix Approach | IEEE Journals & Magazine | IEEE Xplore).

[201]	IEEE 802.11bf/D8.0, “Draft Standard for Information technology – Telecommunications and information exchange between systems Local and metropolitan area networks – Specific requirements, Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications, Amendment 4: Enhancements for Wireless LAN Sensing”. (IEEE SA - IEEE 802.11bf-2025).

[202]	ETSI GR ISC 001 V1.1.1 (2025-03): “Integrated Sensing And Communications (ISAC); Use Cases and Deployment Scenarios”.

[201]	Wymeersch, H., Parssinen, A., Abrudan, T. et al (2022). 6G Radio Requirements to Support Integrated Communication, Localization, and Sensing.  2022 Joint European Conference on Networks and Communications and 6G Summit, EuCNC/6G Summit 2022: 463-469. http://dx.doi.org/10.1109/EuCNC/6GSummit54941.2022.9815783

[202]	3GPP TS 22.220: "Service requirements for Home Node B (HNB) and Home eNode B (HeNB)".

[203] 	3GPP TS 38.104: "Base Station (BS) radio transmission and reception".

[204] 	3GPP TR 38.799: "Study on additional topological enhancements for NR".

[205]	Press Release: Press Information Bureau.

[206]	Next G Alliance, “Roadmap to 6G”. (https://nextgalliance.org/white_papers/roadmap-to-6g/)

[207]	Next G Alliance, “Trust, Security, and Resilience for 6G Systems”. (https://nextgalliance.org/white_papers/trust-security-and-resilience-for-6g-systems/)

[208]	NGMN, “Cloud Native Manifesto”. (https://www.ngmn.org/publications/cloud-native-manifesto.html).

[209]	GSMA RCC.71: “RCS Universal Profile Service Definition Document”.

[210]	Petri Ahokangas; Marja Matinmikko-Blue; Arturo Basaure; Seppo Yrjölä , “Use Cases for Local 6G Networks”, 2024 Joint European Conference on Networks and Communications & 6G Summit, June 2024, Antwerp, Belgium, (https://oulurepo.oulu.fi/bitstream/handle/10024/54699/nbnfioulu-202503252188.pdf?sequence=1)

[211]	“White Paper on 6G Networking”, 6G Research Visions, No. 6 June 2020, University of Oulu, isbn978-952-62-2684-2.pdf.

[212] 	3GPP TS 32.298: "Charging data record (CDR) parameter description".

[213]	3GPP TS 32.299: " Telecommunication management; Charging management; Diameter charging applications".

[214]	X. Lin, L. Kundu, C. Dick, E. Obiodu, TT. Mostak, “6G Digital Twin Networks: From Theory to Practice”, https://arxiv.org/pdf/2212.02032.

[215]	https://arxiv.org/pdf/2212.02032.

[216]	TM Forum, IG1218 Autonomous Networks Business Requirements and Architecture, Autonomous Networks Business Requirements and Framework v3.0.0 (IG1218) – TM Forum.

[217]	Claudio Fiandrino, Giulia Attanasio, Marco Fiore, Joerg Widmer, “Toward native explainable and robust AI in 6G networks: Current state, challenges and road ahead,” Computer Communications, Volume 193, 2022. (Toward native explainable and robust AI in 6G networks: Current state, challenges and road ahead - ScienceDirect).

[218]	L. U. Khan, W. Saad, D. Niyato, Z. Han and C. S. Hong, "Digital-Twin-Enabled 6G: Vision, Architectural Trends, and Future Directions," in IEEE Communications Magazine, vol. 60, no. 1, pp. 74-80, January 2022. ([2102.12169] Digital-Twin-Enabled 6G: Vision, Architectural Trends, and Future Directions).

[219]	NGMN Alliance, 6G Requirements and Design Considerations, February 2023, https://www.ngmn.org/publications/6g-requirements-and-design-considerations.html.

[220]	SWS-240002, "GSMA Proposal on 6G", 3GPP Stage 1 Workshop on IMT2030 Use Cases. (https://www.3gpp.org/ftp/workshop/2024-05-08_3GPP_Stage1_IMT2030_UC_WS).

[221]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[222] 	K.-D. Lee and C. Gray-Preston, "Everyday Living Assisted by 6G Applications and Solutions," IEEE Wireless Communications Magazine, October 2022. (Everyday Living Assisted by 6G Applications and Solutions | IEEE Journals & Magazine | IEEE Xplore).

[223] 	Statista, https://www.statista.com/topics/1143/mining/.

[224]	US Energy Information Administration, Homepage - U.S. Energy Information Administration (EIA).

[225]	NGMN Alliance, ITU-R Framework for IMT-2030: Review and Future Direction, February 2024, ITU-R Framework for IMT-2030 - NGMN.

[226]	“The Public Protection and Disaster Relief Liaison Rapporteur”, (https://www.itu.int/pub/R-QUE-SG05.209-7-2023).

[227] 	“Question ITU-R 209-7/5” Use of the Mobile, Amateur, and Amateur-Satellite Services in Support of Disaster Radiocommunications                                                                        (https://www.itu.int/md/R23-WP5A-C-0182/en).

[228]	 “Guideline of the MIIT and 14 Other Departments on Strengthening Emergency Communication Capacity Construction in Extreme Scenarios” (https://www.gov.cn/zhengce/zhengceku/202501/content_7000295.htm).

[229]	XGMF: "Beyond 5G White Paper ~Message to the 2030s~" (https://xgmf.jp/wp-content/uploads/2025/04/Beyond_5G_White_Paper_v4_0.pdf).

[230]	American Society of Civil Engineering, ASCE7 Minimum Design Loads and Associated Criteria for Buildings and Other Structures, ASCE Library.

[231] 	https://docs.datagnss.com/d303-docs/common/about-rtk/.

[232]	3GPP TS 28.318, “Network and Service Operations for Energy Utilities (NSOEU)”.

[233]	3GPP TR 28.829: “Study on network and service operations for energy utilities”.

[234]	Lai Z, Hu Y C, Cui Y, et al. Furion: Engineering high-quality immersive virtual reality on today's mobile devices[C]//Proceedings of the 23rd Annual International Conference on Mobile Computing and Networking. 2017: 409-421.

[235]	Kelkkanen V, Fiedler M, Lindero D. Synchronous remote rendering for VR[J]. International Journal of Computer Games Technology, 2021, 2021(1): 6676644.

[236]	Stojanovic R, Weinrauch A, Tatzgern W, et al. Efficient Rendering of Participating Media for Multiple Viewpoints[C]//High-Performance Graphics 2023: HPG 2023. Eurographics-European Association for Computer Graphics, 2023: 55-64.

[237]	3GPP TS 37.320: “Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2”.

[238]	3GPP TS 38.331: “NR; Radio Resource Control (RRC); Protocol specification”.

[239]	3GPP TS 32.422: “Telecommunication management; Subscriber and equipment trace; Trace control and configuration management”.

[240]	3GPP TS 23.273: “5G System (5GS) Location Services (LCS); Stage 2”.

[241]	3GPP TS 33.558: “Security aspects of enhancement of support for enabling edge applications”.

[242]	3GPP TS 26.531: “Data Collection and Reporting; General Description and Architecture”.

[243]	3GPP TR 38.843: “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[244]	National Institute of Standards and Technology (NIST), The NIST Cybersecurity Framework (CSF) 2.0, February 2024, (The NIST Cybersecurity Framework (CSF) 2.0).

[245]	3GPP TR 23.700-03: “Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 3”.

[246]	Internet Engineering Task Force (IETF), RFC 7181, The Optimized Link State Routing Protocol Version 2. (RFC 7181: The Optimized Link State Routing Protocol Version 2).

[247]	3GPP TS 23.434: “Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows”.

[248]	3GPP TR 22.874: “5G System (5GS); Study on traffic characteristics and performance requirements for AI/ML model transfer”.

[249]	3GPP TR 22.876: “Study on AI/ML Model Transfer Phase2”.

[250]	3GPP TS 33.501: “Security architecture and procedures for 5G System”.

[251]	3GPP TS 28.104: “Management and orchestration; Management Data Analytics (MDA)”

[252]	3GPP TS 28.404: “Telecommunication management; Quality of Experience (QoE) measurement collection; Concepts, use cases and requirements”

[253]	3GPP TS 28.405: “Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration”

[254]	3GPP TS 28.406: “Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport”.

[255]	GSMA Intelligence, “Network Transformation 2025”, December 2024, https://www.gsmaintelligence.com/research/network-transformation-2025.

[256]	GMSA Intelligence, “The Mobile Economy 2026”, March 2025, https://www.gsmaintelligence.com/research/the-mobile-economy-2025.

[257]	3GPP TS 28.538: “Management and orchestration; Edge Computing Management”.

## [X1] 3GPP TS 23.287 V18.4.0 (2024-10), “5G; Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services”

## [X2] ITU Publications, “AI for Good Innovate for Impact Interim Report 2025; Use Case 4: Low-Cost Decision Making for Group of Vehicles for Safe Mobility in V2X”

## [X3] 3GPP TS 23.286 V18.6.0 (2025-01), “LTE; Application layer support for Vehicle-to-Everything (V2X) services; Functional architecture and information flows”

- [X4] D. F. Kulzer et al., “AI4Mobile: Use Cases and Challenges of AI-based QoS Prediction for High-Mobility Scenarios,” Apr. 2021, doi: https://doi.org/10.1109/vtc2021-spring51267.2021.9449059.

- [X5] Alexandros Palaios et al., “Machine Learning for QoS Prediction in Vehicular Communication: Challenges and Solution Approaches,” IEEE access, vol. 11, pp. 92459–92477, Jan. 2023, doi: https://doi.org/10.1109/access.2023.3303528.

# * * * * Second Change * * * *

## 3.1 Terms

## Dynamic Group Management: For some group-based instances, the V2X UEs are required to be determined dynamically (with the assistance of the V2X application server or independently by the V2X UEs when they are not connected to the network) during the operation, as defined in TS 23.286 [4]. The dynamic group management includes information flows such as group ID mapping, configure request, configure response, configure notification, etc.

## Multi-environment: Multi-environment data refers to metadata such as weather condition, traffic jam factor, etc. from different environments such as highways, avenues, residential, and park, etc.

## pQoS: pQoS refers to predictive QoS data which includes parameters like throughput, latency, jitter, packet loss, etc.

## Pre-programmed ML models: An ML model that has already been trained on a large dataset and can be fine-tuned for a specific task, reducing the need for training from scratch.

## Stationarity Check: A statistical method used to check if a time series maintains consistent statistical properties throughout its duration. It checks for change in unconditional joint probability distribution of wireless network data with time.

* * * Next Change * * * *

# 6	 AI

## 6.xx	Adaptive Group Management and Task Offloading in V2X Platooning for 6G system

### 6.xx.1	Description

3GPP defines vehicle platooning as a service that allows multiple vehicles to form a moving group on the road, dynamically. The leader vehicle regularly shares data with the others to help the platoon operate smoothly [66]. This shared information allows vehicles to travel with very minute gaps, less than a second apart. Platooning is an application layer managed group service, which follows the groupcast communication mode where any UE in the group can act as transmitting UE and the rest act as receiving UEs [X1,X4]. If the V2X application layer specifies a group size and member ID, the information is forwarded to the Application Server (AS) layer to manage groupcast control. V2X group management is handled by the V2X application layer. The V2X Application Server (V2X AS) [66, X3] handles QoS analytics over PC5 and Uu reference points. For V2X communication over PC5 reference point, a standard set of PQIs is defined for Platoon service flows with specific QoS requirements [X1,X4].

Figure 6.xx.1-1: Platooning of vehicles scenario and 6G system interactions

We consider a platooning of vehicles scenario, as shown in Fig 6.xx.1-1 same route (highway/avenue/Residential/Commercial) and connected to the 3GPP network system (5G/6G). The vehicles within the group trail a single leading vehicle positioned ahead of other vehicles in the same group.

The V2X application server, as shown in the above Figure is responsible for group management decisions, including group creation, group deletion, group information query, etc., and their triggers [X3, 247]. In changing wireless network condition, the participating UEs need to be identified dynamically during group management activities. Since TS 23.286 does not cite the motivation for dynamic group formation, we propose a mechanism that utilises a stationarity check of multi-environment data as the trigger for dynamic group formation and reformation. The stationarity check is a process which checks for change in wireless network conditions with time, such as vehicle moving from highway to residential or park or commercial locations. The stationarity check results help in forming the optimal platoon, maintaining safe distances between vehicles, and enhancing overall traffic flow. Most of the applications today use pre-programmed ML models which are designed for specific scenarios, limiting their adaptability to diverse mobility conditions and varying network environments. Whereas the analytics block (MEC) utilises the result of the stationary test to fine-tune the hyperparameters at runtime for more accurate prediction.

### 6.xx.2	Service flow

- Data Collection: Gather vehicular sensor data (location, speed, topography) and environmental data from roadside units, along with network metrics on wireless link conditions from network elements.

- Edge Analytics: Transmit the collected data to the analytics block, hosted at the MEC, for processing.

- Stationarity Testing & Group Update: Perform a stationarity test at the analytics block to reorganize vehicle groups and elect a leader.

- Dynamic Group Formation & Model Adaptation:

  - Share updated group and leader information with all vehicles in the group.

  - Provide stationarity test outcomes to ML models for fine-tuning, improving prediction accuracy.

- Task Offloading Decision: Predict QoS and send results to the leader.

  - If QoS meets the threshold, the leader requests task offloading via the base station to the MEC analytics block.

  - If QoS is below threshold, the leader processes the task locally and shares the decision with the group.

- Task Execution & Dissemination: The MEC processes offloaded tasks and returns results to the leader, who then informs the group to optimize resource usage.

In conclusion, the 6G system shall support efficient dynamic vehicle group control mechanism for V2X platoon services, which includes the following characteristics:

Adaptive ML models: Environment and domain adaptation is a key challenge in ML, especially when models which are trained in one environment/domain need to be retuned effectively to a different but related domain.

QoS prediction-based task offloading: Making V2X Platoon application tasks more cost-effective by efficient task offloading decision by enhanced QoS predictions. With these advancements, the research can transition from theoretical modelling to practical, real-world applications, enhancing the security of V2X systems [X2].

6.xx.3 Existing Features and Gap analysis

3GPP’s V2X framework (Release 16+) provides the architectural foundation and enabler services for platooning and other group-mobility V2X use cases [66]. In platoons, vehicles share similar sensing and computation needs, enabling redundancy reduction through cooperative sensing and task offloading to lower in-vehicle compute loads. Dynamic group management [X3] offers mechanisms to assess wireless link quality and compute resource information, coordinate leader-based offloading, and support collaborative execution, while also enabling network-assisted offloading. However, 3GPP provides infrastructure enablers, not prescriptive decision-making algorithms. The 5G/6G specification specifies that the V2X group management is only handled by the V2X application layer, but the process of deriving this information and implementing these adjustments is not covered [TS 23.287]. Following are the key gaps of note:

- The lack of integration of predictive QoS in group decisions for dynamic group formation in platoon service

- The absence of context awareness for V2X platoon service (e.g., Vehicular data, Road topology and weather information, wireless link state data) in dynamic group management and task offloading strategies.

The following requirements fill the gaps identified in clause 6.xx.2, where support is lacking in the existing standard.

### 6.xx.4	Potential New Requirements

[PR 6.X.X-001]: The 6G system shall support QoS that 6G must satisfy to support real-time decision-making [Table 5.4.4-1: Standardized PQI to QoS characteristics mapping, TS 123.287].

[PR 6.X.X-002]: Based on user consent, operator policy and regulatory requirements, the 6G system shall expose service specific contextual information. Network must expose few RAN related information and UE has to expose vehicular data, such as, position, speed, road topology and condition and weather information to the third-party application.

[PR 6.X.X-003]: Analytics block must refer to dynamic ML models which can be selected based on explicit needs, such as support for online learning, and real-time adaptability (through transfer learning or continual learning).

[PR 5.X.2-004] The 6G system shall support stationarity test based on network information vehicle data or other metadata context.