3GPP TSG-SA WG1 Meeting #111 	S1-253305

25-29 August 2025, Goteborg, Sweden	(revision of S1-252656, S1-252329)

Title:	Use case on 6G providing wireless power transfer service

Agenda Item:	8.1.7

Source:	Turk Telekom, Huawei, HiSilicon,

Contact:	Cagri Ozgenc Etemoglu, Eddy Hall,

cagriozgenc DOT etemoglu AT turktelekom DOT com DOT tr

edward DOT hall AT huawei DOT com

Abstract: This contribution introduces a new use case on 6G network providing wireless power transfer service to mobile devices, e.g. IoT devices.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

<Skip>

[257]	3GPP TS 28.538: “Management and orchestration; Edge Computing Management”.

[x1]	Akpakwu, Godfrey Anuga, et al. "A survey on 5G networks for the Internet of Things: Communication technologies and challenges." IEEE access 6 (2017): 3619-3647.

[x2]	https://www.globenewswire.com/news-release/2024/09/12/2945228/0/en/Industrial-Safety-Market-Set-to-Surge-to-USD-12-39-Billion-by-2032-Driven-by-Technological-Advancements-and-Rising-Safety-Regulations-SNS-Insider.html

[x3]	https://www.rdbatteries.com/blog/post/how-long-do-rechargeable-batteries-last.html

[x4]	ATEX Directive 2014/34/EU: “Equipment and protective systems intended for use in potentially explosive atmospheres”

[x5]	IEC 60079-0:2017: “Explosive atmospheres - Part 0: Equipment - General requirements”

[x6]	IEC 62311:2019: “Assessment of electronic and electrical equipment related to human exposure restrictions for electromagnetic fields (0 Hz to 300 GHz)”

[x7]	Abou Houran, Mohamad, et al. "Wireless power transfer: critical review of related standards." 2018 International Power Electronics Conference (IPEC-Niigata 2018-ECCE Asia). IEEE, 2018.

[x8] 	H. Jiang, Z. Wu, H. Min, “A -31dBm Sensitivity High-Efficiency RF Energy Harvesting System With Burst Charging Mode for IoT Applications,” IEEE Transactions on Circuits and Systems,

* * * Second Change (new text) * * * *

## 10.x	Use case on wireless power transfer

### 10.x.1	Description

Power consumption has always been the key problem for the IoT devices. Considering the rise of the factory management expenses due to labour and device cost for battery change or wiring cost, the deployed base station could act as steady source of energy via electromagnetic radiation of wireless signals to support operation of those devices, i.e. base station provides wireless power transfer (WPT) service.

Powering the IoT devices with wired connection or batteries presents several challenges including operational costs, logistical complexity, and cost of wiring or battery replacement, particularly in large-scale industrial environments [x1]. Failure to provide the regular maintenance of wired connections/batteries depending on the environment can lead to fatal accidents such as fire bursts, explosions, hazardous gas emissions. To prevent such accidents, industrial safety market is expected to grow substantially and reach 12.39 billion by 2032 [x2]. One typical example is the instruments/sensors in the chemical factory to monitor the processing procedure. Currently, these instruments/sensors are wired deployed or equipped with a battery. For the wired deployment, it usually involves the labour cost, plastic pipe cost, construction of shallow trench, etc., which leads to a huge deployment cost. To match ever changing product requirements, layout of factory production lines is changing rapidly leading to more wiring costs and excessive delays in the reconfiguration of the production lines due to the inherent inflexibility of wired deployments. On the other hand, for the case of battery equipment, periodic or temporary battery replacement is expected every 2-3 years [x3] which involves not only high management cost but also several risks due to potential human operational errors. Under some hazardous areas/core production areas, wired deployment or battery replacement are prohibited or even impossible.

Due to the concerns stated above, in environments where flexible deployment, robustness and high availability of sensor measurements with minimum downtime (i.e. less human intervention) are critical, wireless power transfer provides a unique solution with intrinsic features not available in alternatives. Still WPT service needs to satisfy required compliance with industrial safety regulations for the target environment. For the case of chemical factory, the WPT system needs to comply with explosive atmosphere regulations (ATEX / IECEx standards ([x4], [x5]) and must ensure the electromagnetic radiation levels remain within safe limits for personnel [x6], [x7]. Being a highly flexible service, wireless power transfer may run during periods and/or within areas with minimal to no human presence allowing the limits for power transfer to increase for optimized efficiency and effectiveness without causing health hazards.

Ambient IoT (AIoT) considered passive energy harvesting to address the issue, e.g., solar, wind, RF-powered – the IoT device passively harvests energy based on existing wireless signals in the environment and does not interact with the network.  But, powered by ambient energy harvesting requires the AIoT device to be located near an energy source, such as heat, wind or base station. In some cases, there may be no energy source in the environment and IoT devices cannot harvest energy (e.g., solar-powered devices cannot work at night). In other cases, AIoT devices may not be able to harvest enough energy from the environment. For example, when using passive energy harvesting, AIoT devices passively receive wireless signals (which may be a communication signal sent to other users) for charging. The power of passively received signals is often low and may not meet the charging power required by AIoT devices, and the communication waveform is designed for data transmission rather than only energy delivery. Hence, the energy harvesting efficiency from the ambient environment cannot be guaranteed. Ambient energy harvesting is dependent on the external environment which possibly cannot provide a steady supply of energy. WPT service can be used alongside ambient energy harvesting, or other power source techniques to ensure a resilient power supply to remote devices. Using WPT service alongside other power source techniques would also decrease the amount of power required from WPT leading to lower power emissions thus mitigating any potential harmful effects of the service.

In order to deal with the situation described above, more active power delivery for low-power IoT devices can be considered which uses radio frequency wireless power transfer technology. The charging station transmits radio energy signals to low-power devices, which receive them using a rectifying antenna and recharge their batteries. When it comes to 3GPP, base stations can actively transmit energy signals to the low-power devices in response to  charging requests, i.e. wireless power service. The 6G network can provide wireless power service (e.g., inform base station/charging station to transmit energy signals) after receiving the charging request from the low-power IoT device. Alternatively for certain environments instead of relying on charging requests, the 6G network can actively monitor power levels of devices. The 6G network can also flexibly prioritize WPT service for different types of IoT devices based on their functionality (e.g., an IoT device responsible for measuring hazard indicators should receive higher priority than an IoT device measuring non-critical informational indicators). The IoT device can exchange related charging status information to network to improve charging efficiency (e.g. report received signal strength indicator).

### 10.x.2	Pre-conditions

Various sensors are widely deployed within industrial buildings/factories to monitor the environmental information such as temperature/humidity/air/water quality and/or infrastructure status such as equipment vibration, surface deformation for factory safety and continuous operation. These sensors are registered to the network to report real-time information. Instead of using a wired connection, these sensors are equipped with energy storage components, such as capacitors and batteries, and these components can be recharged. When the battery level is below a threshold, these sensors could send a request to the network for wireless energy service. According to the instructions of the 6G network, the base stations can emit wireless energy signals.

The deployment environment should meet RF safety requirements for industrial facilities. Critical safety sensors (e.g. fire detection, gas leak and pressure monitoring) would have priority access to wireless power service. The WPT system would be capable of handling both stationary sensors and mobile devices (handheld or any otherwise non-stationary device) within the coverage area.

### 10.x.3	Service Flows

- Wireless IoT devices S1 to Sn are deployed within a warehouse/factory to obtain and report environmental information, such as smoke/temperature, etc.

- 6G network indicates that wireless power service is available.

- After a period of time continuous working, devices S1 to Sn are almost out of the stored energy and request energy service from the network. The charge request may include identification information, message identifiers, charge duration, current energy status, estimated battery remaining time,  etc.

- Upon the requests, the 6G network would authenticate the UE, authorize a priority level for charging (i.e. critical safety sensors have higher priority) and indicate/coordinate the base station(s) to provide wireless energy service to the devices S1 to Sn.

- During charging in case a higher priority critical charging request received by the 6G network, the network should immediately prioritize this new critical request and if needed temporarily suspend non-critical device charging.

- Using the received energy signals, the devices S1 to Sn store energy for future usage.

- The devices S1 to Sn send charge termination request to 6G network when the stored energy reach certain threshold.

- The 6G network stops the wireless power service, e.g. the network notifies the base stations (or charging stations) to stop sending energy signals.

- IoT devices S1 to Sn remain normal work during wireless charging and continue to monitor environmental data.

Figure x.3-1: Example Flow for Wireless Power Transfer

With regard to the wireless power service duration (how long the network emits wireless power signals each time) and frequency (how often the network emits wireless power signals), one of the main factors is the device’s capability. If a device has greater energy storage capacity, it would require longer service time and lower service frequency. On the contrary, if the device has smaller energy storage capacity, it generally would require less service time and higher service frequency. Another factor is the operation flow of the device, which means how often the UE would use the stored energy to do transmission operation or sensing operation. More intensive operation flow would lead to higher wireless power transfer frequency. Hence, the wireless power transfer time would generally span from seconds to hours and wireless power transfer service frequency would generally span from several times per minutes to once per day.

### 10.x.4	Post-conditions

Thanks to the wireless power transfer services provided by the network, the sensor based IoT devices could be recharged without involvement of labour effort. By avoiding manual labour the risks associated with human errors could be minimized, continuous operation of the factory can be assured, critical IoT devices could always have power. Besides lifespan of the IoT devices could be greatly increased, and hence the factory management cost due to battery replacement is significantly decreased. The network-based wireless power transfer can provide timely, continuous, accurate, and adequate energy, which is a reliable source for IoT devices. With WPT service, industrial facilities could handle power needs of their ever increasing critical sensors with unprecedented reliability and flexibility.

### 10.x.5	Existing features partly or fully covering the use case functionality

SA1 has studied environment monitoring based on ambient IoT sensors in TR 22.840 [1], especially, use case on ambient IoT sensors in smart homes, further specified in clause 6.48 of TS 22.261. In AIoT, the service requirements are defined from the communication aspects. However, one of the most important aspects of the sensor based IoT devices without battery/with limited battery is energy supply. The AIoT sensors only consider energy harvesting from the ambient environment. The ambient energy availability can be opportunistic and unsteady and therefore the power supply of the AIoT-based sensors cannot be guaranteed.

### 10.x.6	Potential New Requirements needed to support the use case

[PR 10.x.6-1] Subject to regulatory requirements, the 6G network shall be able to provide wireless power service to both stationary and mobile IoT device(s) within the coverage area.

[PR 10.x.6-2] Subject to operator’s policy, the 6G system shall be able to support wireless power service control and management, e.g., collect energy service request, activate and/or deactivate wireless power service.

[PR 10.x.6-3] The 6G IoT devices shall be able to request and receive wireless power service from 6G network.

[PR 10.x.6-4] Subject to operator’s policy, the 6G network shall support to authenticate an IoT device(s) to request wireless power service.

[PR 10.x.6-5] Subject to operator’s policy, the 6G network shall support to authorize with priority an IoT device(s) of prioritized users for using wireless power service.

[PR 10.x.6 -6] Subject to operator’s policy, the 6G network shall be able to provide 3GPP WPT service for 6G IoT devices with performance specified in Table 10.x.6-1.

Table 10.x.6-1: KPIs for 3GPP WPT for IoT device

| Scenario |  | Charging Duration (note 1) | Charging Power Level (note 2) | Charging Latency (note 3) | Charging Service Availability (note 4) | Charging Service Reliability (note 5) | Charging Range | Device density | IoT Devices moving speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WPT for Low Power IoT devices |  | TBD | -30dBm or 1 µW or 1 µJ/s | TBD | 99% | TBD | 300m | <500/km2 (note 6) | Stationary & Mobile |
|  | NOTE 1: Charging Duration means the amount of time an IoT device takes from the time it initiates a charge request to the time it completes the charge. NOTE 2: Charging Power Level is the delivered power based on the power level of the received signals at the RF front end. This is based on the analysis in Annex A. NOTE 3: Charging Latency means time elapsed between the initiation of charging request and the availability of wireless power service at an IoT device. NOTE 4: Charging Service Availability means the proportion of successfully completed charging service times to the total number of services. NOTE 5: Charging Service Reliability means the proportion of the time for receiving signal power to reach the charging threshold to the charging duration NOTE 6: It is obtained from the survey of several chemical factories, which could be typical value. |  |  |  |  |  |  |  |  |

* * * Third Change (new text) * * * *

### Annex A	Wireless power transfer KPI value derivation

The KPIs for Wireless power transfer in the present document are derived based on the evaluation of the link budget as shown in Table Annex A-1.

Table Annex A-1: KPI derivation for 3GPP WPT for IoT device

| Carrier Frequency | 6.7 | 4.9 | 3.5 | 2.6 | 0.9 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth | 15 | 15 | 15 | 15 | 15 |
| Tx Power(W) | 40 | 40 | 40 | 40 | 40 |
| Tx Power(dBm) | 46.02 | 50 | 50 | 50 | 50 |
| Tx Antenna Gain(dBi) | 25 | 22 | 20 | 18 | 12 |
| EIRP(dBm) | 71.02 | 72 | 70 | 68 | 62 |
| Multi-Station | 5 | 5 | 5 | 5 | 5 |
| Coherent Gain | 5 | 5 | 5 | 5 | 5 |
| Rx Antenna Gain | 13 | 11 | 8.5 | 7.5 | 3 |
| RF Power Sensitivity (dBm) | -30[x8] | -30 | -30 | -30 | -30 |
| Link Budget (dB) | 124.02 | 123.00 | 118.50 | 115.50 | 105.00 |
| UMa NLOS Distance (m) | 318.64 | 362.97 | 321.58 | 311.42 | 282.17 |

It is noted that according to the link budget evaluation, the RF wireless power transfer could support up to about 300 meters coverage with -30dBm RF sensitivity, i.e. delivering 1 µW or 1 µJ/s.

* * * End of Change * * * *