3GPP TSG-SA WG1 Meeting #111 	S1-253317

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:	Siemens, EDF

New use case title:	New use case on 6G-enabled Decentralized Smart Grid Control

Draft TS/TR:	3GPP TS / TR 22.870

Agenda item:	8.1.8

Document for:	Approval

Contact:	Michael Bahr ( bahr et siemens dod com )
Daniel Hauer ( daniel dod hauer et siemens dod com)
Kim Schindhelm ( corina desh kim dod schindhelm et siemens dod com )

Abstract: New Use Case on 6G-enabled Decentralized Smart Grid Control for 3GPP SA1 TR 22.870 Clause 11 Further Use Cases on Industry and Verticals.

## 11.x	Use case on 6G-enabled decentralized smart grid control

### 11.x.1	Description

This use case addresses the transformation of traditional, fossil-fuel-based power grids into intelligent, renewable-powered distribution systems—an evolution that is not only technological but also strategic, given that the smart grid is a critical infrastructure underpinning modern society.

As renewable energy sources, proactive loads (e.g., EV chargers), and energy communities become increasingly integrated, low-voltage (LV) grids—historically passive and over-provisioned—must evolve into actively managed systems. These grids now require real-time monitoring, control, and coordination to maintain stability, efficiency, and resilience.

Traditionally, grid operation has relied heavily on redundancy and conservative planning to ensure resilience. For example, the N-1 contingency criterion mandates that transformer substations operate well below their physical limits to accommodate potential failures. In practice, this results in significant unused capacity, which is increasingly unsustainable as electrification and renewable integration accelerate. Moreover, the grid shall remain operational even under communication loss or cyber-attacks. This has led to architectures that assume worst-case scenarios, often resulting in over-provisioning.

This use case involves a hierarchical grid structure, where medium-voltage (MV) networks supply multiple low voltage (LV) grids. Each LV grid is managed by a distributed LV grid computer system (DS), equipped with 6G communication interfaces. These decentral systems coordinate with MV controllers and neighbouring substations to manage grid flexibility—such as load shifting or energy storage utilization—based on real-time demand and grid conditions.

This use case shows a paradigm shift: by enabling fully decentralized, real-time cluster coordination among local grid cells via 6G, meshed DSs can form resilient clusters during fault scenarios. These clusters autonomously optimize local grid behavior, maintaining stability without backend connectivity. This approach allows for more realistic and efficient fallback modes, reducing the need for excessive reserves and unlocking critical latent capacity that is urgently needed to support the energy transition. Figure 11.x.1-1 visualizes the concept of decentralized, real-time cluster coordination among local grid cells. Multiple DSs (DS1-6) are part of one MV grid with a central controller (CS) and each LV grid can manage its own flexibilities. During full connectivity, the CS can optimize the whole MV grid by coordinating all LV grids. If the connection between CS and its DSs is corrupted or fails, the LV grids form clusters with neighbouring LV grids (e.g. DS2 with DS3). Within the clusters the LV grids can optimize themselves and avoid being limited to a worst-case fallback for the case of no connection.

Figure 11.x.1-1: Example of intelligent, renewable-powered distribution systems

Table 11.x.1-1: Operational scenarios – illustration

|  | Normal mode: All LVs belong to one MV cell and are connected (through their corresponding decentralized unit) to MV controller |
| --- | --- |
|  | Connectivity from LVs to MV controller is lost: Needs to be omitted (e.g. by connecting to gNB in reach, UPS in gNB, etc.), otherwise cluster mode |
|  | Cluster Mode (local) Connectivity loss to MV of one or more LVs: Create communication clusters of LVs in vicinity to enable decentralized data exchange between e.g. LV2 and LV3 |
|  | Cluster Mode: Failure that does not allow data exchange between LVs or LVs and MV: Form communication clusters where possible (to allow for data exchange between as many LVs as possible) |

Figure 11.x.1-2: Concept of decentralized, real-time cluster coordination among local grid cells in different operation scenarios

Table 11.x.1-1: Description of operational scenarios

| Scenario | Description | Technical Requirements | Gaps/Challenges |
| --- | --- | --- | --- |
| Normal Operation | Normal Grid operation where all LV grids have full 6G communication with the MV grid controller. The MV grid controller will then collect all LV measurement data and calculate the optimal setpoints. | - Low energy consumption: Support for energy-harvesting sensors. - Coverage: 100% area coverage, including hard-to-reach locations (e.g., basements). - User data rate: ~10 Mbit/s; - Time synchronization: < 1 ms E2E - Interoperability: Standardized onboarding, firmware updates. - Maintenance: Long-term QoS, automated software updates. - Guaranteed QoS: Bandwidth and service levels must be maintained. | - automated sensor onboarding and firmware updates. - Coverage assurance in underground or shielded environments. |
| Cluster Mode (Backbone Network Unavailable) | Communication to MV grid controller (backbone) is not available, and therefore all DS try to form local clusters using available 6G cells for cluster-wise optimizations. | - Cluster Mode: Flexible cluster formation across LV grids and substations. - Guaranteed bandwidth/service levels must be maintained. - Cluster-local communication with minimal latency. - Ultra-precise time sync (< 2 µs) in decentralized clusters | - Ultra-precise time sync (< 2 µs) in decentralized clusters is challenging - Dynamic cluster management protocols for the Smart Grid domain. |
| 3. Failure Mode (Grid Failure Scenario) | A failure affects the operation of one LV grid leading to a blackout. Failure protocols and high-resolution measurements have to be transmitted for root cause analysis. | - Time synchronization: < 2 µs for PMUs/differential protection E2E - Backup power: 30 min UPS for communication systems. - High bandwidth: >100 Mbit/s for high-res data. - Spontaneous high data transmission with guaranteed latency. - Dynamic power adaptation of 6G transmitters. | - Energy-efficient ICT operation - Latency guarantees under failure conditions are difficult to maintain. |
| 6G Sensing – Weather | Using 6G sensing to get information on local weather (e.g. rain) can improve the MV/LV optimization algorithms, as they highly rely on coarse forecasts. | - Local weather detection (sunshine, rain, temperature). | - 6G-native weather sensing capabilities. |
| 6G Sensor Localization | Using 6G to localize all sensors can be additionally used to find wrongly placed sensors and simplify sensor onboarding. | - Sensor localization without GPS <=1 m | - High-accuracy outdoor localization via 6G. |

Potential sustainability impacts

Table 11.x.1-2: Sustainability effects

| Environmental |  | Societal |  | Economic |  |
| --- | --- | --- | --- | --- | --- |
| 1st order effects | 2nd order effects | 1st order effects | 2nd order effects | 1st order effects | 2nd order effects |
| More local energy generation and consumption, including self-consumption | Higher density of sustainable assets | Cheaper energy | Greater satisfaction in industries and the general population | Reduced need for grid replanning | Less financial burden on low-income households |
| Efficient hardware usage | Minimized infrastructure needs | Sense of doing good | Increased GDP | Lower CAPEX/OPEX | Avoidance of blackouts |
| Higher % of renewables |  | Social safety | Community empowerment |  |  |
| Less CO₂ emissions |  |  |  |  |  |
| Higher density of PV, chargers, heat pumps |  |  |  |  |  |
| Increased energy consumption from 6G | GHG emissions from production/maintenance | User adoption barriers | Digital divide | High deployment and maintenance costs | Potential job losses in traditional energy sectors |
| Use of rare materials | E-waste from outdated components | Privacy concerns | Need for education and retraining |  |  |

### 11.x.2	Pre-conditions

- Decentralized system units are deployed and registered in the 6G network; their communication is secured.

- Flexibility-enabled assets (batteries, PV, EV chargers) are available in LV grids.

- 6G network supports cluster-mode fallback scanning and peer discovery.

- Local controller-to-controller communication available as redundant path in absence of backbone.

- Minimum local bandwidth reserved for emergency cluster coordination

### 11.x.3	Service Flows

<Describe the sequence of events that explain what needs to happen, preferably as a numbered list, e.g. 1. User makes a voice call, 2. Called party receives alerting message.>

- MV Grid Controller calculates and sends setpoints to DSs via 6G: The MV controller gathers real-time data from all LV grids and computes target values (setpoints) like power limits. These setpoints guide each DSs to adjust local loads and generation to keep the grid stable and efficient. The instructions are sent over the 6G network for timely and reliable control.

- DSs monitor grid conditions continuously: Local sensors provide real-time measurements to DSs to detect changes or faults in the LV grid. This allows immediate response to variations or anomalies.

- In normal conditions, MV controller maintains full communication with DS: All DSs send updated status and receive new setpoints continuously, ensuring centralized and coordinated control.

- If communication to MV controller is lost, DSs form local 6G clusters: DSs switch to a decentralized mode, coordinating only among themselves within a cluster using 6G cells. This ensures local balancing and resilience despite communication backbone failure.

- DSs within clusters optimize power sharing and load balancing locally: They exchange flexibility information to distribute demand and generation, avoiding overloads and maintaining supply without central guidance.

- Once backbone communication is restored, normal operation resumes: DSs resynchronize with the MV controller, updating setpoints and restoring centralized grid management.

### 11.x.4	Post-conditions

- DSs successfully implement received setpoints, adjusting local loads and generation to meet grid targets. This ensures that energy supply and demand are balanced locally, contributing to overall grid stability and efficiency.

- In case of communication loss to the MV grid controller, DSs formed resilient local clusters that maintain power balance within their geographic area. These clusters enable decentralized coordination, reducing the risk of widespread outages and allowing continued operation despite communication backbone failures.

- Local cluster communication optimizes flexibility sharing, preventing overloads and maintaining voltage/‌frequency within regulatory limits. This adaptive response helps to prevent blackouts and to meet strict power quality and safety standards even during faults or emergencies.

- Once communication with the MV controller is restored, DSs seamlessly resynchronize and revert to centralized control. This transition minimizes disruption and restores optimal grid-wide management.

- Grid resilience and supply security are improved by enabling real-time fault detection and localized response. Faster detection and reaction reduce blackout risks and support compliance with regulatory reliability criteria (e.g., N-1 contingency).

- Efficient use of renewable resources and storage assets is maintained, supporting sustainability goals. The system maximizes clean energy integration and reduces carbon emissions by effectively leveraging distributed flexibility.

.

### 11.x.5	Existing features partly or fully covering the use case functionality

- TS 22.104 contains service performance requirements for electric-power distribution and smart grid and central power generation. Table 5.2-1 list corresponding service performance requirements for related use cases / scenarios, for instance, on ultra-reliable low latency communications (URLLC). Related use case with corresponding KPI requirements are (a) electrical distribution – distributed automated switching for isolation and service restoration with end-to-end latency < 5 ms and service area of 30 x 20 km (b) primary frequency control, (c) distributed voltage control (d) distributed energy storage (e) distributed energy storage – data collection (f) advanced metering (g) intelligent distributed feeder automation (h) smart distribution – transformer terminal (i) high speed current differential protection, (j) distributed energy resources (DER) and microgrids with e2e latency of 3 ms and communication service availability of 99.9999%, (k) ensuring uninterrupted communication service availability during emergencies.

- TS 22.261: Service requirements for the 5G system; includes general support for IoT, URLLC, and industrial use cases.

- Support of IEEE 1588 / Precision Time Protocol in TS 22.104 Clause 5.6.1. IEEE 1588 protocol is used to synchronize clocks across networked devices with sub-microsecond accuracy. It operates over Ethernet and supports high-precision time distribution without requiring GPS at every node.

- User-specific clock synchronicity accuracy level 4 (TS 22.104, Table 5.6.2-1) requires < 1 µs 5GS synchronicity budget for up to 100 UEs.

- Several clock synchronization service performance requirements from IEC 61850-9-2 are listed in TS 22.104 Table 5.6.2-1. Support for precise time distribution and clock synchronization specific to smart grid applications e.g. IEC 61850-9-3, IEC 61850-90-5 is required in TS 22.104 Clause 5.6.1.

- TS 22.261 Clause 6.26.2.2 requires (unreliable) multicast communication with communication types Ethernet and Ipin 5G LAN virtual networks.

### 11.x.6	Potential New Requirements needed to support the use case

[PR 11.x.6-1]	The 6G systems shall support autonomous cluster formation and indirect and direct device 6G communication under communication backbone/ core failure conditions.

[PR 11.x.6-2]	The 6G system shall provide secure, resilient local communication fallback mechanisms.

[PR 11.x.6-3]	The 6G system shall provide continuous communication with agreed QoS between UEs associated with the same gNB for up to 24 h in case backbone connection of the gNB to the 6G network fails.

[PR 11.x.6-4]	The 6G system shall provide continuous communication with agreed QoS between UEs associated with neighboring gNBs for up to 24 h in case backbone connection of the gNBs to the 6G network fails.

[PR 11.x.6-4]	The 6G system shall be able to setup direct device communication between UEs in range in case communication between these UEs is not possible anymore through direct network communication.

[PR 11.x.6-5]	The 6G system shall enable dynamic cluster reconfiguration triggered by network topology changes or link failures for at least 24 hours (best case until communication network issues are resolved).

[PR 11.x.6-7]	The 6G system shall be able to provide sufficient resources for QoS communication services for cluster (re-)configuration with both centralized and decentralized application protocols.

[PR 11.x.6-6]	The 6G system shall guarantee emergency bandwidth reservation for critical cluster coordination traffic during degraded network states (e.g. loss of communication backbone / core connection).

[PR 11.x.6-8]	The 6G system shall facilitate distributed communication protocols that support cluster formation and cluster-level coordination under constrained network condition (efficient signalling in decentralized mode) based on local communication connectivity and provided application constraints.

[PR 11.x.6-9]	The 6G system shall guarantee <1 ms end-to-end latency and sub-microsecond synchronization accuracy (e.g. for control message exchange) in a service area of 20 km x 30 km.

[PR 11.x.6-9]	The 6G system shall support periodic reliable L2 multicast (in both decentralized and normal mode) including TX (sending timestamp) for TOF compensation < 1 ms accuracy