3GPP TSG-SA WG1 Meeting #111	S1-253338

25-29 August 2025, Goteborg, Sweden 	(revision of S1-25xxxx)

Source:	InterDigital, Turk Telekom

Use case title:	New use case on Smart Shopping Tracker

Draft TS/TR:	3GPP TR 22.870 v 0.3.1

Agenda item:	8.1.4

Document for:	Approval

Contact:	Catalina Mladin, Catalina.Mladin@Interdigital.com

## Abstract: This contribution proposes a Smart Shopping Tracker use case, with the sensing functionality including  Wi-Fi STA sensing

## ---------------- Change 1: -----------------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[x1]	Research and Markets, “Indoor Location Market Report”, March 2025, https://www.researchandmarkets.com/reports/5971012/indoor-location-market-report

## ---------------- Change 2: All new text ------------------------------------------

## 7.x	Use case on Smart Shopping Tracker

### 7.x.1 Description

The indoor location analytics is expected to see exponential growth from $14.09 billion in 2024 to $41.66 billion in 2029 at a compound annual growth rate (CAGR) of 24.2%. The growth in the forecast period can be attributed to initiatives for smart cities, inefficiency of the GPS technology in indoor premises, demand for navigation, growing need for accurate and real-time location services and increasing adoption of data analytics in the retail sector [x1]. This use case demonstrates the business opportunities of ISAC enabled indoor analytics for MNOs and service providers to offer novel services to the retail sector. The proposition is that a shop owner in a larger shopping mall is interested in knowing which products customers are checking out most in their store. The store owner then finds out about a new service a company called SENSOR - not affiliated with the shopping mall or with any particular MNO - offers to retailers, allowing them to learn which parts of their store (and products displayed in that area) customers are most interested in. This is conveyed as a 3D representation of the store's interior and a heatmap-like overlay showing the customer interest. Another important benefit of sensing by 6G is the fact that risk of personal privacy being compromised is much less compared to camera-based sensing approaches.

The scenario is illustrated in Figure 1 and illustrates SENSOR (third-party service provider), the store owner and two MNOs, MNO A and MNO B. Furthermore, the store (shown on the bottom left) rents a broadband connection from MNO A which is operational inside the store. Inside the store are users with 6G UEs from MNO A and MNO B and all devices are 6G and Wi-Fi sensing capable.

Figure 7.x.1-1: Smart Shopping Tracker Scenario Utilising Sensing Results from Multiple MNOs.

### 7.x.2	Pre-conditions

SENSOR has contracts with multiple MNOs and it is assumed that both MNO A and MNO B have 6G coverage of the shopping mall and have sensing capabilities in their 6G networks. Furthermore, the store owner also bought a broadband package from MNO A that comes with a sensing-enabled Wi-Fi AP and offers general internet access to all staff at the store.

As illustrated in Figure 1, both MNOs have an AF (Application Function) deployed in their network offering a Wi-Fi sensing enablement via an application on all 6G UEs.

UE1 and UE2 are registered with MNOA and UE3 is registered with MNOB. All three UEs are 6G and Wi-Fi sensing-enabled and have also registered with the AF of their operator.

Users of UE1, UE2, and UE3 have explicitly opted for being part of the sensing operations offered by their respective MNOs. This consent allows them to control when and where they can be part of a sensing operation, possibly in exchange for potential incentives.

The shop owner receives SENSOR services with information about which products were of most interest to their customers. Once the shop owner completed their sign-up process with SENSOR and booked a sensing service, SENSOR issues a sensing service request to MNO A and B providing the necessary information about the target sensing service area (e.g. cartesian coordinates of the shop outline), the desired sensing result KPIs for the sensing results, and the daily opening times of the store when sensing results should be provided.

### 7.x.3	Service Flows

At the opening time of store's business days, both MNOs start setting up their sensing activities to provide the sensing results SENSOR requested, executing the following service flows

- Using the provided location (i.e. TSSA) of the store, both MNO start continuously checking for available sensing entities (TRPs and UEs) in the TSSA for sensing operations.

- MNO A identifies UE1 and UE2 to be in proximity of the store and MNO B identifies UE3.

- Both MNOs start performing sensing operations with the identified UEs and sensing data is sent to the MNO’s CN for processing into sensing results for potential exposure to SENSOR.

- While continuously assessing the KPIs of the generated sensing results (e.g. target object identification confidence level, resolution accuracy), both MNOs' networks determine that the sensing result KPIs have not been met and consults their AF for assistance information to utilise Wi-Fi sensing of all three UEs located inside the TSSA.

- The AF of each MNO reaches out to the registered UEs that are capable of performing Wi-Fi sensing to provide sensing data using Wi-Fi APs that are discoverable by UEs.

- As UE1 and UE2 are with MNO A and the store’s AP is from the same MNO, the AF of MNO A assists MNO A’s CN to perform Wi-Fi sensing. MNO B’s AF can only assist with sensing data from the Wi-Fi scan results.

- Both MNOs are using the Wi-Fi sensing data albeit to different extent in their CNs and fuse it with the 6G sensing data to process them into sensing results and expose them to SENSOR.

### 7.x.4	Post-conditions

Both MNO’s can successfully provide sensing results that meet the requested KPIs through 6G and Wi-Fi sensing data fusion resulting in SENSOR to offer their service to the store's owner (i.e. generate a heatmap of customer interest overlayed onto a 3D representation of store interior).

The store owner can access the third-party service provider’s website through a browser and app at any point in time and check out in which aisle and shelve customers spent most of their time in the store.

### 7.x.5	Existing features partly or fully covering the use case functionality

TR 22.837 [9] has described use cases to monitor micro doppler effect by ISAC caused by chest rise/fall during sleeping. The sensing results represent the human respiration rate. TR 22.837 [9] has also described use cases for coarse gesture recognition for application navigation and immersive interaction.

In this use case, 3GPP ISAC is expected to leverage Wi-Fi STA for detection and tracking of comprehensive characteristics of individual environmental object to meet the same sensing KPIs per object type.

### 7.x.6	Potential new requirements needed to support the use case

Functional Requirements:

[PR7.x.6-1] Subject to user consent, operator’s policy and regulations, 6G Network should be able to collect sensing-related capabilities from sensing-enabled 6G UEs with sensing-enabled Wi-Fi STA functionalities.

[PR7.x.6-2] 6G Network should be able to configure sensing-enabled 6G UEs and sensing-enabled Wi-Fi STA to perform sensing.

[PR7.x.6-3] 6G Network should be able to decide whether 3GPP sensing, or Wi-Fi sensing, or both should be performed.

[PR7.x.6-4] 6G Network should be able to switch between 3GPP and Wi-Fi sensing while executing a sensing activity.

[PR7.x.6-5] 6G Network should be able to fuse 3GPP sensing data and Wi-Fi sensing data into sensing results.

[PR7.x.6-6] 6G Network should be able to expose a sensing result to more than one trusted third-party service providers.