3GPP TSG SA WG 1 Meeting #111 	S1-253344

25-29 August 2025, Goteborg, Sweden

Source:	Philips International B.V.

pCR Title:	New use case on end-to-end energy saving by cooperating UEs

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Walter Dees (walter.dees@philips.com)

Abstract: This pCR proposes a new use case for clause 5.8 Sustainability and Energy Efficiency

1. Introduction

Clause 5.8 in TR 22.870 contains several use cases related to improving energy efficiency. This contribution proposes a new use case for this clause.

2. Reason for Change

In current cellular systems, two or more UEs work independently of each other, even if carried by the same user and sharing a common subscription, such as a smartphone and smartwatch or smart glasses. And thus, these UEs are subject to a similar environment in terms of the cellular infrastructure or user interactions. Also each of these UEs would have to set up and maintain connection with the base station, whereby each UE incurs the overhead of these connections. This is a lost opportunity to reduce the end-to-end energy consumption, i.e., energy consumption of the two or more UEs, and the energy consumption of the network infrastructure.

For instance, a user, Dave, has a smart phone and smart glasses. Dave may be carrying out a call via his smart glasses, but (1) Dave may not be able to finish the call due to a low battery level of the smart glasses because the smart glasses require too much power to communicate (directly) with the base station and/or (2) the base station may need to use increased transmission/reception power to communicate with the smart glasses compared with the transmission/reception power required to communicate via the smart phone.  Thus, the system performance can be improved by exploiting the fact that Dave is carrying two UEs and they can be coordinated by the cellular system to cooperate with each other, and hence are able to optimize the end-to-end energy savings.

3. Proposal

Include the following use case in Clause 5.8.

* * * First Change * * * *

---------- Use Case template ----------

## 5.8.x 	 Use case on end-to-end energy saving by cooperating UEs

#### 5.8.x.1			Description

Users own and carry an increasing number of User Equipment (UEs) of different types, each of the UEs with their respective (shared) subscriptions so that each of the UEs can connect to the cellular network. Examples of those UEs include smart phones, smart watches, or smart glasses. Similarly, users when travelling, working, or enjoying their hobbies rely on their UEs for both entertainment and work. Oftentimes, users may reside or move together as a group,

In current cellular systems, those UEs work independently of each other, even if carried by the same user, and thus, each being subject to the same communication overhead. Similarly, users that reside or move together as a group are subject to similar radio environment parameters, mobility and/or user interactions. This is a lost opportunity to reduce the end-to-end energy consumption as a whole.

For instance, a user, Dave, has a smart phone and smart glasses. Dave may be carrying out a call via his smart glasses, but (1) Dave may not be able to finish the call due to a low battery level of the smart glasses because the smart glasses require too much power to communicate (directly) with the base station and/or (2) the base station may need to use increased transmission/reception power to communicate with the smart glasses compared with the transmission/reception power required to communicate via the smart phone.  Thus, the system performance can be improved by exploiting the fact that Dave is carrying two UEs and they can be coordinated by the cellular system to cooperate with each other optimizing the end-to-end energy savings.

#### 5.8.x.2 	Pre-conditions

Dave subscribes to the communication services of Operator A and registers his UEs, namely his mobile phone and smart glasses, as cooperating devices. Both his mobile phone as well as his smart glasses can work independently from each other. They can act as cooperating devices if they are in proximity of each other. The proximity can be detected e.g. through the use of location or ranging services or through non-3GPP connection between the two devices).

Operator A uses a cellular system that provides enhanced energy savings by coordinating the cooperation between UEs.

#### 5.8.x.3	Service Flows

1.	Dave decides to go for a long walk. He uses his smart glasses to show him navigation instructions whilst doing some sightseeing. He uses his mobile phone to run some other applications e.g. to keep in touch with his friends and receive some phone calls. Although he charged his mobile phone last night, his smart glasses were charged only two days ago. Luckily, the network knows that are the UEs are in close proximity with each other and can act as a group of cooperating UEs. Given that the UEs are close to each other, the measurements to be performed on wireless signals are expected to be quite similar. Therefore, in order to save energy, the network requests measurements from only one of the UEs. Using similar mechanisms, the signalling can be reduced for paging and during handover.

2.	Dave, after walking 2 Km, is getting a bit tired. When taking a break, he would like to share a video that he made with his smart glasses with his friend Luigi. However, the battery of his glasses is at a very low level and the video is quite big. However, the smart phone still has enough battery and the smart phone can cooperate in the uplink data transfer to increase reliability while reducing the energy consumption of the glasses. Also the base station would need to use less transmission power and perform less retransmissions because of the good quality connection with the smart phone.

#### 5.8.x.4	Post-conditions

Our user Dave enjoys high quality of service, and Dave’s devices have a longer battery life so that he can access the 6G network services for a longer period of time without recharging his devices. The network (e.g., base stations) also benefits energy-wise of the device cooperation since signalling, e.g., measurements are reduced.

#### 5.8.x.5	Existing features partly or fully covering the use case functionality

TS 22.261 [14] has several requirements related to optimizing performance for multiple UEs, however, these requirements are more related to broadcasting downlink data and relaying, not related to cooperation/coordination between UEs for energy savings.

For example, the following requirements have been specified in clauses 6.2.2, 6.4.2.2, 6.9.2, 6.40.2.1, 6.43.2 of TS 22.261 [14]::

The 5G network shall allow operators to optimize network behaviour (e.g. mobility management support) based on the mobility patterns (e.g. stationary, nomadic, spatially restricted mobility, full mobility) of a UE or group of UEs.

…

The 5G network shall optimize the resource use of the control plane and/or user plane to support bulk operation for high connection density (e.g. 1 million connections per square kilometre) of multiple UEs.

The 5G system shall support a timely, efficient, and/or reliable mechanism to transmit the same information to multiple UEs.

…

The 5G system shall support the relaying of traffic between a remote UE and a gNB using one or more relay UEs.

The 5G system shall support same traffic flow of a remote UE to be relayed via different indirect network connection paths.

The 5G system shall support different traffic flows of a remote UE to be relayed via different indirect network connection paths.

…

The 5G system shall be able to expose aggregated QoS parameter values for a group of UEs to an authorized service provider.

…

The 5G system shall enable an authorized 3rd party to provide policy(ies) for flows associated with an application. The policy may contain e.g. the set of UEs and data flows, the expected QoS handling and associated triggering events, other coordination information.

The 5G system shall support a means to apply 3rd party provided policy(ies) for flows associated with an application. The policy may contain e.g. the set of UEs and data flows, the expected QoS handling and associated triggering events, other coordination information.

NOTE:	The policy can be used by a 3rd party application for coordination of the transmission of multiple UEs’ flows (e.g., haptic, audio and video) of a multi-modal communication session.

Furthermore, as specified in TS 38.300 [221] clause 16.21, a non-3GPP connection may exists between UEs in case of an indirect connection path, and the presence of such non-3GPP connection is conveyed to the base station. This implies that the the base station is aware that the two devices are in close proximity of each other without having to rely on 3GPP location or ranging services.

#### 5.8.x.6	Potential New Requirements needed to support the use case

[PR 5.8.x-1] The 6G system shall support network energy savings by coordinating collaboration between multiple UEs.

[PR 5.8.x-2] The 6G system shall support UE energy savings by coordinating collaboration between multiple UEs.

* * * End of Change * * * *