3GPP TSG-SA WG1 Meeting #111 	S1-253348

25-29 August 2025, Goteborg, Sweden

Source:	Philips International B.V.

New use case title:	New use case on AI agent assisted backward compatibility enhancement

Draft TS/TR:	3GPP TR 22.870

Agenda item:	8.1.3

Document for:	Approval

Contact:	Walter Dees (walter.dees@philips.com)

Abstract: This pCR proposes a new use case for clause 6.x AI for 6G in TR 22.870

1. Introduction

Clause 6.x in TR 22.870 contains several use cases related to Artificial Intelligence (AI) indicating potential new requirement for 6G. This contribution proposes a new use case on AI agent assisted backward compatibility enhancement.

2. Reason for Change

Current backward compatibility mechanisms in 3GPP specifications primarily rely on the principle that a UE ignores unsupported parameters, and while this approach ensures interoperability across releases, it leads to under-utilization of device capabilities and may result in suboptimal performance. Additionally, specification changes introduced long after a release of the specifications is frozen could result in behavioural inconsistencies or missed opportunities for optimization.

Integrating an AI agent within the UE can enable interpretation of unsupported parameters and adaptive behaviour correction, thereby bridging the gap between different specification release versions and between software/firmware version upgrades. This approach enhances backward compatibility while leveraging existing UE capabilities more effectively

3. Proposal

Include a use case in Clause 6.x of TR 22.870 describing the integration of a backward compatibility enhancing AI agent.

* * * First Change * * * *

## 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or nonspecific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[X]		 3GPP TS 24.554 “Proximity-services (ProSe) in 5G System (5GS) protocol aspects”

[Y]		 3GPP TS 24.501 “Non-Access-Stratum (NAS) protocol for 5G System (5GS)”

* * * Next Change * * * *

---------- Use Case template ----------

## 6.x	Use case on backward compatibility enhancing AI agent

### 6.x.1	Description

In a 6G network, a device such as a User Equipment (UE) may implement an initial version of a 6G feature (e.g., from the initial release of 6G), while the network, or a peer UE, may implement a more recent version of the same communication feature, as specified in a subsequent release, which may result in additional parameters signalled by the network or the peer UE. Considering the supported release by the UE, the additional parameters communicated by the network, or the peer UE, may be ignored by the UE as the UE cannot process them.

For instance, consider a 5G example in which UE-1 and UE-2 both support 5G ProSe direct discovery (5G ProSe-dd) from Release-17 and Release-18, respectively. UE-2 may include in the PC5 discovery message for restricted 5G ProSe direct discovery solicitation the optional field: “Discoveree user info” (i.e., the target UE’s Application Layer ID, as per Table 10.2.1.3 of [X] v18.0.0). However, UE-1 will ignore the additional parameter as it is not specified in Release-17. This results in UE-1 potentially responding to the solicitation message based only on the matching of the discovery query filter(s) (see clause 6.2.14.2.2.4 in [X] v17.9.1), leading to unnecessary messages to be sent and/or the wrong UE to be selected.

Another shortcoming that devices may run into concerns changes to specifications long after they have been frozen. A specific example refers to changes reflected through Category-F and/or mirrored CRs (e.g., CRs 6659 (Rel-17), 6660 (Rel-18), and 6661 (Rel-19) implemented in TS 24.501 [Y], concerning lost text on REGISTRATION COMPLETE message). Additionally, capabilities supported by a UE may enable it to exhibit behaviours associated with a feature that is introduced in a subsequent release, for instance limiting access through a PLMN-RAT combination as requested by the network may be adhered to by the UE using its supported capabilities and may not necessarily be limited to UEs supporting Access technology utilization control (ATUC) capability (as specified in Rel-19 [Y]).

Whether it is additional signalling parameters, or specification changes that are not reflected by the UE implementation yet, the UE may still support some logic (i.e., may be functionally capable) to make use of the additional parameters or exhibit/omit a certain behaviour. However, as the UE is configured to ignore additional parameters according to the current backward compatibility mechanism, or due to UE software/firmware upgrades requiring a longer time to be provisioned, the UE may be under-utilizing its capabilities/features or exhibiting unwanted behaviours. Thus, to address these issues, the UE may integrate an Artificial Intelligence (AI) agent designed to enhance backward compatibility. This AI agent may be tasked with processing and interpreting unsupported parameters (e.g., Information Elements) in control messages or with assisting the UE to exhibit or correct a certain behaviour, hence bridging the gap between the initial and the subsequent releases of 6G and between UE software/firmware version upgrades.

### 6.x.2	Pre-conditions

-	Rob buys the first 6G UE that hits the market and operates with the initial release of 6G.

-	Two years later, Rob still has his phone, and he is travelling to Technology country. In technology country, the local operator runs the latest 6G release, using additional parameters (e.g., information elements) in control message.

-	Rob’s UE has an integrated backward compatibility enhancing AI agent.

### 6.x.3	Service Flows

- Rob’s UE sends a request to register to the network and indicates in the request its list of capabilities indicating support for feature-A, and support for a backward compatibility enhancing AI agent.

- The network in Technology country determines that Rob’s UE supports feature-A and the backward compatibility enhancing AI agent based on the indicated capabilities and is aware of a critical change associated with feature A and impacting UE behaviour (e.g., UE should (or should not) exhibit a certain behaviour-X).

- When a control message is received by Rob’s UE, Rob’s UE detects the unknown new additional parameter(s) (e.g., typically ignored) and feeds the control message to the on-device backward compatibility enhancing AI agent which processes the control message, including the additional parameter(s).

- The AI agent determines based on the additional parameters that Rob’s UE should exhibit behaviour-X (e.g., send a message M to the network) when a set of conditions is met. The AI agent determines whether the set of conditions are met, whether it is authorized to cause the UE to exhibit the requested behaviour-X (i.e., send message M to the network), and whether Rob’s UE is capable to exhibiting behaviour-X.

- Upon successfully verifying the AI agent’s authorization, Rob’s UE exhibits the requested behaviour. Otherwise, Rob’s UE falls back to the current backward compatibility mechanism.

- Rob’s UE and the network continue their operations seamlessly.

### 6.x.4	Post-conditions

- Rob’s UE makes use of an enhanced backward compatibility mechanism bridging the gap between its 6G supported features and their updated versions introduced in subsequent 6G releases. Rob’s UE continues to operate seamlessly with the network.

- The network in Technology country maintains optimal performance and avoids wasted resources, even when interacting with UEs operating with older releases of 6G.

### 6.x.5	Existing features partly or fully covering the use case functionality

None.

### 6.x.6	Potential New Requirements needed to support the use case

[PR 6.x.6-1] Based on operator policy, the 6G system shall provide support for the integration of an on-device backward compatibility enhancing AI agent to overcome limitations and/or impose restrictions of UE behaviour based on 6G specification updates.

[PR 6.x.6-2] Based on operator policy and user consent, the 6G system shall support mechanism(s) for updating an on-device backward compatibility enhancing AI agent’s resources to adapt to critical changes reflected in 6G specification updates.

[PR 6.x.6-3] Based on operator policy, the 6G system shall support mechanisms for controlling, monitoring and evaluating the performance of the on-device backwards compatibility enhancing AI agent.

[PR 6.x.6-4] The 6G system shall support a fallback mechanism from the backwards compatibility enhancing AI agent to the legacy backward compatibility mechanism.

* * * End of Change * * * *