3GPP TSG SA WG 1 Meeting #111 	S1-253349

25-29 August 2025, Goteborg, Sweden

Source:	Philips International B.V.

pCR Title:	New use case on sensing during emergency call

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.4

Document for:	Approval

Contact:	Walter Dees (walter.dees@philips.com)

Abstract: This pCR proposes a new use case for clause 7.x Integrated Sensing and Communication for 6G in TR 22.870.

1. Introduction

Clause 7.x in TR 22.870 contains several use cases related to Integrated Sensing and Communication indicating potential new requirements for 6G. This contribution proposes a new use case on integrating wireless sensing with emergency communication.

2. Reason for Change

In many countries, when a UE is used to make an emergency call, it is required to be able to determine the location of the UE that is used to makes the emergency call, in order to better assist first responders. A good example of this are the requirements for 911 calls, for which the location accuracy requirements keep improving as the technologies for determining location are progressing, see for example [X].

It is envisioned that wireless sensing can further improve assistance to first responders. If an accident or other emergency takes place in an area in which wireless sensing is available, sensing results can be used to better understand the context and circumstances around the emergency and aid first responders. They can also be used to further improve location accuracy.

3. Proposal

Include a use case in Clause 7.x of TR 22.870 describing the integration of wireless sensing with emergency communication.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or nonspecific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[X]		https://www.federalregister.gov/documents/2025/05/07/2025-06865/wireless-e911-location-accuracy-requirements

* * * Next Change * * * *

---------- Use Case template ----------

## 7.x 	Use case on integrating sensing with emergency calls

#### 7.x.1			Description

In many countries, when a UE is used to make an emergency call, it is required to be able to determine the location of the UE that is used to make the emergency call is required, in order to better assist first responders. A good example of this are the requirements for 911 calls, for which the location accuracy requirements keep improving as the technologies for determining location are progressing, see for example [X].

It is envisioned that wireless sensing can further improve assistance to first responders. If an accident or other emergency takes place in an area in which wireless sensing is available, sensing results can be used to better understand the context and circumstances around the emergency and aid first responders. They can also be used to further improve location accuracy.

#### 7.x.2 	Pre-conditions

Operator A offers integrated sensing and communication services in Bay City.

Alex’s mobile phone supports RF based sensing features as well.

#### 7.x.3 	Service Flows

1.	Alex is driving his car in Bay City. He is in a hurry and is distracted. He crashes into a big tree which comes tumbling down hurting several bystanders and blocking the road.

2.	Alex is severely hurt but is able to make an emergency call. Alex’s emergency call is received by the 911 dispatch center. He can hardly speak so Alex is not able to explain where the accident took place.

3. 	Operator A offers not only location services, but also integrated sensing and communication services, which were automatically triggered when Alex issued his emergency call. Network infrastructure performs RF sensing in cooperation with Alex’s mobile phone to determine obstacles and the location of Alex. Based on the combined location and sensing results provided to the 911 dispatch center, they are able to accurately pinpoint the place of the accident and an ambulance and police car are given the location. Alex’s phone performs RF sensing, and transmits sensing measurements including heart rate and respiration rate as part of the emergency call.

4. 	Since it was close to a park, several bystanders are gathering around the accident, further blocking the road. Operator A does not only provide the location information to the 911 dispatch center, but is also able to provide sensing results of the area surrounding the accident. This shows a crowd starting to gather and blocking the road. The dispatch center is able to tell this to the ambulance and police car so that they can approach the accident from the right street.

#### 7.x.4 	Post-conditions

The first responders are able to determine the condition of Alex based on the emergency call, find Alex very quickly  using the combined sensing and location results, and assess the situation so that they approach the emergency situation from the right street.

#### 7.x.5 	Existing features partly or fully covering the use case functionality

TS 22.137 [6] summarizes a number of requirements for sensing for the 5G system. However, none of them relates to integrating sensing with emergency calls.

#### 7.x.6	Potential New Requirements needed to support the use case

[PR 7.x-1] Subject to regulatory requirements and operator policy, the 6G system shall support a means to trigger sensing of an area surrounding the location of an emergency call if the location is within a sensing service area.

[PR 7.x-2] Subject to regulatory requirements and operator policy, the 6G system shall support a means to correlate the location of the UE and the sensing results and provide the combined location and sensing information to an emergency response center.

[PR 7.x-3] Subject to regulatory requirements, operator policy, and user consent, the 6G system shall support a means to perform wireless sensing of a user during an emergency call, and provide sensing information to an emergency response center as part of the emergency call.

* * * End of Change * * * *