3GPP TSG-SA WG1 Meeting #111 	S1-253354

Gothenburg, Sweden, 25-29 August 2025	(revision of S1-25xxxx)

Source:	Xiaomi

pCR Title:	pCR for UC 7.20 regarding non-sensing UE as consumer of sensing results

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.4

Document for:	Approval

Contact:	Gordon Young (gordonpetery@xiaomi.com)

Abstract: In this document, we address concerns over use case 7.20 “autonomous driving based on network-assisted sensing”, as captured in TR22.870v0.3.1

At SA1#110, new use case 7.20 introduced a new requirement whereby the sensing service result is delivered directly to the “UE or an application on the UE” (e.g. in vehicle), for the UE application to locally process the received sensing result. In the scenario pre-conditions 7.20.2 it is suggested this delivered sensing result will be combined with the autonomous driving service of company X, to which Alice is subscribed to enable enhanced functionality only for this UE or UE application (vehicle), “Alice’s vehicle supports Autonomous Driving based on Network-assisted Sensing”.

It is also stated that company X’s subscriber is allowed to use Operator A’ ISAC service. However no dependency or cross service subscription check is referred to when capturing the current requirement as per 7.20.6. It is assumed for the remainder of this paper that no check is necessary and indeed no additional service subscription is needed for the reception of sensing results from the 6G network. Further clarification from the proponents may be useful in this regard.

In order to consume sensing results requires registration to the 6G system for sensing services and being recognized as a trusted third party. It is indicated that in this use case the UE is not supporting 3GPP sensing in as much it is not contributing to sensing service delivery through provision of sensing information to the 6GS sensing service. It is noted that even a vehicle without 3GPP sensing capabilities may be supported in a 6G sensing service, through support of non-3gpp sensors.

Additionally in order to determine the sensing service and hence sensing results required the UE will need also to indicate parameters for the sensing service configuration that it desires to receive, such as target sensing area, and other required performance configuration including reporting rate.

What is not covered in the existing requirement is whether the UE is connected or registered to the network delivering the sensing service, however it is the understanding that as no other 3rd party server is involved that this scenario is indeed only applicable for the UE connected to the 6G network delivering the sensing results, via the sensing service. In one instance the UE is connected to the network for reception of sensing results derived from the networks sensing service.

The existing requirement in TR22.870 indicates “UE or an application on the UE” can obtain the sensing result. Since in this scenario the UE does not support 3GPP sensing service and is only registering as a service consumer (as a 3rd party service may do), it is unclear whether in fact the UE (as per 3GPP definition) is able to consume the sensing results. As such one understanding is that the UE will support an implementation wherein an application server and application client type solution will be implemented locally for the registration for sensing service delivery and the subsequent processing and combining with the service offered by company X. As such the requirement to the UE should be removed in recognition that the UE supports an application for acting as a consumer of a sensing service.

Additionally since the service results delivery is based on successful registration and authorisation and connection in order to receive the results there is no need for user consent when registering for this sensing consumer service.

Also the 6G network needs to be able to charge for delivery of the sensing service, since this is to be delivered direct from the 6G network new charging functionality is needed.

### 7.20.5	Existing features partly or fully covering the use case functionality

Use Case on Sensing Assisted Automotive Maneuvering and Navigation described in clause 5.8 of TR 22.837 [9] includes a similar scenario; however, the sensing result is sent to third party application, instead of the UE. This is captured in TS22.137 e.g.

Subject to operator’s policy, the 5G network shall be able to provide secure means to report sensing result to a trusted third-party requesting information about a target object when specific requested conditions are met.

NOTE: These conditions could be e.g., the target object distance from the restricted area border or entering restricted area.

Considering that the vehicle capable of Autonomous Driving usually supports sensing result calculation and with the combination of both 3GPP sensing results and non-3GPP sensing data, the performance can be significantly improved, it is better that the 3GPP sensing results is provided to the UE directly.

There are also several additional benefits of a UE directly obtaining 3GPP sensing results from the 3GPP network compared to acquiring it from third party application server:

Reduced Latency: When the UE obtains sensing data directly from the 3GPP network, it eliminates the additional step of the application server retrieving the data first. This direct communication reduces latency, which is crucial for real-time applications such as autonomous driving and advanced driver-assistance systems (ADAS).

Improved Reliability: Directly obtaining data from the 3GPP network reduces the dependency on the application server's availability and performance. This can enhance the reliability of the sensing data, ensuring that the UE receives timely and accurate information.

Enhanced Security: Direct communication between the UE and the 3GPP network can provide better security measures, as it minimizes the number of intermediaries involved in the data transmission. This reduces the risk of data breaches or tampering.

Consistency and Accuracy: Directly obtaining sensing data from the 3GPP network ensures that the UE receives the most up-to-date and accurate information. This consistency is vital for applications that require precise and real-time data.

### 7.20.6	Potential New Requirements needed to support the use case

[PR 7.20.6-1] Subject to regulation and operator policy, the 6G system shall be able to provide a mechanism for a network operator to authorize a UE supporting an application to obtain sensing result as a consumer for a specific service as a trusted third party.

NOTE:	The application supported on the UE is authorized in accordance with the authorization process of a 3rd party service for the delivery of sensing services. The routing and connection for registration, authorization, and sensing result delivery are FFS.

[PR 7.20.6-2] The UE supporting an application for the reception of sensing results as a consumer of the sensing service from the 6G network, is connected to the network delivering the sensing results.

[PR 7.20.6-3] The 6G system shall be able to support charging for the delivery of sensing results to a UE supporting an application receiving sensing results as a consumer for a specific service.