3GPP TSG-SA WG1 Meeting #111 	S1-253359

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:	Qualcomm Incorporated

New use case title:	Use case on joint QoS handling for 6G Service

Draft TS/TR:	3GPP TR 22.870.031

Agenda item:	8.1.2

Document for:	Approval

Contact:	Francesco Pica<fpica@qti.qualcomm.com>

Abstract: This contribution proposes a new use case which focuses on enhancing QoS handling to support distributed compute in the 6G system.

* * * First Change * * * *

### 9.x.1	Description

Mobile devices, including wearables, XR devices, laptops, smartphones, cameras, or other IoT devices, are increasingly incorporating computing capabilities. These devices often run compute-intensive applications such as Large Language Models (LLM), scene understanding, graphics rendering, avatar generation.

While these devices’ capabilities are getting better to support such applications, it is inevitable that some devices will be much more powerful than others. For example, a laptop’s computing capability is more powerful than wearables. Therefore, there is the need for distributed computing operation, where UE application clients can distribute their computing workloads (applications) to other UEs, edge servers, and/or cloud servers available within their network. For example, for an LLM workload, there could be different compute service modes: compute locally on device, compute remotely on edge server, or compute remotely on cloud server. The 6G network can assist distributed compute for client applications via richer compute discovery and dynamic selection of compute serving modes to balance power vs user experience.

Distributed compute for workloads such as AI inference, XR, and AR virtual assistance often involve multiple simultaneous data flows with variety of traffic types. For example, compute for an XR application may involve offloading 3D reconstruction (3DR), hand tracking, and rendering tasks (as shown in Fig. x):

- Uplink input stream: head tracking information, hand tracking information, camera images.

- Downlink output stream: Rendered video, and 3D mesh.

Fig. x:  XR application – example of compute related service flows

In another example, Image/Video Generation via LLMs can include:

- Uplink input stream: Text Prompt

- Downlink output stream: Generated image/video, and audio summary

In the above examples, there could be different groups of flows, where e.g.:

- Each flow may be uplink or downlink.

- Each flow may correspond to different traffic type, e.g., text, audio, image, video, etc.

- Each flow can have different QoS, e.g., data rate, requirements.

QoS requirements for multiple flows can be interdependent, i.e., satisfying QoS for one flow (or some flows) only can be insufficient to meet application-level requirements. Furthermore, there can be multiple combinations of QoS profiles, for different groups of data flows, that may be acceptable for the application.

To address those QoS challenges, there should be the ability to provide a UE indication or preference, to the network, to assist managing groups of flows associated to a service, e.g. ensuring that all flows in the group are admitted simultaneously and corresponding resources allocated based on joint QoS requirements. Furthermore, the UE could also provide alternative groups of data flows with associated QoS profiles, and the network can dynamically select the optimal group / QoS based on requirements and resources availability.

### 9.x.2	Existing features partly or fully covering the use case functionality

### Existing requirements in TS 22.261:

The 5G system shall enable an authorized 3rd party to provide policy(ies) for flows associated with an application. The policy may contain e.g. the set of UEs and data flows, the expected QoS handling and associated triggering events, other coordination information.

The 5G system shall support a means to apply 3rd party provided policy(ies) for flows associated with an application. The policy may contain e.g. the set of UEs and data flows, the expected QoS handling and associated triggering events, other coordination information.

NOTE:    The policy can be used by a 3rd party application for coordination of the transmission of multiple UEs’ flows (e.g., haptic, audio and video) of a multi-modal communication session.

### 9.x.3	Potential New Requirements needed to support the use case

[PR 9.x.6-1] Based on operator policy, the 6G system shall enable a UE to indicate preference, to the network, on QoS handling of a group of service data flows associated with an application e.g. for joint admittance and resource allocation by the network.

[PR 9.x.6-2] Based on operator policy, the 6G system shall enable a UE to indicate, to the network, alternative groups of service data flows associated with an application, with different QoS requirements for each group, so that the network can dynamically select an optimal group and QoS, e.g. based on network resource availability.

* * * End of  Change * * * *