3GPP TSG-SA WG1 Meeting #111 	S1-253362

25-29 August 2025, Goteborg, Sweden

Source:	Turkcell

New use case title:	New use case on Multi-MNO Cooperative Service Continuity in Disaster Scenarios

Draft TS/TR:	3GPP TR 22.870

Agenda item:	8.1.3

Document for:	Approval

Contact:	Mehmet Basaran, mehmet.basaran@turkcell.com.tr

Abstract: This contribution proposes a new use case on multi-MNO cooperative service continuity in disaster scenarios.

* * * First Change, all new text * * * *

## 6.X	Use case on multi-MNO cooperative service continuity in disaster scenarios

## 6.x.1	Description

This use case addresses the lack of autonomous, intelligent, and dynamic fallback coordination mechanisms across MNOs in disaster scenarios, aiming to overcome current limitations in static roaming, rigid policies, and manual provisioning through agentic AI-enabled cooperation.

In the event of a disaster (e.g. earthquake, flood, wildfire), critical network infrastructure (e.g., fibre-based backhaul) operated by one Mobile Network Operator (MNO-A) may be damaged, resulting in lower network capacity, or become unavailable in the affected area. Meanwhile, another operator (MNO-B) may continue to operate infrastructure (e.g., microwave-based backhaul) in the same geographical region. For example, the fibre-based backhaul network of MNO-A may be damaged and become unavailable while the microwave-based backhaul network of MNO-B may still be available in the same geographical area.

This use case addresses the scenario where MNO-B temporarily provides service to MNO-A’s subscribers for essential and emergency services, such as communication with emergency responders, alarm messages, or location updates and go beyond traditional disaster roaming which deals mainly with pre-established and static policies.

The coordination between the two MNOs is performed autonomously by intelligent network agents embedded within an AI-native agentic architecture that are needed for real-time, autonomous, and context-aware coordination between PLMNs in highly dynamic disaster scenarios. These agents are capable of assessing the situation, exchanging information securely, and negotiating temporary network resource sharing agreements in real-time.

While high-level mutual disaster roaming agreements remain necessary, the use of inter-PLMN AI agents enables real-time negotiation, configuration, and policy enforcement reducing reliance on static provisioning and manual coordination when the disaster occurs.

This scenario in the disaster area is illustrated in figure below:

Figure 6.x.1: Inter-MNO cooperative service continuity via agent-based coordination in disaster scenario

It is supposed that 6G system introduces autonomous agents capable of real-time negotiation, decision-making, and inter-operator coordination. In particular, in this use case:

- MNO-A’s agent detects service degradation or network disruption- and signals for disaster-mode fallback.

- MNO-B’s agent receives the request, evaluates current network load and policies, and dynamically allocates resources.

- Both agents coordinate securely and automatically using predefined or AI-learned policies, leveraging protocols for inter-domain decisions and 3GPP inter-PLMN interfaces.

The AI agents ensure trustworthy, auditable (i.e. traceable behaviour), and context-aware resource allocation and user admission, minimizing manual intervention. This also supports service continuity for autonomous systems (e.g., AMRs, UAVs) involved in rescue operations, which may rely on real-time low-latency connectivity.

This use case is critical for enhancing communication resilience. Ensuring basic connectivity (e.g., emergency calls, alerts, and location tracking) can save lives during the initial hours following a disaster. It also ensures communication among first responders, civil protection teams, and autonomous systems (like AMRs or UAVs) that rely on mobile infrastructure. While national roaming agreements exist in some countries, current deployments are often static, manually controlled, and not optimized for rapid autonomous disaster response.

### 6.x.2	Pre-conditions

- A disaster has caused partial or full disruption of MNO-A’s RAN or core network in the affected region.

- MNO-B has active and available network (e.g., base stations, core network, edge computing) in the same geographical area.

- Subscribers of MNO-A are physically located within the coverage area of MNO-B.

- Network agents (as part of an agentic AI-based infrastructure) are deployed and authorized by both MNOs to support inter-PLMN coordination under disaster scenarios. These agents complement existing roaming mechanisms by negotiating roaming authorization and access based not only on subscription and roaming agreements, but also real-time context such as disaster conditions, network availability, and policy constraints to enable more dynamic and adaptive roaming decision-making and supporting faster service continuity for affected UEs.

- Policy frameworks exist (predefined or AI-learned) to guide acceptable roaming or resource sharing terms in emergency cases.

### 6.x.3	Service Flows

1. Detection: MNO-A’s network agent detects service degradation or network disruption in a specific region (e.g., via telemetry loss, traffic analysis, or user reports).

2. Assessment: MNO-A’s agent classifies the situation as a disaster event and enters emergency fallback mode.

3. Discovery: MNO-A’s agent identifies neighbouring MNOs with viable coverage (e.g., MNO-B). This is achieved through existing broadcast PLMN identifiers, known inter-PLMN relationships, and roaming capability information. The agent determines which neighbouring MNOs are potentially able to provide fallback connectivity to the affected subscribers of MNO-A.

4. Negotiation: the agents of MNO-A and MNO-B communicate securely to initiate context-aware negotiation for temporary service provisioning, including, e.g.:

- Authentication policies for roaming,

- Resource availability,

- QoS limits,

- Traffic prioritization,

- Duration and area of service support.

The agents coordinate these parameters autonomously and securely, reducing reliance on static configurations and enabling faster service continuity under extreme conditions.

5. Provisioning: MNO-B’s agent activates network resource and capabilities to support temporary access for MNO-A subscribers under disaster conditions to provide fallback connectivity based on the outcome of inter-agent negotiation.

6. Connection: affected UEs of MNO-A attach via MNO-B’s infrastructure and gain access to predefined emergency services.

7. Supervision: both agents coordinate to supervise the ongoing registration status, session continuity, and policy adherence for disaster-roaming UEs. This includes verifying that UEs remain authorized, service conditions are maintained, and that the roaming session can be sustained or handed back to the home network once available.

### 6.x.4	Post-conditions

- MNO-A subscribers in the affected area are temporarily connected via MNO-B’s network infrastructure and have access to limited emergency services.

- Service is maintained until MNO-A restores infrastructure or the disaster scenario ends.

- Inter-PLMN agent coordination results in finalized policy decisions and access outcomes that are traceable and compliant with operator-defined trust framework.

### 6.x.5	Existing features partly or fully covering the use case functionality

- 3GPP TS 23.501 / TS 23.502: support for inter-PLMN roaming and user equipment registration in visited PLMNs.

- 3GPP TS 24.193: Access Traffic Steering, Switching and Splitting (ATSSS): support for multi-access path management.

- GSMA NG.126: Support for national roaming and network sharing for disaster response (mostly static or policy-based).

### 6.x.6	Potential New Requirements needed to support the use case

[PR 6.x.6-1] The 6G system should support for real-time inter-PLMN agent negotiation of service level agreements under disaster-triggered fallback scenarios.

[PR 6.x.6-2] The 6G system should provide capability for AI-native agent coordination to trigger, monitor, and terminate shared network resources or roaming autonomously without human intervention.

[PR 6.x.6-3] The 6G system should provide policy frameworks that enable inter-PLMN cooperation during emergency fallback scenarios, including mechanisms for autonomous trust negotiation, access authorization, usage restrictions, and prioritization of emergency services.

[PR 6.x.6-4] The 6G system should support secure information exchange and traceability between network functions or agents involved in inter-PLMN service continuity, including mechanisms for validating decision correctness and enabling auditability in dynamic cooperation scenarios (e.g., disaster recovery).

[PR 6.x.6-5] The 6G system should support inter-PLMN coordination to enable service continuity using network resources (e.g., dynamic slice instantiation, network function exposure) from different operators, including dynamic establishment of required network capabilities.

[PR 6.x.6-6] The 6G system should support mechanisms to manage UE registration and security context transfer dynamically in disaster fallback scenarios.

[PR 6.x.6-7] The 6G system should enable dynamic enforcement of pre-agreed disaster policies through inter-PLMN AI agent coordination, reducing the need for per-event manual configuration.

* * * End of Change * * * *