3GPP TSG-SA WG1 Meeting #111 	S1- 253418

25-29 August 2025, Goteborg, Sweden 	(revision of S1-253202r2)

Source:	China Unicom, Huawei

New use case title:	New Use case on resolving IMS media related service conflicts

Draft TS/TR:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Qun, Wei, weiqun5@chinaunicom.cn

Abstract: This document provides a new use case “IMS Media Related Service”.

1. Introduction

Multiple media related services have been introduced in Rel-18 and Rel-19, i.e., IMS data channel, AR media processing, avatar media processing. In addition, IMS traditionally offers various media capabilities, e.g. play annoucement, DTMF Collection, etc. Those media related services have different control points and use different network resources.

2. Reason for Change

This CR propose the IMS media related service as a new use case to ensure the quality of IMS media related service by avoiding potential service conflicts caused by invoking multiple media related services in the same session.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change (New use case)* * * *

5.7.x	IMS Media Related Service

5.7.x.1	Description

Multiple media related services have been introduced in 5G. For instance, as defined in TS 23.228 [142], IMS data channel, AR communication, avatar communication etc. On the other hand, IMS network offers traditional media capabilities, e.g., play announcement, DTMF Collection, etc. Those media related services may cause potential conflicts for the following reasons:

1)	Media related services have different control point, e.g., play announcement controlled by IMS AS, data channel controlled by DCSF, avatar communication controlled by DC AS.

2)	Media related services are handled in different NFs, e.g., play announcement handled by MRF, IMS data channel handled by MF.

3)	Media related services have similar triggering time, e.g., play announcement and data channel can be triggered anytime during the session, AR capabilities triggered after the session establishment.

5.7.x.2	Pre-conditions

George, as a subscriber from MNO A, subscribes to IMS data channel service and avatar communication service. IMS data channel service allows George to establish IMS data channel and use data channel application. Avatar communication service allows George to use his own avatar during the video call.

CleanClinic, a healthcare company offering video consultants, subscribes to play announcement service and IMS data channel service from MNO B. Play annoucnment service allows the CleanClinic to play certain announcement when the patient is waiting for his/her physician. IMS data channel service allows physicians to establish IMS data channel and use data channel application to allow patients to share pictures of symptoms.

5.7.x.3	Service Flows

1.	George initiates a call session with audio/video and IMS data channel to CleanClinic. While waiting for his physician, CleanClinic plays some music to George via play announcement service. The IMS data channel service from both MNO A and MNO B, and avatar communication service from MNO A need to be paused during the waiting period.

2.	When the physician picks up the call, the voice needs to re-route, and IMS data channel service from both MNO A and MNO B, and avatar communication service from MNO A need to be resumed.

5.7.x.4	Post-conditions

George hangs up the call after consulting with his physicians.

5.7.x.5	Existing features partly or fully covering the use case functionality

Editor's Note: The gap analys is FFS.

5.7.x.6	Potential New Requirements needed to support the use case

[PR.5.7.x.6-001] The IMS shall support means to ensure user experience when multiple IMS media related services are triggered within one call session simultaneously by one user or multiple users.

NOTE 1:	The IMS media related service can include supplementary services, IMS data channel based service, immersive communication service, etc.

Editor's Note: The requirement above is FFS.