3GPP TSG SA WG 1 Meeting #111 	S1-253460

25-29 August 2025, Goteborg, Sweden	(revision of S1-253242r1)

Source:	Huawei

pCR Title:	Pseudo-CR on Update to clause 11.1 “Use case on communication on board of UAM aircrafts”

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.8

Document for:	Approval

Contact:	Alice Li, alice.li1@huawei.com

Abstract: This document clarifies the target scenario for which a set of KPIs are defined for the use case in clause 11.1.

R1 is to address the comments received.

1. Introduction

KPIs are defined in clause 11.1.6 for the on-board communication use case in a UAM aircraft, covering both sensing and communitcaiton perspectives.

2. Reason for Change

The use case in clause 11.1 mentions the necessity of addressing passenger communication onboard UAM. Passengers onboard can access internet, use video conferencing or video chat, and enjoy immersive multimedia services such as cloud gaming. It is therefore necessary to capture these scenarios with the associated KPI requirements.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.3.1.

* * * First Change * * * *

## 11.1	Use case on communication on board of UAM aircrafts

### 11.1.1	Description

Urban air mobility (UAM) is a new safe, secure and more sustainable air transportation system for passengers and cargo in urban environments, enabled by new technologies and integrated into multimodal transportation systems. The transportation is performed by electric vertical take-off and landing (eVTOL) aircrafts, remotely piloted or with a pilot onboard [32]. In February 2024 an eVTOL named as "PROSPERITY" [33] that can contain 5 passengers conducted a Shenzhen-Zhuhai test flight, which was a cross-sea and cross-city route. KT showed, at the MWC2024, their UAM Skypath solution, which can provide 5G service for UAM aircrafts flying at 300 – 600 meters altitude.

Compared with the common UAVs (Uncrewed Aerial Vehicle), the UAM aircrafts have the following major differences:

- 	Large size and heavy weight, higher AGL (above ground level)

Low-altitude airspace usually refers to the airspace with a vertical distance of less than 1000 m from the ground, and it can be extended to less than 3000 m according to the characteristics and actual needs of different regions [34]. The AGL of UAM aircrafts can be up to 1000 m, while the AGL of small UAVs is less than 300m. Communication for the UAM aircrafts with higher AGL need to be considered.

- 	Higher reliability and safety requirement with human beings onboard

UAM aircrafts share some common low altitude airspace. As predicted by Professor Shen, there will be about 100,000 UAVs flying in Shenzhen's sky at the same time in the future [36]. Considering the area of Shenzhen is 1997 km2, there will be about 50 UAVs flying in 1 km2 airspace. To ensure high reliability and safety, the UAM aircrafts must be aware of the object information that are near its flight trajectory.

To guarantee the safety of aircrafts, Detect and Avoid (DAA) technology is widely used in UAVs by using a combination of sensors, cameras, and radar to continuously monitor the UAV's surroundings [38]. These sensors detect obstacles, other aircraft, and potential hazards in the flight path. The system then processes this information in real-time and adjusts the UAV's flight path to avoid collisions. However, the capabilities of the sensors on UAVs are not able to sense the blockage far away. Since the UAM aircrafts carry human beings on board, UAV's DAA system is not adequate to ensure the safety of passengers onboard. By also utilizing the 3GPP sensing service, the safety of UAM aircrafts flying in the common airspace can be guaranteed, as well as the running efficiency of UAM aircraft can be improved so as to transport more passengers or goods.

Typically, the message size for sensing one object could be 1 kbyte [37], including information of size/position/speed/direction. Around 25 objects (25 kbyte) per frame (20 ms) need to be sensed for an aircraft. The reliability requirement for UAM aircrafts is about 99.9 %.

In addition, with passengers onboard, human communication on board the UAM has to be considered. Passengers onboard can access internet, use video conferencing or video chat, and enjoy immersive multimedia services such as cloud gaming [11.1x].

* * * Second Change * * * *

### 11.1.6	Potential New Requirements needed to support the use case

[PR 11.1.6-1] The 6G System shall support the transmission of sensing result information to a UAM aircraft about its surroundings with the following KPI requirements.

Table 11.1.6-1: Performance requirements for transmission of sensing result information to a UAM aircraft

|  | Typical transmission interval | Altitude AGL | Typical message Size | End to end Latency | Reliability |
| --- | --- | --- | --- | --- | --- |
| Sensing result to a UAM aircraft | [20 ms] | up to 1000 m | [25 kbyte] | [20 ms] | [99.9 %] |
| NOTE: Typically, the message size for one object detected and/or tracked via sensing is 1 kbyte [37]. It is assumed that around 25 objects (25 kbyte) per frame (20 ms) are sensed surrounding an aircraft. The reliability requirement for UAM aircrafts is about 99.9 %. |  |  |  |  |  |

[PR 11.1.6-002] The 6G System shall support services provided to the UAM applications (for the passengers and devices onboard the aircraft) with the following KPI requirements.

Table 11.1.6-2: Performance requirements for UAM onboard aircraft services

| Services | Data rate | End to end Latency | Altitude AGL | Reliability (note 4) | Service area |
| --- | --- | --- | --- | --- | --- |
| 8K video live broadcast | 100 Mbps Traffic from the UAM aircraft (note 1) | 200 ms (note 1) | up to 1000 m | 95 % | Urban, scenic area |
|  | 600 kbps Traffic towards the UAM aircraft (note 1) | 20 ms (note 1) | up to 1000 m | 95 % |  |
| Video streaming | 4 Mbps for 720p video 9 Mbps for 1080p video Traffic from the UAM aircraft (note 1) | 100 ms (note 1) | up to 1000 m | 95 % | Urban, rural area |
|  | 100 Mbps for 8K video Traffic from the UAM aircraft (note 1) | 100 ms (note 1) | up to 1000 m | 95 % |  |
| Remote controller through HD video | >=25 Mbps Traffic from the UAM aircraft (note 1) | 100ms (note 1) | up to 1000 m | 99 % | Urban, rural area |
|  | 300 kbps Traffic towards the UAM aircraft (note 1) | 20 ms (note 1) | up to 1000 m | 99 % |  |
| Video conferencing or video chat | 25 Mbps Traffic from passengers onboard UAM aircraft (note 1) (note 2) | 100 ms (note 1) | up to 1000 m | 99 % | Urban, rural area |
|  | 25 Mbps Traffic towards passengers onboard UAM aircraft (note 1) (note 2) | 100 ms (note 1) | up to 1000 m | 99 % |  |
| Immersive multimedia service (e.g., cloud gaming) | 500 kbps Traffic from passengers onboard UAM aircraft (note 2) | 50 ms (note 3) | up to 1000 m | 99 % | Urban, rural area |
|  | 100~500 Mbps Traffic towards passengers onboard UAM aircraft (note 2) (note 5) | 50 ms (note 3) | up to 1000 m | 99 % |  |
| NOTE 1: These values are aligned with the KPIs for services provided to the UAV applications in TS 22.125 [35], Table 7.1-1. NOTE 2: The value is per passenger; and it is assumed that up to 4 passengers per UAM aircraft use communication services simultaneously. NOTE 3: According to TR 26.928 [50], typically a 50ms latency is a required for cloud gaming use case. NOTE 4: According to [11.1y], the reliability of real-time services is set to 99%, and that of video streaming is set to 95%. NOTE 5: Data rate is calculated assuming typical parameters (e.g. resolution, refresh rate and compression rate). Some codecs may further drop the bitrate requirement. |  |  |  |  |  |

* * * Third Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

…

[11.1x] “Urban Air Mobility Operational Concept (OpsCon) Passenger-Carrying Operations”, https://ntrs.nasa.gov/api/citations/20205001587/downloads/UAM%20Passenger-carrying%20OpsCon%20-%20v14%20GP%20accept.pdf

[11.1y] “Definition of Use Cases, Requirements and Reference Network Architecture”, https://www.season-project.eu/wp-content/uploads/Deliverables/Deliverable-2-1_SEASON_V2.1.pdf