3GPP TSG SA WG 1 Meeting #111 	S1-253166

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:	Qualcomm Technologies, Huawei

pCR Title:	Update to Use Case 7.6

Draft Spec:	3GPP TR 22.870 v031

Agenda item:	8.1.4

Document for:	Approval

Contact:	Lola Awoniyi-Oteri <oawoniyi@qti.qualcomm.com>

Abstract: Use case proposing updates to the Road Digitalization in clause 7.6 to align the numbers with RAN1 parameters.

1. Introduction

Align the assumptions and sensing performance requirements in clause 7.6 with RAN 1 analysis

2. Reason for Change

To align the assumptions and sensing performance requirements in clause 7.6 of TR 22870 with section 7.9.6 of TR 38.901 and section 6 of TR 37.885 for highway service area. Specifically, we would like to:

- revise the following values in Table 7.6.1-1 as follows:

- Vehicle size as 5m x 2m x 1.6m

- For highway service area, maximum vehicle velocity as 140km/h(=38.89m/s) with corresponding distance between two vehicles as 78m (= 38.89m/s*2s)

- revise the outdoor service area correspondig to highways in Table 7.6.6-1 as follows:

- sensing target velocity as “moving upto 140 km/h” and

- sensing target density as ≤72 (= 1000/(78+5)*6 vehicles per 1000m x 24m

By aligning the max velocity to a higher value of 140 km/h as per 37.885, we will be able to set a reasonable lower limit on sensing target density.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v031

* * * First Change * * * *

## 7.6	Use case on road digitalization

### 7.6.1	Description

Many transportation and urban applications require real-time and citywide traffic flow estimation, which is the basis for transportation planning and traffic control. Estimated traffic flow is generally represented by the number of cyclists/vehicles/pedestrians passing a reference location per unit of time and can be virtualized by a third-party application.

In order to enable real-time navigation for automatic driving or traffic flow monitoring, it is necessary to define the upper limit of sensing object detection/tracking required by a 3GPP sensing service, since any 3GPP based sensing detection/tracking coexists with communication services and is not cost-free for operators by sharing. 3GPP system shall allocate limited but sufficient spectrum resources, power, base stations and/or road side units and etc for sensing operations. Such kind of necessity can be represented by sensing target density.

For example, the number of vehicles that needs to be simultaneously detected and tracked at a crossroad may be up to 1000 cars per [km2], including all stationary and moving vehicles occupying that cross-road temporally.

The assumptions of this use case are described in the Table 7.6.1-1.

Figure 7.6.1-1: Sensing area

Table 7.6.1-1: Assumptions

| Sensing target | Size (Length x Width x Height) | Velocity | Distance between vehicles[96] | Service area (Note 1) |
| --- | --- | --- | --- | --- |
| Vehicle | 5m x 2m x 1.6m | Up to 5km/h(static/moving) | 3 m | Crossroad (note 1) |
|  |  | Up to 140 km/h | 78 m | Highway (note 2) |
| NOTE 1: One crossroad area is approximated by 1000 m dual three-lane carriageway (24 m), by assuming 250 m per direction at the crossroad. NOTE 2: Based on an assumption of dual three-lane carriageway (1000 m x 24 m). |  |  |  |  |

### 7.6.2	Pre-conditions

Good partnership and cooperation are established between Traffic Department A and Mobile Network Operator B. Traffic Department A subscribes to the 3GPP wireless sensing service from MNO B for the real-time road digitalization.

In order to monitor traffic volume constantly, MNO B has deployed and activated base stations and road side units around traffic intensive areas, such as crossroads, urban roads and highways, in order to provide a wide sensing coverage and capability to detect and track all moving objects, including vehicles, bicycles and pedestrians effectively.

NOTE:	For the ease of elaboration, the base station is acting as sensing transmitter and/or sensing receiver in this case. Other sensing modes can also be feasible and useful as well.

### 7.6.3	Service Flows

Figure 7.6.3-1:  Target density at crossroad

Figure 7.6.3-2:  Target density at highway

1.	Traffic Department A receives a lot of complaints from local residents about heavy traffic at crossroads and highways due to ongoing city construction or bad route planning.

2.	Traffic Department A requests MNO B to detect and track at least [100] sensing objects simultaneously including cyclists, pedestrians and vehicles at crossroads shown in Figure 7.6.3-1, and at least [50] sensing objects simultaneously including high-speed vehicles at overpasses shown in Figure 7.6.3-2. According to preferred sensing target density, base stations and road side units deployed by MNO B along these areas constantly monitor the road situation by appropriate sensing operations according to these requests.

3.	MNO B collects 3GPP sensing data from base stations and road side units, and non-3GPP sensing data from sensors (e.g. radar, cameras) if available. Then MNO B provides Traffic Department A with real-time traffic flow information, including vehicle volume of different directions within a unit time (e.g. every 10 minutes, every half hour), average queuing time for a vehicle/pedestrian as the crossroad, etc.

4.	For more demanding traffic monitoring, Traffic Department A requests MNO B to detect and track more objects simultaneously at given crossroads and overpasses during rush hours. MNO B adapts to the request by updating the configuration parameters of sensing operations.

5.	Based on sensing results exposed by MNO B for road digitalization, Traffic Department A figures out solutions to improve the utilization of public roads at real-time by finer spatial/temporal traffic control and traffic divergence.

### 7.6.4	Post-conditions

Thanks to the network-wide coverage, the bird's-eye-view of the base station, road digitalization is enabled by capturing real-time information of the road environment.

### 7.6.5	Existing features partly or fully covering the use cast functionality

TS 22.137 [6] has described object detection and tracking by a number of sensing service categories.  However, there is essential KPI missing from each category at given sensing accuracy and service latency. For example, Category 3 does not elaborate how many vehicles from a sensing service area would like to be detected and/or tracked by 6GS. On the other hand, sensing, data communication and other 6G services enabled for the same service area shall share limited 6G spectrum, power, and entities effectively. Therefore, this use case is proposed to clarify preferred sensing target densities, from sensing service perspective, for object detection and tracking within a few typical urban transport scenarios.

In summary, the capability to provide suitable mechanisms for a trusted third-party to request the sensing target density is not covered.

Table 7.6.5-1:  Title Needed; Notes in table are missing.

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 3 | 95 | 1 | 1 | 1 [3], [4] | 1 | 1 [3], [4] | 1 x 1 [3] | 100 [2], or 1000 (note 3); 5000 for detection in highway | 0.05 to 1 | 2 | 2 | Indoor/outdoor (e.g. detection and tracking of human, animal, UAV) |

### 7.6.6	Potential New Requirements needed to support the use case

[PR 7.6.6-1]	Subject to regulation and operator's policy, 6G system shall efficiently support sensing target density requested by the third party for a given sensing service.

[PR 7.6.6-2] The 6G system shall be able to support the following KPIs:

Table 7.6.6-1:  Performance requirements for road digitalization

| Scenario | Service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Sensing target density （note 3） | Sensing target Velocity | Max sensing service latency [s] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |  |
| Road digitalization | Outdoor | 95 | 0.5 (vehicle) 1（pedestrian） | N/A | 0.5 | N/A | 0.2 | 0.2 m/s | ≤ 750 vehicles Per 1000 m x 24 m ≤100 pedestrian per 1000 m x 24 m (note 1) | Static/moving, up to 5 km/h | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 |
|  | Outdoor | 95 | 3 | N/A | 3 | N/A | 1 | 1 m/s | ≤72 vehicles per 1000 m x 24 m (note 2) | Moving, up to 140 km/h | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 |
| NOTE 1: One crossroad area is approximated by 1000m dual three-lane carriageway (24 m), by assuming 250 m per direction at the crossroad. NOTE 2: Based on an assumption of dual three-lane carriageway (1000 m x 24 m). NOTE 3: The definition of sensing target density is described in Clause 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

.

* * * End of Change * * * *