3GPP TSG SA WG 1 Meeting #111 	S1-253580

25-29 August 2025, Goteborg, Sweden	(revision of S1-253150r1)

Source:	Orange

pCR Title:	Pseudo-CR TR 22.870 on 6.20 6G network providing on-demand networking with AI Agent

Draft Spec:	3GPP TR 22.870 v 0.3.1

Agenda item:	8.1.3

Document for:	Approval

Contact:	philippe.lottin ( @ ) orange.com

Abstract: The present contribution proposes, in the use case 6.20 on 6G network providing on-demand networking with AI Agent, Description section, to introduce services not connected with an AI agent (IoT, robots, etc.).

1. Introduction

During the last SA1#110 meeting, the new use case “6.20 Use case on 6G network providing on-demand networking with AI Agent” was introduce in the TR 22.870 Study on 6G Use Cases and Service Requirements; After further study, it appears that an enhancement proposal can be done in the Description section.

2. Reason for Change

The current proposal does not take into account certain scenarios that may also be considered.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v 0.3.1. in the Description section. No need to modify existing Potential New Requirements has been identified.

* * * First Change * * * *

## 6.20	Use case on 6G network providing on-demand networking with AI Agent

### 6.20.1	Description

ITU-R involves AI as a new key capability in 6G. It requires specialized use cases by leveraging data collection, local or distributed compute offload, and the distributed training and inference of AI models [27]. As an emerging technology, AI Agent technology can meet such requirements as an intelligence component in 6GS to understand user’s intent, split task into network task, combine and chaining network capability, monitoring task and report to user. Figure 6.20.1-1 depicts procedures of how AI Agent processes a user intent requirement:

Figure 6.20.1-1: AI-Agent based intent translation and task execution

-	Intent input: user provides intent to network to express user request, e.g. it could be in natural language “I want to watch a high-definition video”.

-	Intent translation: Network AI Agent receives intent requirements, translate it into network requirements.

-	Task distribution: Network AI Agent analyses which network functions should be involved to fulfill the task, such as session management related network functions for QoS level guarantee.

-	Task execution & interaction: Different network functions will be involved with different task. Private network functions components (private network for robots, IoTs, etc.) can also be involved.

-	Monitoring & QoS assurance: Real-time monitoring and feedback to make sure whether service is satisfying user intent or not.

* * * End of Change * * * *