3GPP TSG SA WG 1 Meeting #111 	S1- 253617

25-29 August 2025, Goteborg, Sweden	(revision of S1-252607,3105r1)

Source:	LG Electronics

New use case title:	Use case on supporting environmental awareness data management using collaborative service robots

Draft TS/TR:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.3 AI

Document for:	Approval

Contact:	Ki-Dong Lee (kidong.lee (at) lge.com), Hyunsook Kim, Myungjune Youn, HyunJung Choe

Abstract: As a follow-up action presented in S1-250049 at SA1#109, this contribution is intended to introduce considerations point identified with one of the use cases in FS_SOBOT TR (5.7 of TR 22.916) into TR 22.870.

1. Introduction

See below.

2. Reason for Change

See below.

Summary of Changes (in 3617):

- Deleted 001 (UAC, covered by 3409, which LGE co-signed)

- Removed MEC-related PR and descriptions

3. Conclusions

See below.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.3.1.

* * * First Change * * * *

## 6.x	Use case on supporting environmental awareness data management using collaborative service robots

### 6.x.1	Description

Environmental monitoring—especially geo-surface sensing—is a broad domain within remote sensing applications, as the geo-surface is one of the most information-rich zones in our daily lives. It involves collecting data and information, pre-processing them, collaboration between UEs and network entities (e.g., Multi-access Edge Computing (MEC) in the RAN or MEC in the core network of a 6G system), delivering data and/or information, transferring AI/ML models, sharing inference results, and more. It encompasses a wide range of use cases:

- Agricultural monitoring, such as detecting changes in the surface of farming fields (e.g., plant growth or health conditions) [r1];

- Wildlife preservation, including the detection and tracking of movements or behavioral changes in specific animal species of interest [r2];

- 3D architectural design and urban planning, where geo-surface sensing technologies support both new development and renovation efforts. These applications are enhanced through data collection, analysis, and prediction, often enabled by collaboration among multiple entities (e.g., autonomous robots capable of performing coordinated tasks in various environments), all of which require reliable communication.

- Fire spreads in urhan environment: Urban fire spreading requires vigilant monitoring because fires quickly propagate from building to building via close proximity of combustible materials, wind-blown embers, and heat radiation. Early detection and prediction rely on advanced systems such as thermal imaging, satellite earth observation, and computational fire spread models that analyze environmental factors like fuel moisture, vegetation, building materials, and weather conditions. These tools enable timely firefighting responses by forecasting fire dynamics and spread pathways, ultimately protecting lives and infrastructure in dense urban areas. Continuous sensing and model-based simulation are critical for understanding fire behavior in complex urban settings, supporting strategic suppression efforts before fires escalate beyond control. Satellite thermal data combined with AI-powered prediction systems help provide near real-time fire alerts, even under challenging visibility conditions, making urban fire monitoring more effective and proactive. [r5]

Additional applications continue to emerge across various domains. These use cases typically generate large volumes of measurement data, necessitating a certain level of pre-processing and efficient sharing among participating agents for effective data fusion. Optimizing the quality of data fusion outcomes is a key objective, and simultaneously, the efficiency of the processes involved—such as minimizing energy and time consumption—is critically important.

In alignment with the United Nations’ Sustainable Development Goals (SDGs), it is essential to study and define recommendations and requirements for the development of resource-efficient and sustainable solutions.

Below are examples illustrating how edge cloud service can benefit service robots:

- Intelligent navigation: Real-time information about the robot's surroundings, including obstacle locations, can be provided through edge cloud service. This assists in improving navigation performance and avoiding collisions.

- Object recognition: Edge cloud service enables robots to recognize objects in their environment, enhancing their ability to interact with objects and accomplish tasks.

- Human-robot interaction: By leveraging edge cloud service, robots can communicate and interact with humans in a more natural manner. This facilitates better comprehension and response to human commands.

Edge cloud service is a promising new technology that has the potential to revolutionize the way that service robots are used. By providing robots with access to more powerful computing resources, edge cloud service can improve the performance and efficiency of robots, which can lead to new and innovative applications for service robots.

Fig. 6.x.1-1 presents some examples of usage scenarios where MEC (Edge cloud service) is used for geo-surface sensing data management. The expected types of aerial robots are fully autonomous robots, semi-autonomous robots, remote-controlled / teleoperated robots. The robots are expected to be able to meet the orientation value requirement, required by the robot operator or by the operating applications that are coordinated by the robot operator in the edge cloud.

Fig. 6.x.1-1: Usage scenarios of geo-surface sensing data management. Some features and KPI characteristics for communication are recommended [r3].

### 6.x.2	Pre-conditions

UAV (uncrewed aerial vehicle) 1 (UE1) and UAV 2 (UE 2) are aerial service robots (UEs) euiped with data collection capabilities (e.g., 3GPP sensing, non-3GPP sensing, HD cameras).

UAV 1 and UAV 2 are able to communicate with a 6G NTN node (satellite) and/or a 6G TN node (or 6G RAN, base station). They are able to send data to an MEC server via 6G NTN, 6G TN or a combination of them.

UAV 1 and UAV 2 send data to an MEC server accordding to the characteristics of the data identified by the server: for example, some data needs to be sent in a real-time manner whereas other data don’t have to be sent as urgently as data that require real-time transmission.

### 6.x.3	Service Flows

- UAV 1 (UE1) and UAV 2 (UE2) perform data collection over a designated area of interest as specified by an application (e.g., image capture, video recording, or remote sensing).

- Data collection amount and time to transmit/share with suitable access control:

  - UAV 1 (UE1) has collected a large amount of data and is also expected to keep collecting a large amount.

  - UAV 2 (UE2) has collected relatively a smaller amount of data and is not expected to experience an increase in the amount of data to be collected.

  - UAV1 (UE1) started pre-processing of collected data whereas certain portion of the data is left for MEC server to process.

  - UAV 1 (UE1) recognized that transmission of the suitable portion of collected data to MEC server (e.g., for AI task offloading) is necessary and UAV 1 initiates an access attempt to the radio network of 6G system, which needs a suitable access priority.

  - Time elapsed. UAV 2 (UE2) also needs to initiate an access attempt to the radio network of 6G system, which needs a suitable access priority.

- The collected data is transmitted to either a terrestrial network (TN) node, a non-terrestrial network (NTN) node, or both simultaneously – enabling multipath transmission for a single session – and subsequently forwarded to the MEC server for further processing.

- UAV 1 and UAV 2, equipped with fine-grained control of their orientation, adjust accordingly to focus on the newly specified sub-region and proceed to collect and transmit data to the MEC server.

- While UAV 1 is collecting data along a predefined trajectory, it encounters the need to perform a handover due to changing network conditions.

- UAV 1 continues transmitting the data being collected, and no significant degradation in session continuity is observed during the handover, thanks to the enhanced capabilities of the 6G system.

NOTE: In conventional 3GPP terminology for human users, the term "degradation" typically relates to "user experience." However, in this context, the focus is on maintaining a minimal level of degradation for intelligent robots and their applications which perform computation, analysis, inference, and decision-making via AI functionality. Ensuring continuity and quality of data delivery is essential for sustaining the performance and reliability of such machine-driven processes.

- UAV 1 continues transmitting the data being collected, and no significant degradation in session continuity is observed during the handover, thanks to the enhanced capabilities of the 6G system.

- While performing data collection, UAV 1 began to experience degradation in data rate offered by the 6G TN and it requests more network resources to the 6G system in order to overcome the degradation.

- 6G system offers NTN resources for UAV 1 with propagation delay-related information (e.g., X ms (GEO case), or Y ms (LEO case) and connection quality-related information (i.e., expected disruption time).

- UAV 1 check with the MEC server if the delay bound is acceptable when it use both TN path and NTN path for a single session and if the expected disruption time is acceptable, based on the information provided by the 6G system.

- It is agreed that the delay bound and expected dirsuption time are acceptable.

### 6.x.4	Post-conditions

UAV 1 (UE1)’s and UAV 2 (UE2)’s access attempts were able to receive suitable opportunity/priority (e.g., suitable access parameters values), and successfully completed the access to initiate AI-related data transmission.

UAV 1 and UAV 2 were able to pinpoint the specific areas of interest, designated by an application.

UAV 1 was able to continue transmitting the data being collected with no significant degradation in session continuity.

UAV 1 (UE1) was able to use both TN and NTN resources (i.e., multiple data transmission paths) even when the TN connection experiences lower data rate.

UAV 1 was able to check if the delay bound and expected disruption time are acceptable before making path relocation.

### 6.x.5	Existing features partly or fully covering the use case functionality

Clock synchronisation: 3GPP TS 22.104

- clause 5.6.1 Clock synchronisation service level requirements

- clause 5.6.2 Clock synchronisation service performance requirements

- clause 7.2.3.2 Clock synchronisation requirements

NOTE 1:	The types of sensor data and media that robots are collecting, pre-processing and sharing with each other and/or with edge cloud (or edge server, cloud server) are related to the need of fulfilling the above sets of requirements.

Multi-path relay: 3GPP TS 22.261

- clause 6.9.2.1 support of a traffic flow of a remote UE via different indirect network connection paths

Positioning: 3GPP TS 22.261

- clause 7.3.2 High accuracy positioning performance requirements (see also clause 5.7.1 of 3GPP TS 22.104 for Factory of the Future scenario)

Efficient user plane: 3GPP TS 22.261

- Clause 6.5 Efficient user plane

Service continuity: 3GPP TS 22.263

- clause 5.5 Service continuity

NOTE 3:	The term “continuity of traffic session” can be interpreted in a few different ways, depending on the type of robotic applications. For example, for real-time acoustic and/or audio sensing data, it can be interpreted as “service continuity” (i.e., within 20 ms of interruption), for 3D RF sensing measurement data, it is recommended to ensure up to 10ms of interruption not including the intrinsic propagation delay (e.g., a total of approximately 500 ms for one round trip to a ground station over a GEO satellite, 4 * 125 ms) [r4]. For example, the minimum value occurs when a UE is located at a point on the equator closest to the satellite, approximately 36 000 km  / (3 * 10^5 km / s) ≈ 120 ms..

### 6.x.6	Potential New Requirements needed to support the use case

[PR.6.x.6-002] The 6G system shall provide a means to ensure continuity of traffic sessions needed for multiple UEs (service robots) collaborating for an AI task when some of those UEs (service robots) need handover.

[PR.6.x.6-004] While a UE (service robot) uses multiple direct network connections for a single traffic session (e.g., one connection via TN and the other via NTN), the 6G system shall provide a means to provide an acceptable range of arrival time difference(s) of information delivered via different connection paths and to indicate its capability to meet the range requested by the trusted third party.

Editor’s Note: This requirement is FFS.

[PR.6.x.6-005] The 6G system shall provide a means to expose propagation delay-related information to a trusted third party (e.g., robotic application) when offering candidate connection path to switch to (e.g., from TN to NTN, or from TNT to TN).

[PR.6.x.6-006] The 6G system shall provide a means to provide expected disruption time for a UE (service robot) when communication path switching (e.g., from TN to NTN) is necessary so that the UE  can select suitable alternative path according to its preference and traffic characteristics.

* * * Second Change * * * *

# 2	References

[r1]	G.W. Morgenthaler, et al, "Feasibility of Using a Miniature-UAV Remote-Sensing/Precision-Agriculture System (MINI-UAV RSPA SPACE SYSTEM) to Increase Crop Yields, Lower Costs, Save Water, and Reduce Pollution of Air, Soil, and Water," Aerospace Research Central Nov. 2012. Online: https://arc.aiaa.org/doi/abs/10.2514/6.IAC-06-D3.2.09

[r2]	J. J. Lee, "Minimizing Sensor Fusion Disruptions in UAV-Based Collaborative Remote Sensing for Wildlife Preservation," IEEE Sensors Letters, vol. 8, Iss 4., 2024. Online: https://ieeexplore.ieee.org/document/10439637

[r3] 	ATIS Next G Alliance, 6G Applications KPI database, https://6gkpi.atis.org/application-groups/

[r4] 	K.-D. Lee, “An efficient real-time method for improving intrinsic delay of capacity allocation in interactive GEO Satellite networks,” IEEE Transactions on Vehicular Technology, vol. 53, no. 2, pp. 538 – 546, March 2004.  Online: https://ieeexplore.ieee.org/document/1275718

[r5] 	S. Stevens and D. Rush, “Urban fire spread modelling: a review of dynamic computational models and potential for application to informal settlement fires”, International Journal of Disaster Risk Reduction, vol 124, June 2025. Online: https://www.sciencedirect.com/science/article/pii/S2212420925003528

* * * Next Change * * * *

<Proposed change in revision marks>