3GPP TSG SA WG 1 Meeting #111 	S1-253661

25-29 August 2025, Goteborg, Sweden	(revision of S1-253237)

Source:	SoftBank Corp.

pCR Title:	HAPS-enabled Persistent Wide-Area IoT and Integrated Sensing Services

Draft Spec:	3GPP TR 22.870 V0.3.1

Agenda item:	8.1.4

Document for:	Approval

Contact:	Shiro Fukumoto (shiro.fukumoto01@g.softbank.co.jp)

Abstract: This use case explores the capabilities of persistent HAPS platforms for providing efficient connectivity to massive IoT deployments across large, underserved geographical areas. It also introduces the concept of Integrated Sensing and Communication (ISAC) from HAPS, where the platform simultaneously supports IoT communication and performs wide-area sensing, offering synergistic value-added services.

1. Introduction

This contribution proposes a 6G use case for Integrated Sensing and Communication (ISAC) using High-Altitude Platform Stations (HAPS). A previous version of this proposal (S1-252479) was discussed at the SA1#110 meeting but was not approved due to a lack of sensing KPIs. This updated proposal addresses that issue by defining the necessary KPIs and clarifying texts, such as the use of 3GPP waveforms for sensing. The supporting reference(s) for these KPIs will be provided at the next meeting (SA#112).

2. Reason for Change

Adding the text proposals in the new sub-clause in the clause 7 “Integrated Sensing and Communication”.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 V0.3.1.

* * * First Change * * * *

7.x	Use case on HAPS-enabled Persistent Wide-Area IoT and Integrated Sensing Services

7.x.1	Description

This use case leverages the persistence and wide coverage of HAPS to provide efficient connectivity for massive Internet of Things (IoT) deployments across large geographical areas (e.g., agriculture fields, remote industrial sites, environmental monitoring zones) where terrestrial coverage is sparse or absent. Furthermore, it explores the integration of sensing capabilities (ISAC - Integrated Sensing and Communication) on the HAPS platform itself. The HAPS can collect data from ground-based IoT sensors and simultaneously perform wide-area 6G wireless sensing using radio signals transmitted and received by HAPS, and correlate sensing data with communication services.

This enables a service for persistent wide-area situational awareness to provide a more comprehensive and timely area model of the physical environment than either data source alone.

7.x.2	Pre-conditions

- HAPS platform, basically Heavier-than-Air (HTA) type, deployed over the area of interest.

- Massive numbers of IoT devices deployed on the ground within HAPS coverage.

- HAPS platform has necessary communication payload for IoT connectivity and sensing equipment.

- Feeder link (Backhaul) to ground and/or inter-HAPS may be used, for aggregated IoT data and sensing data.

- Application servers for IoT data processing and sensing data analysis exist.

7.x.3	Service Flows

- IoT Connectivity:

  - Ground-based IoT devices wake up and transmit small data packets (e.g. sensor readings) towards the HAPS.

  - HAPS receives signals from a large number of devices.

  - HAPS aggregates IoT data and forwards it via its feeder link to the core network and application servers.

- Integrated 6G Wireless Sensing:

  - HAPS performs 6G wireless sensing by transmitting and receiving radio signals to derive observables such as

  - Angle of Arrival (AoA), Time of Arrival (ToA)/Round Trip Time (RTT), Observed Time Difference of Arrival (OTDOA) and/or Frequency-Difference of Arrival (FDoA).

  - Processing can be performed on-board HAPS or ground equipment via feeder link, and they correlate the IoT data and the targeted sensing data to generate an enhanced insight, which is then forwarded to the application server.

  - Sensing information can also be used to optimize communication services (e.g. detect interference, adjust beams) or provide standalone sensing services (e.g. precision agriculture maps, environmental reports).

7.x.4	Post-conditions

- Massive IoT devices have periodic connectivity via HAPS.

- Aggregated IoT data is available for analysis and action.

- Wide-area sensing data from HAPS is available.

- Potential correlation/fusion of communication and sensing data provides enhanced insights.

7.x.5	Existing features partly or fully covering the use case functionality

- Massive Machine Type Communications (mMTC) features.

- Existing sensing technologies (though integration with 3GPP comms payload is new).

- Basic connectivity and backhaul.

7.x.6	Potential New Requirements needed to support the use case

[PR 7.x.6-1] The 6G system shall support massive IoT services for efficient operation via HAPS, accounting for doppler, extended delay, large cell size and UE energy efficiency.

[PR 7.x.6-2] The 6G system shall support 6G wireless sensing from HAPS which may include the capability to exploit AoA, ToA/RTT, OTDOA and/or FDoA. The target KPIs are listed in Table 7.x.6-1.

Table 7.x.6-1: KPIs on HAPS-enabled Persistent Wide-Area IoT and Integrated Sensing Services

| Scenario | Object Type | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Spatial resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| HAPS-enabled Persistent Wide-Area IoT and Integrated Sensing Services | Outdoor | [95] | [5] | [10] | [0.5] | [0.5] | [0.5] | [0.1] | [200] | [0.1] | [5] | [1] |
| Note: These KPIs are defined based on the characteristics of HAPS operation. |  |  |  |  |  |  |  |  |  |  |  |  |

[PR 7.x.6-3] The 6G system shall support a service for persistent wide-area situational awareness by fusing massive IoT communication data with integrated sensing data from HAPS.

* * * End of Change * * * *