3GPP TSG-SA WG1 Meeting #111 	S1-253139r1

Goteborg Sweden, 25-29 August 2025	(revision of )

Title:	Update on Use case on multi-sensor fusion based sensing for UAV takeoff and landing

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda Item:	8.1.4

Source:	CMCC

Document for:	Approval

Contact:	Pengtai Qin, qinpengtai@chinamobile.com

Han Meng, menghanyjy@chinamobile.com

Abstract: Update on Use case on multi-sensor fusion based sensing for UAV takeoff and landing.

1. Introduction

The aim of this contribution is to update the requirement of use case 7.15 by addressing the Editor’s Note in this use case.

2. Reason for Change

Try to address EN by adding a note and update requirement for use case 7.15.

3. Conclusions

The EN could be removed and the update requirement could be agreed.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change, all new text * * * *

7.15	Use case on multi-sensor fusion based sensing for UAV takeoff and landing

7.15.1	Description

Logistics companies are looking to UAVs to save manpower, costs, and time as well as improving quality and efficiency.  So far, UAVs have been mainly used for the short distance deliveries. For example, they delivery medicines or blood donations to hospital; they fly samples to the laboratory in the chemical industry; and they fly spare parts to the production line in the automotive industry. In the German city of Bonn, for instance, DHL has been testing the delivery of medicines to a hospital across the Rhine River [198].

UAVs may operate in complex environments such as urban areas, forests, and mountains. These environments pose challenges for traditional navigation and sensing systems. Multi-Sensor Fusion based sensing can overcome these challenges by integrating data from different sensors to provide a more accurate representation of the environment. For example, cameras can provide visual information, Radar and sensing base stations, which has the same sensing process logic, can detect moving targets and its distance. By fusing these data sources, UAVs can better navigate through complex terrains and avoid obstacles during takeoff and landing.

Although latest technology improvements have helped UAVs to be more suitable for these new scenarios, there are still many challenges [199], including:

1. Performance deterioration of satellite based sensors: Tall buildings in the urban area could lead to the urban canyon effect, which creates a NLOS environment where the UAV GNSS positioning accuracy would deteriorates greatly [200], especially in take-off and landing phases.

2. Constrained sensing capabilities: Currently, UAVs are typically using sensors like cameras and millimetre wave radar for obstacle avoidance, but these sensors can only sense the targets within a short distance (<60m), and couldn’t be used during night time or in bad weather, which constrain the UAV’s flying speed and service time.

Multi-Sensor Fusion based sensing can enhance the accuracy and reliability of navigation, which is crucial for UAV safe takeoff and landing. Meanwhile, Multi-Sensor Fusion based sensing can help UAVs detect obstacles, assess the landing area, and adjust their flight path in real time, thereby reducing the risk of collisions and ensuring a smooth landing.

In general, Multi-Sensor Fusion based sensing plays a crucial role in assisting UAV takeoff and landing by enhancing safety, navigating complex environments, and enabling autonomous operations. With the continuous advancement of 6G technology, we can expect to see further improvements in Multi-Sensor Fusion based sensing in 6G systems, leading to more reliable and efficient UAV operations.

7.15.2	Pre-conditions

Figure 7.15.2-1: Multi-Sensor Fusion based sensing for UAV takeoff and landing

The UAV operator provides package delivery service in an area which is covered by 6G network. The UAV operator subscribes to the UAV flight takeoff and landing supporting service from the 6G network operator.

The 6G network operator provides sensing service in an area by deploying different sensors (including camera, radar, and sensing base station) around the UAV landing area. And those sensors can report respective sensing data to 6G network.

7.15.3	Service Flows

1. The UAV operator plans to use one UAV to deliver packages and the UAV operator/UAV requests the 6G network to provide UAV flight takeoff, landing and obstacle avoidance supporting service. . In addition, the UE on board this UAV also sends its UE’s identification, trajectory information, required alarm information (e.g. risk collision), and requirements for the received sensing service (e.g. sensing accuracy, refreshing rate, etc.) to the 6G network operator

2. The 6G network operator activates the UAV flight takeoff supporting function and trigger the 3GPP sensing and non-3GPP sensing (directly or via agent) around UAV takeoff area.

3. Different sensors then send the different (3GPP and none-3GPP) sensing data to the 6G network to start sensing data fusion processing.

4. The 6G network calculates the sensing result based on the received sensing data and provide the fusion sensing result to UAV operator/UAV to assist UAV in takeoff phase.

5. Once the UAV takes off successfully, the UAV operator/ UAV triggers the 6G network to stop UAV flight takeoff and landing supporting service.

6. During the flight, once a risk collision is detected or predicted based on the sensing result (e.g. an unregistered UAV or bird is predicted entering the flight path and may collide with UAV), the 6G network can deliver information of risk collision for precaution. The 6G network may also recommend a flight path adjustment to the UAV operator/UAV to help with the UAV flight.

7. When the UAV flies back above the load area, the UAV operator/UAV triggers the 6G network start UAV flight takeoff and landing supporting service again. The 6G network start multi-sensor fusion based sensing and provides fusion sensing result to the UAV to support landing.

7.15.4	Post-conditions

UAV delivers package successfully with the support of 6G network.

7.15.5	Existing features partly or fully covering the use case functionality

SA1 has identified a series of requirements related to non-3GPP sensing data and captured them in TS 22.137 [6].

5.2.1	General

Subject to user consent, regulation, and operator’s policy, the 5G system should support the joint processing of the 3GPP sensing data and non-3GPP sensing data to derive a combined sensing result.

5.2.4	Security and privacy

The 5G system shall support encryption, integrity protection, privacy of the 3GPP sensing data, non-3GPP sensing data and sensing results, to protect the data inside the 5G system.

And there is non-3GPP sensors related requirement captured in clause 4.2 of TS 22.137 [6].

These non-3GPP sensors could include radar camera or Wi-Fi sensing. While the mechanism of these types of sensing is not considered in this specification.

7.15.6	Potential New Requirements needed to support the use case

[PR 7.15.6-1] Subject to regulatory requirements and operator’s policy, 6G network should support suitable means to trigger non-3GPP sensing data collection from non-3GPP sensors.

NOTE: 6G network can collect non-3GPP sensing data from non-3GPP sensors directly or from third party.

[PR 7.15.6-2] The 6G system shall be able to provide sensing service by enabling multi-sensor fusion based sensing as described in the following Table 7.15.6-1.

Table 7.15.6-1: Performance requirements for Multi-Sensor Fusion based sensing for UAV takeoff and landing

| Scenario | Object Type | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Spatial resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| UAV takeoff and landing | Outdoor | [95] | [0.1] (note 1) | [0.1] (note 1) | [1] | [1] | [0.5] | [1] | [100-500] | [0.2] | [5] | [5] |
| NOTE 1: The KPI values for accuracy of positioning are centimetre-level sourced from reference [199], [200] NOTE 2: The KPI values of this use case are effective for sensing in specific area for UAV takeoff and landing. |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *