3GPP TSG-SA WG1 Meeting #112 	S1-254217

Dallas, USA 17-21 November 2025	(formerly S1-253356)

Source:	Xiaomi

pCR Title:	pCR for New use case UE sensor groups

Draft Spec:	TR 22.870 v0.4.1

Agenda item:	8.1.4.2

Document for:	Approval

Contact:	Gordon Young <gordonpetery@xiaomi.com

Abstract: this paper proposes a new use case for a sensor group for a single UE, wherein a vehicle has connected two independently operated sensors, each providing a different sensing view of its surrounding environment.

<FIRST CHANGE>

## 7.X	Autonomous driving service with a UE using multiple sensors

### 7.X.1	Description

Vehicles equipped with a UE supporting 3GPP sensing service may have one or more sensors installed in a distributed manner which may or may not be a 3GPP sensor e.g. camera or LiDAR, in order to support better coverage of the vehicle’s sensed local environment.

A UE when registering as a sensing entity for a sensing service will indicate to the 6G network the capabilities of its available sensors independently, so that the 6G system is able to determine whether to select it for sensing operation for a particular target sensing service area. In some instances when the UE has more than one sensor, each sensor can be selected for sensing operation on an individual basis, and each may have different view of its environment.

Figure 7.X.3-1. Truck with two sensors, front and rear facing

In some instances the vehicle may be able to pre-process the respective sensors’ measurement data creating a fused sensing result for reporting from the UE. The UE reports the respective sensor measurement data for each sensor or fused measurement result in accordance with the sensing service configuration.

The view of the sensing service target area (TSSA) for a sensor is directly related to the position, the antenna aperture and the orientation of the sensor relative to the true position of the UE. In the case of a vehicle with 2 sensors each sensor may be positioned on the vehicle so that it provides distinct or different view from the vehicle of the target sensing service area, e.g. on a car or transport vehicle sensors may provide forward facing and rear facing views.

For road users operating in a TSSA the activating a UE sensor is determined by the 6G network function controlling the authorized sensing service, with agreement of the UE connected to the sensors.

The sensing service managed by the sensing controlling function may be provided by the 6G system or a trusted third party.

In this case the sensors could be considered as a sensor group, wherein they have a know proximity to one UE which is responsible for managing the sensors, and communicating with the sensing service. However in this scenario the sensors are configured to one UE and not associated with separate UEs with individual sensors as in [9] section 5.19.

During discovery for sensing entities to support a sensing service the MNO P sensing service identifies available sensing capable UEs in or near the TSSA, in addition as a part of the discovery the 6G network sensing function may determine that a vehicle has more than one sensor, potentially with different views of the target sensing service area.

In order to ensure network efficiency rather than have the UE send all available sensing data for both sensors all the time when the UE is in an active sensing operation, selecting one or more of the sensors individually may be advantageous. For example in some instances one of the sensors may not be facing the target sensing service area. Alternatively one of the sensors, e.g. acting as a sensing receiver, may not be facing in the direction of a sensing transmitter (bistatic configuration) to provide meaningful sensing measurement data.

In this case the sensor connections to the UE are considered to be ideal (no loss) and can be wireless or wired.

In a scenario where a vehicle with two sensors moves through a target sensing service area, depending on the 6G network sensing control function configuration, the UE may be configured to activate one or other or both of the available sensors independently, and report the measurement data to the 6G network individually. This configuration and activation may relate to the direction of travel of the vehicle as it passes across a target sensing service area. At some instance the vehicle could be configured to activate both connected sensors, and additionally report measurement data for both sensors separately, or as an alternative if sensing data fusion is supported at the vehicle UE, may be configured to report a fused sensing report combining sensing measurements received independently from each sensor for the environment around the vehicle.

As it moves away from the target sensing service area, the 6G network sensing control function may then decide to deactivate one, or both, of the vehicle sensors in order to maintain an overall efficient sensing service configuration, e.g. considering other available sensing entities and resources at its disposal for sensing the target sensing service area.

### 7.X.2	Pre-conditions

Vehicles A, B, C and D are registered subscribers to a third party autonomous driver service X which provides V2X services and sensing services of different levels of autonomy or functionality to subscribers in a sensing service area e.g. mapping, warning and sensing services to aid drivers of vehicles A, B, C and D.

Autonomous driving service X has an agreement with MNO P for provision of sensing service and V2X services including localisation and user privacy for reported service results.

Vehicles A, B, C and D are registered with MNO P and are in network coverage and connected.

Vehicle A has 2 sensors fitted 1 front and 1 rear. Service X and MNO P’s 6G sensing service control function is aware of the two sensors and vehicle a has provided this capability information to sensing service of MNO P. Initially only the forward sensor on vehicle A is activated and reporting sensing measurement information for the configured target sensing service area.

Vehicle D is stationary.

### 7.X.3 	Service Flows

- The vehicles A, B, C and D are travelling in a rural area and approaching a T-junction. Traffic is approaching from both the left and right of the junction and vehicles A and B need to turn into the road passing the T-junction safely, with vehicle A turning left and vehicle B turning right (traffic drives on the left of the road in this scenario).

- The visibility at the junction is poor due to high hedges and poor light etc., and vehicles A and B are reliant on the service X for safe passage, and avoiding of traffic.

Figure 7.X.3-2 Vehicles approaching T-junction and after vehicle A has turned with both sensors active

- Approaching the junction the sensing service X determines that it wishes to have the rear sensor on vehicle A activated, so that it can have an enhanced view towards vehicle C, after vehicle A has entered the same lane of traffic. Also the rear view will give a clear view of vehicle D as vehicle A passes, including capturing its size and if it starts moving, to be able to inform the sensing service of vehicle D potentially causing a problem for vehicle B which is intending to turn in the opposite direction to vehicle A, and into the same lane as vehicle D.

- Vehicle A provides sensor measurement data for both sensors to the MNO P sensing service being operated and delivered to service provider X.

- The MNO P sensing service with knowledge regarding the rear sensor orientation (field of view) is able to fuse the sensing data of the additional sensor from vehicle a with the other received sensing data from vehicles A, B, C and D to create the required sensing result.

- Vehicle D moves off, gaining speed and approaches the junction.

- MNO P forwards the sensing result in accordance with the sensing service requirements to service provider X.

- Sensing service provider X delivers autonomous driving service combined with sensing results to vehicles A, B, C and D.

### 7.X.4 	Post Conditions

Sensing service provider detects and determines the size of vehicle D early as vehicle A approaches and passes it, and is able to ensure vehicles A and B navigate the junction safely.

### 7.X.5	Existing features partly or fully covering the use case functionality

The following requirements exist for sensing entity configuration, but do not consider a UE with more than one available sensor for independent operation.

TS 22.137 [6] section 5.1.

5.1 General

…

It is important to consider energy efficient sensing operations which can include temporarily disabling sensing transmitters and receivers that are not involved in sensing and communication operations or adjusting the sensing operation parameters (e.g., sensing frequency) to minimize energy consumption. …

5.2 Requirements

5.2.1 General

…

The 5G system shall be able to provide 5G wireless sensing service in a target sensing service area location using sensing transmitters and sensing receivers.

Subject to regulation and operator policy, the 5G network shall be able to activate, configure, and deactivate 5G wireless sensing based on parameters such as location and network conditions (e.g., network load). …

5.2.2	Configuration and authorization

Subject to regulation and operator’s policies, the 5G network shall be able to configure and/or authorize or revoke authorization of sensing transmitter(s) and sensing receiver(s) for 5G wireless sensing service.

NOTE 1:	Such configuration and authorization can be based on sensing transmitter or sensing receiver location, specific time, sensing duration, sensing accuracy, target sensing geographical area, establishing of communication to transfer sensing data, etc.

NOTE 2:	Such configuration and authorization can also include the selection of multiple sensing transmitters/receivers for 5G wireless sensing services. …

In addition relating to sensing groups or use of multiple sensors in proximity to a UE, TR 22.837[9] section 5.19 specifies the need to identify a sensing group (of sensing entities) in proximity to a UE requesting a sensing service. Groups are established based on group members locations being in proximity to a UE requesting a sensing service and it is only captured for the handling of delivery of a sensing service to a UE requesting the sensing service within the sensing group. This was not carried on to normative requirement for 5GA sensing.

[9] CPR 7.1.3-2	Subject to operator’s policy, the 5G network shall be able to provide secure means to enable trusted third-party to request discovering a sensing group in the proximity of the UE that is requesting a 5G wireless sensing service from application server.

NOTE:	[9] Network Exposure CPR 7.1.3-2 represents PR 5.19.6-2

### 7.X.6	Potential New Requirements needed to support the use case

[PR 7.X.6-1] Based on third-party request and subject regulatory requirements, operator’s policy and the user consent, the 6G system shall be able to discover a sensing entity with more than one independent sensor in order to support a sensing service.

[PR 7.X.6-2] A 6G system shall be able to independently configure the sensors of a sensing entity supporting more than one sensor directly connected to the UE, to enable an efficient and resilient sensing service for a target sensing service area.

NOTE 1:	This requirement assumes that activation of the second rear sensor on vehicle A is independent or the front facing sensor also connected to the UE and is activated by the 6G network with a sensing service configuration independently from the front facing sensor and its activation status.

NOTE 2:	Sensors connected to a UE are assumed to have an ideal connection, and not utilizing MNO spectrum.