3GPP TSG SA WG 1 Meeting #112 	S1-254108r1

17-21 November 2025, Dallas, Texas, USA	(revision of S1-254108)

Source:	SoftBank, NEC, Rakuten Mobile, NVIDIA, Nokia

pCR Title:	Update on Use case 6.24 distributed 6G network for AI computing

Draft Spec:	3GPP TR 22.870v0.4.1

Agenda item:	8.1.3.2

Document for:	Approval

Contact:	Shunsuke.kawashima at g.softbank.co.jp

1. Introduction

Further study found that the description of network capabilities for distributed AI computing needs to be clearer, especially for dynamic adaptation and session continuity.

This contribution revises PR 6.24.6-2 to explain dynamic traffic route control and coordination with the application server, and adds a new requirement PR 6.24.6-4 on session continuity during workload relocation or user mobility.

2. Reason for Change

PR 6.24.6-2 is updated to state that the 6G network can provide Service Hosting Environments and traffic routes dynamically, and can consider AI application needs such as latency tolerance and computation intensity in coordination with the application server.

A new requirement, PR 6.24.6-4, is added to specify that the 6G network can keep session continuity and service performance when workloads are moved or users move between Service Hosting Environments.

These changes make the requirements clearer and better reflect the needed functions for energy-aware and compute-aware distributed AI computing.

3. Conclusions

-

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870v0.4.1.

* * * First Change * * * *

6.24	Use case on distributed 6G network for AI computing

6.24.1	Description

As outlined in this document, numerous high demand use cases in 6G networks, such as augmented reality and autonomous vehicles, are anticipated to heavily rely on AI, requiring significant computational load and substantial power consumption. Currently, many network configurations centralize these computational loads in urban areas, with power supply also concentrated in these areas. In the context, it is concerned that the balance between power consumption and supply are collapsed, which bring a power outage in the urban area.

Ideally, a balance between power consumption and supply is achieved. The power supply is physically distributed across different areas, each with certain constraints (e.g. an upper limit on power supply). Power generation changes according to the demand within the coverage area. It is expected that computation can be physically distributed, and the use of computations can be adapted to match the power supply. Therefore, mitigating these imbalances to maximize overall computational resources and energy efficiency is highly recommended, especially in the context of AI workloads.

As shown in Figure 6.24.1-1, this use case aims to distribute computational workload by appropriately selecting a computing site by routing traffic, particularly considering the available computational resources and surplus power in each area, while also accounting for the specific needs of AI-related tasks. Operators can create optimal computing site selection policies based on typical power usage patterns and the dynamic demands of AI applications to achieve efficient energy consumption. Furthermore, this use case incorporates low latency requirements per traffic flow in the policy and the consequent optimized routing, which is crucial for many AI-powered services.

Figure 6.24.1-1: Distributed 6G Network for AI Computing - The sites A, B, C are the deployed Service Hosting Environment

6.24.2	Pre-conditions

1.	Operators distribute computational resources across sites and use the electric power from the areas where these sites are located, to provide computational services.

2.	Third-party electric power monitoring systems provide detailed electric power consumption data per computing site via APIs.

3.	Operators inference the balance of power consumption and supply for specific time periods for each computing site.

4.	Operators can pre-define their own appropriate computing site selection policies based on user attributes (users’ locations, communication time, users’ profiles, etc.) and the latency requirements of the users’ communications.

6.24.3	Service Flows

Based on electric power usage patterns, and power supply information provided by power supply monitoring systems outside 3GPP networks, operators use their own optimal selection policies to select the computing sites and then map computation workloads onto the computing sites. These policies are dynamically adjusted according to the specific location, communication time, users’ profiles, and latency requirements of the users’ communications, as well as the balance between power consumption and the available power supply which is managed outside 3GPP networks. The selected computing site from these policies must optimize network load and electric power consumption in real time, achieving better distributed electric power consumption and improved overall energy efficiency.

6.24.4	Post-conditions

The introduction of the computing site selection policies enhances energy efficiency and achieves balancing of power consumption which is especially important during peak electric power consumption periods. This enables operators to achieve sustainable and energy efficient network operations.

6.24.5	Existing features partly or fully covering the use case functionality

Energy efficiency has been considered in clause 6.15, TS 22.261 [14]. However, Considerations on a balance between power consumption and supply are still insufficient, especially in achieving significant energy efficiency improvements and distributing computational resources that are aimed at this use case. Thus, new requirements are further needed.

6.24.6	Potential New Requirements needed to support the use case

[PR 6.24.6-1] The 6G network shall be able to collect energy related data of the Service Hosting Environment.

[PR 6.24.6-2] The 6G network shall be capable of providing appropriate Service Hosting Environment in order to accommodate compute and communication (e.g. traffic load) resources to meet service communication requirements (e.g. bitrate and latency) and service computing requirements (e.g., route, latency tolerance, computation intensity).

[PR 6.24.6-3] The 6G network shall be able to consider energy related information from energy management system, which is outside of 3GPP networks, to support mechanisms in order to effectively use the energy, e.g. efficiently use the supplied energy from the energy supplier.