3GPP TSG SA WG 1 Meeting #112 	S1-254126r1

17-21 November 2025, Dallas, Texas, USA	(revision of S1-254126)

Source:	Verizon, EDF, NOVAMINT, Itron

pCR Title:	Pseudo-CR on KPI update on Utility Direct Transfer Trip

Draft Spec:	3GPP TR 22.870-V0.4.1

Agenda item:	8.1.8

Document for:	Approval

Contact:	samita.chakrabarti@verizon.com

Abstract: Follow-up FFS on KPI table update for section 11.24 on Utility Direct Transfer Trip protection. This contribution is jointly developed with Electric Power Research Corporation in USA.

1. Introduction

SA1#111 introduced the Utility vertical usecase for fault protection from high voltage to prevent large area power shutdown. The mechanism may be used in other scenarios in electric grid system for fault protection.  The S1-253541 was approved last time with a note for updating the KPI table .

2. Reason for Change

Updating the KPI table in section 11.24 with approximate data available from the Utility companies.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to the next revision of 3GPP TR 22.870 V0-4.1

Other changes: comments from Monday Nov 17th:

- Remove the additional EN

- When >2 devices are used ( i,e, multicast, broadcast). Note the DTT usecase typically are used  between 2 UE devices. they can use multiple unicast messages between a sender  device and multiple devices.  Thus the connection is always between 2 devices

- Resolved EN for figure description

- Added Transfer interval definition

- Added Itron as a supporter

* * * First Change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

…….

Sensing target density: total number of objects to be sensed per geographic area. It is a measure of how many objects the 3GPP system can detect, identify and/or track within a target sensing area.

Service Hosting Environment: the environment, located inside of 6G network and fully controlled by the operator, where Hosted Services are offered from.

Serving satellite: a satellite providing the satellite access to a UE. In the case of NGSO (Non-Geostationary Satellite Orbit), the serving satellite is always changing due to the nature of the constellation.

Transfer interval: As defined in TS 22.104 [64]

## 3.2	Symbols

Void.

## 3.3	Abbreviations

* * * Next Change * * *

## 11.24	Use case on utility direct transfer trip for distributed energy resources integration and protection

### 11.24.1	Description

Low latency communication from device to device is required for many grid protection and control functions. Rapid communication of a fault condition enables protective actions such as breaker trips to take place before equipment damage occurs. Communication is also used to prevent false tripping and unnecessary outages. Many complex factors related to the type of protection, and the electrical characteristics of the circuits being protected, affect the choice of protection schemes and the resultant communication requirements. In simple terms, the more electrical power being delivered, the faster the required response, and the higher the required reliability.

Growing numbers of interconnected DER sites (examples: solar and wind) have increased the need for protection. Large DER sites may deliver hundreds of kW to a few MW of generated load back to the utility grid. These loads can potentially destabilize the grid without fast and reliable monitoring and control, resulting in communication latency requirements in the low 10s of ms. It is difficult to reliably provide that performance with current wireless networks, so an optical fibre connection is often required. Since these DER sites are frequently rural, there is little to no existing fibre, so the installation can be prohibitively expensive. There is a need and opportunity for 6G networks to meet these requirements at a much lower cost than new fibre installations. Also, a node-to-node communication ability will be useful for low or no coverage areas of the PLMN for reliability of the Grid.

The benefits of 6G in supporting DER

The positive impact from the 6G system use case will enable the construction of renewable energy sources (solar and wind) in more locations, by avoiding costly installation and maintenance of fibre networks.

Energy resources: As explained, the 6G system can play a central role in sustainable and resilient use of energy resources via by enabling renewable energy installation in more remote locations at a much lower cost.

Emissions: The use of wireless instead of requiring the installation of new fibre reduces materials consumption and emissions from the fibre installation process.

Trustworthiness: There is an introduced risk of cyber-attacks from introducing connected devices across society-critical infrastructures such as utilities that require security measures towards various threat vectors. Also, the risk for privacy intrusion needs to be considered. The efficient secure protocols defined in 3GPP networks and deployed by operators are typically combined with end-to-end security between the devices implementing the protection system.

Infrastructure: The 6G system with appropriate and new requirements can serve the growing utility infrastructure needs for better control and protection of renewable energy generation in an efficient way. Vulnerability includes the increased risk of cyber-attacks as mentioned, something that comes with any wireless network operations.

Figure 11.24.1-1:  Direct Transfer Trip system with DER (source: EPRI)

* * * Next Change * * *

### 11.24.6	Potential New Requirements needed to support the use case

[PR 11.24.6-1] Subject to Operator’s policy, the 6G network shall support end-to-end, secure and reliable IoT device communication with the network following KPI’s in Table 11.24.6-1

[PR 11.24.6-2] Subject to Utility Company and Mobile Operator’s policy, the 6G network should provide alternate means to communicate between devices at both ends in the event when a UE loses its connectivity to the 6G system.

Table 11.24.6-1: KPI for direct trip transfer (DTT) use case

| Use case |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Data rate down link (kbit/s) | Data rate up link (kbit/s) | End-to-end latency: maximum (ms) | Message size (Byte) | # of UE connection | Communication service availability: target value | Transfer Interval (ms) |
| Direct Transfer Trip NOTE | [50 – 5000] | [50-5000] | [<5 - 10] | [1500] NOTE1 | [2 - 10] | [> 99.999] | [ 100 ] |
| NOTE: DTT uses periodic and aperiodic communication service supporting messages for fault location, isolation, and service restoration. NOTE1: Typical GOOSE message sizes are 500-byte in IEC 61850 [356] messages plus any overhead due to IP tunnelling, security etc make 1500 kBytes approximately.. DTT messages are always passed between 2 protection devices(UE) |  |  |  |  |  |  |  |