3GPP TSG-SA WG1 Meeting #112 	S1-254170r1

Dallas, USA, 13 – 17 November 2025	(revision of S1-253134r1)

Source:	China Mobile, FAU

New use case title:	Resubmit new use case on Predictive Maintenance in Smart Factories

Draft TS/TR:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.4.2

Document for:	Approval

Contact:	Xiaonan Shi, shixiaonan@chinamobile.com

Shiying Zheng, zhengshiying@chinamobile.com

Mai Alissa, mai.alissa@fau.de

Abstract: In this use case on Predictive Maintenance in Smart Factories, the focus is on predicting equipment failures through more comprehensive sensing and data collection, thus better to predict if it needs a repair to avoid breaking down. This predictive approach is beneficial because it leverages 6G system capabilities (including ISAC and optional massive sensor access) to analyze

vibrations, enabling early detection of irregularities and providing comprehensive insights into equipment operating status, which brings additional benefits to factory operations and the communication network (6G system).

The use case on Predictive Maintenance in smart factories has been elaborated in the “German Perspective on 6G –Use Cases, Technical Building Blocks and Requirements” White paper,  which has been deeply studied by FAU (Friedrich-Alexander-Universität Erlangen-Nürnberg) , and also has been elaborated in the 5G-ACIA’s work item (WI087 ‘Early considerations of network capability evolution for industries and Automation’).

In R1, the following changes are given below:

Firstly, only keep the PR1 that due to the PR2 has already been reflected in PR1.

Secondly, in section 7.y.1, that give more description of the predictive maintenance. That in this paper, only focus on the sensing of the equipment location, for example, an abnormal drop of the robotic arm led to a change in its position, or, if the robot is broken down and fails to follow the designed forwarding route, then in the wrong position.

At last, change the accuracy of positioning estimate by sensing to 5m, because that this is related to the bandwidth can be available in FR1 frequency range. And in this revision, range resolution of ≤5 m is coming from the fact that up to a 30 MHz of BW can be available in FR1 frequency range.

* * * First Change, all new text * * * *

7.y	Use case on Predictive Maintenance in smart factories

7.y.1	Description

Predictive maintenance is an advanced maintenance method which involves periodic or continuous monitoring of equipment condition to predict if it needs a repair to avoid breaking down. The method allows to minimize unplanned downtime for otherwise reactive equipment repairs, reduce maintenance costs, maximize equipment lifespan and increase revenues as a result.

The core idea of predictive maintenance​ is to use data and advanced technologies to predict​ when a piece of equipment is likely to fail, thereby scheduling maintenance just before the failure occurs, at the most optimal time.

Predictive maintenance relies on to continuously collect data. This data is transmitted to a computer system and processed using the Internet of Things (IoT), big data analytics, and machine learning/artificial intelligence algorithms. By analyzing changes in data patterns (e.g., an abnormal drop of the robotic arm led to a change in its position, or, if the robot is broken down and fails to follow the designed forwarding route, then in the wrong position.), the system can identify potential signs of failure. Ultimately, the system alerts the maintenance team, notifying them to pay attention to a specific piece of equipment, enabling precise intervention.

The use case takes advantage of the deployed indoor 6G system where its RAN nodes and/or UEs are enabled with the capability to collect RF sensing data (ISAC), optional support for massive sensor access capability. For multiple types of manufacturing equipment, it is possible to predict impending failures by analyzing their equipment vibration amplitude. This phenomenon is confirmed by experienced factory employees who are able to predict certain types of equipment failures based on their perception of such vibration amplitude. Such vibrations on equipment surface and can be detected by the phase changing measurement of RF sensing system and assigned to respective parts of the monitored equipment.

Predictive maintenance needs to be based on a large amount of historical data and current real-time data. To achieve best performance, including accuracy and real-time responsiveness, etc. Without modifying the equipment, ISAC can provide supplementary data to achieve a more comprehensive sensing of the equipment's operating status. Based on these more comprehensive information, better predictive maintenance could be realized, such as detecting irregularities in otherwise nominal operation patterns and identifying types and locations of impending failures.

Implementing predictive maintenance using the described method will minimize unplanned downtime, reduce maintenance efforts, maximize equipment lifespan and thus increase benefits, which could provide services to companies running manufacturing facilities, operators of automated warehouses, operators of 6G-enabled automated construction sites.

The use case is mainly applicable to private spaces, like inside of factories or warehouses, however it can also be useful for scenarios where a relevant type of equipment that requires monitoring is deployed in public spaces or in their immediate proximity (e.g. 6G-enabled automated construction sites, road construction sites).

7.y.2	Pre-conditions

1.The network supports ISAC capability and optionally supports massive communication capability.

2.If predictive maintenance needs to be carried out by combining data such as equipment operating status, the equipment needs to be connected to a private network.

3.Edge servers are responsible for periodically processing data such as equipment operating status, and provide decision support for predictive maintenance work.

7.y.3	Service Flows

1. Edge servers monitor the operating parameters of equipment (such as production line robotic arms, warehouse conveyors, etc.). When potential fault risks (such as excessive bearing wear trends, abnormal motor vibration) are detected based on these sensing data, and then generate maintenance subtasks, publish them to the public task board, and label task characteristics (including fault warning level, required tool type, estimated maintenance duration, whether it falls within the scope of automated maintenance tasks, etc.).

2. For tasks within the scope of automated maintenance, the predictive maintenance results will be provided to the automated maintenance functional entity, and this functional entity issues and executes the tasks via the 6G network, such as pausing equipment operation. For tasks outside the scope of automated maintenance, the task information is sent to management staff.

7.y.4	Post-conditions

Equipment failures can be realized with minimal modifications owing to the immediate support from 6G system. In turn, equipment failures can be detected and resolved in advance. Services can continue to operate, reducing the time for production suspension and maintenance.

7.y.5	Existing features partly or fully covering the use case functionality

In TS 22.137 [6], there are requirements specified for 5G system on integration of sensing and communication.

Based on operator’s policies, operator’s control and regulation, the 5G system shall be able to collect 3GPP sensing data from sensing receivers for processing.

Subject to user consent, regulation, and operator’s policy, the 5G system should support the joint processing of the 3GPP sensing data and non-3GPP sensing data to derive a combined sensing result.

Subject to operator’s policy, the 5G network shall provide secure means for a trusted third-party to request 5G wireless sensing service based on specific parameters (e.g., refresh rate, period of time, sensing KPIs, geographical location) and to receive the corresponding sensing results.

In TS 22.137 [6], no specific requirement indicators are proposed. However, the requirement put forward in other use cases in TR 22.870 already cover those of this use case. Therefore, this use case only proposes specific requirement indicators for the predictive maintenance use case. However, the existing requirements don’t support the higher frequency bandwidth monitoring needed in industry and automation.

7.y.6	Potential New Requirements needed to support the use case

[PR 7.y.6-1] The 6G network shall be able to provide the sensing results for predictive maintenance by detecting the status information (e.g.: robotic arm’s location or the location of the robots in factory) of equipment in factory as described in the following Table 7.y.6-1:

Table 7.y.6-1:	Performance requirements of sensing results for Predictive Maintenance in smart factories

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refresh rate [s] | Missed detection [%] | False alarm [%] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Predictive maintenance | Indoor/ outdoor | 95 | £5 | £5 | N/A | N/A | £5 Note1 | N/A | 1000 Note2 | 1 | N/A |  | 5 |
| NOTE1: Range resolution of ≤5 m is coming from the fact that up to a 30 MHz of BW can be available in FR1 frequency range. Assuming frequency modulated continuous waveform, Δr = c/(2*BW) provides 5m of range resolution. If the 50 MHz of BW can be available, that the range resolution can be improved to 3m. Using larger BW in FR2 – FR4 will enable correspondingly better range resolutions. For example, the 400 MHz waveform in FR2 would enable 38 cm of range resolution. NOTE2: Sensing latency requirement of 1000 ms (so as the refresh rate) is not strict. The changes in the equipment are slowly over time. Also the status of manufacturing equipment is not expected to change rapidly. NOTE3: The discussed use case is based on the assumption that a monostatic FMCW implementation of S in ISAC is possible. If ISAC definition is clear (waveforms and hardware), then we need to know the “current scope of ISAC's considerations” to conclude if the use case is feasible. How much of system resources are available to sensing functionality (time, BW), what waveforms are possible, does hardware allow monostatic operation? Assuming monostatic FMCW, sensing a surface vibration with such a radar is a well-known concept essentially similar to range based interferometry. FMCW waveform is used to isolate the reflecting surface in range and then track phase vs time of that range bin to recover the surface vibration spectrum. For the concept to deliver highest possible accuracy, it would benefit from operating at highest available frequency band with largest available channel BW, and with a solid SNR of course. With that the vibration frequency accuracy can achieve sub-Hz resolution and micrometer level of accuracy for surface vibration amplitudes [1]. [1] L. Ding, M. Ali, S. Patole and A. Dabak, "Vibration parameter estimation using FMCW radar," 2016 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), Shanghai, China, 2016, pp. 2224-2228, doi: 10.1109/ICASSP.2016.7472072. |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *