3GPP TSG-SA WG1 Meeting #112 	S1-254244r1

17-21 November 2025, Dallas, USA	(revision of S1-254020, S1-254244)

Title:	Consolidation of AI Functional Requirements

Agenda Item:	8.1.10

Source:	Qualcomm

Contact:	Lola Awoniyi-Oteri (oawoniyi@qti.qualcomm.com)

# Abstract: This documents provides (with track changes) preliminary comments and updates to the Initial allocation of PRs in S1-254020, covering Clause Y.1.8 (AI). Further comments/updates may be provided in a later revision.

In r1, additional changes/updates were proposed in Table Y.1.8-5 and Table Y.1.8-6

# ============================ Text from S1-254020 =============================

# Y	Consolidated Potential Requirements

### Y.1.8	AI

Editor’s Note: This includes (but not limited to) PRs from clause 6.

NOTE:	The mention of AI capabilities such as AI Agent doesn’t imply or preclude any architecture assumption or solution.

Table Y.1.8-1 – General /AI requirements for 6G system

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.8-1-1 | Subject to operator policy and user preference, the 6G system shall be able to support mechanisms (e.g. AI capabilities) to predict UE behavior (e.g. based on device UE type, historical data, mobility patterns, etc.) for efficient resource allocation and planning. NOTE: This requirement may require collaboration of the network entities supporting AI capabilities. | PR 6.4.6-1 PR 6.4.6-2 | Proposed merged CPR on AI capabilities for prediction of UE behavior |
| CPR Y.1.8-1-1 - | Based on operator policy, the 6G system shall support AI capabilities. NOTE: Example of AI capabilities is the system ability to predict the UE behaviour (based on UE type, historical data, mobility patterns, etc.) and use that for the allocation and planning resources efficiently. | PR 6.4.6-1 | General, Prediction based on AI capabilities |
| CPR Y.1.8-1-2 - | Based on operator policy, the network entities supporting AI capabilities shall be able to collaborate upon request. | PR 6.4.6-2 | General Colloboration between network entities |
| CPR Y.1.8-1-23 | Based on operator policy and user consent, the 6G system shall be able to support mechanisms (e.g. AI capabilities in the network and UEs) allowing the network and UEs to negotiate communication parameters for a communication service. | PR 6.4.6-3 | AI capabilities for communication service provision |
| CPR Y.1.8-1-34 | The 6G network shall support the use of AI for the operations and management (OAM) of the 6G network, to assist with its energy efficiency and carbon emissions reduction. | PR 6.16.6-1 | energy |
| CPR Y.1.8-1-5 TBD | The 6G network shall be able to consider energy related information from energy management system, which is outside of 3GPP networks, to support mechanisms in order to effectively use the energy, e.g. efficiently use the supplied energy from the energy supplier. | PR 6.24.6-3 | energy |
| CPR Y.1.8-1-4 | Subject to user consent and operator policy, the 6G system shall be able to support mechanisms (e.g. AI capabilities such as AI agent) in the 6G system to enable real-time call quality analytics and dynamic optimizations. | PR 6.54.6-1 | AI capabilities for quality optimizations |
| CPR Y.1.8-1-5 | Subject to user consent, and operator policy, the 6G network shall be able to receive and analyse aggregated call quality data, and apply enhancements using mechanisms (e.g. AI capabilities such as AI agent) in the 6G network. | PR 6.54.6-2 | AI capabilities for quality optimizations |
| CPR Y.1.8-1-6 | Subject to user consent and operator policy, the 6G system shall be able to support continuous enhancement of mechanisms (e.g. AI capabilities such as AI agent) in 6G network, from user experience feedback, to improve policies. | PR 6.54.6-3 | AI capabilities for continuous quality optimizations and enhancements |
| CPR Y.1.8-1-7 | Subject to operator policy and user consent, the 6G network shall be able to optimize the user experience by actively informing UE about availability of related network services based on information such as device type, device location, mobility behaviour, connection status, subscription information, etc. | PR 6.45.6-1 | UE service awareness |
| CPR Y.1.8-1-8 | Subject to operator policy and user consent, the 6G system shall support dynamic configuration of UEs and the associated 3GPP services (e.g., communication service, sensing service, computing service, AI service) to adapt to varying conditions (e.g., network conditions, application needs, resource availability) to provide optimal service operation. NOTE: The optimal service operation aims to deliver 3GPP network services efficiently and effectively, maximizing user satisfaction while minimizing resource utilization. | PR 6.45.6-2 | Dynamic configuration for service delivery |

Table Y.1.8-1 – General AI requirements for 6G system

Table Y.1.8-2 – Network AI Agent

Editor’s Note: This includes requirements related to network AI.

NOTE:	The mention of AI capabilities such as AI Agent doesn’t imply or preclude any architecture assumption or solution.

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.8-2-1 | Subject to operator’s policy and user preference, the 6G network shall support mechanism, e.g. AI capabilities such as AI Agent, to provide suitable 3GPP service or combination of multiple 3GPP service(s) in response to received intent(s) service request (expressed via intent or not) (e.g. from subscriber/users, an authorized third party or a third-party AI Agent). NOTE 1: The mention of AI capabilities such as AI agent doesn’t imply or preclude any architecture assumption or solutions. NOTE 1: The services may be provided on-demand at a given time and location. NOTE 2: During service provision network-related information, information from trusted third parties or user-related information (such as user mobility context and subscription information) may be considered. | PR 6.6.6-4 PR 6.20.6-1 PR 6.21.6-2 PR 6.32.6-1 PR 6.43.6-1 PR 6.51.6-1 PR 6.6.6-2 PR 6.44.6-2 PR 6.43.6-2 | Proposed merged CPR on Network AI agent, Service providesion service based on service request expressed via intent or not |
| - | Based on operator policy, the 6G network shall support mechanisms (e.g. AI capabilities such as AI Agent) to provide multiple 3GPP services, in response to received intent(s) (e.g. from a third-party AI Agent). | PR 6.6.6-4 | Network AI agent, Service providesion service based on intent |
| - | Subject to operator’s policy, the 6G system shall be able to support mechanism to provide 3GPP service to applications on one or multiple UEs belonging to a user, based on the received intent from the user. | PR 6.20.6-1 | Network AI agent Service providesion to multiple UEs service based on intent |
| - | The 6G network shall provide 3GPP services (including communication, sensing and computing) with QoS assurance based on intent received (e.g. from subscribers). | PR 6.21.6-2 | Network AI agent, provide service based on intent |
| - | The 6G network shall support mechanism, e.g. AI capabilities such as AI Agent, to provide suitable 3GPP service or combination of multiple 3GPP services to subscribers requested by received intent from the user. NOTE: The mention of AI capabilities such as AI agent doesn’t imply or preclude any architecture assumption or solutions. | PR 6.32.6-1 | Network AI agent, providesion of multiple de 3GPP services based on intent |
| - | Subject to operator’s policy, the 6G network shall be able to provide means for the authorized third party to request 3GPP service by intent. | PR 6.43.6-1 | Network AI agent, provide service request from 3rd party based on intent |
| - | Based on operator policy, the 6G system shall support mechanisms (e.g. AI capabilities such as AI agent) in the 6G network to provide 3GPP/6G services, which includes coordination of multiple 6G services (e.g. communication, sensing, AI service). | PR 6.51.6-1 | Network AI agent, provide Coordination of multiple services |
| CPR Y.1.8-2-2- | Based on operator policy and user consent preference, the 6G network shall be able to take into account information related to user mobility context, subscription information when invoking 3GPP services based on received intent(s) from the user. | PR 6.6.6-2 | Network AI agent, Service provision using user/ subscriber info. |
| CPR Y.1.8-2-3- | Based on operator policy and user consent, the 6G network shall support mechanisms (e.g. AI capabilities such as Al agent) to provide 3GPP services on demand based on the received intent(s) from user by taking into account of network-related information and information from trusted third-party. | PR 6.44.6-2 | Network AI agent, Service provision based on intent using network-related or third party info |
| CPR Y.1.8-2-4- | Subject to operator’s policy, the 6G network shall be able to support mechanisms (e.g. AI capabilities such as AI agent) to provide the on-demand 3GPP service at a given time and location area based on the authorized third party’s request by intent. | PR 6.43.6-2 | Network AI agent, Service provision based on intent at a given location and time |
| CPR Y.1.8-2-5 | Based on operator policy and user consent, the 6G network shall support mechanisms (e.g. AI capabilities such as AI Agent) to send intent(s) related to 3GPP services towards a third-party AI Agent (e.g. proactively or in response to a received intent), also taking into account information related to user mobility context, subscription information. | PR 6.6.6-3 | Network AI agent, intent |
| CPR Y.1.8-2-26 | Subject to Based on operators’ policy and user consents preference, the 6G system shall support mechanisms (e.g. AI capabilities such as AI agent) to translate intent received (e.g. from subscribers) into request(s) for service(s) and associated service performance requirements. NOTE 2: AI capabilities such as AI Agent can, for example, accept intent received from the user, translate it into network requirements, and activate the corresponding 3GPP services (e.g. communication service, sensing service, AI services) with QoS guarantee when they are being consumed. | PR 6.21.6-1 | Network AI agent, Iintent translation |
| CPR Y.1.8-2-3,5 | Based on operator policy and user consent preference, the 6G network shall support mechanisms (e.g. AI capabilities such as AI Agent) to send intent(s) related to 3GPP services towards a third-party AI Agent (e.g. proactively or in response to a received intent), also taking into account information related to user or network information from an authorized third party. mobility context, subscription information. | PR 6.6.6-3 | Network AI agent, Iintent transmission |
| CPR Y.1.8-2-4 | Subject to operator policy and user preference, the 6G network shall be able to provide mechanisms to provide QoS assurance for 3GPP service(s) provided based on service requests (e.g. expressed via intent) from subscriber/users or an authorized third party or a third-party AI agent. | [PR 6.50.6-2] [PR 6.21.6-2] [PR 6.21.6-4] | Proposed merged CPR on QoS assurance for 3GPP services requested with or without intent |
| - | Subject to operator policy, the 6G network shall be able to support a mechanism to guarantee the user experience when providing combined 3GPP service (e.g., combines 6G AI service and communication service). | [PR 6.50.6-2] | Performance Guarantee for requested 3GPP services |
| - | The 6G network shall provide 3GPP services (including communication, sensing and computing) with QoS assurance based on intent received (e.g. from subscribers). | PR 6.21.6-2 | Performance/QoS Assurance for request based on intent |
| - | The 6G network shall support a mechanism (e.g. AI capabilities such as AI agent) to provide mechanisms to monitor and evaluate the quality of the provided 3GPP service and perform adaptations if needed based on the evaluation. | [PR 6.21.6-4] | Performance monitoring and evaluation |
| CPR Y.1.8-2-57 | Subject to operator policy and user preference, the 6G system shall support a mechanism to identify and associate the source (e.g. home robot, or specific AI Application (e.g. 3rd AI Agent)) of the intent with a subscriber/user and with the actions resulting from the handling/processing of the intent. | PR 6.21.6-3 | Network AI agent, intent Intent identification and association with source |
| CPR Y.1.8-2-68 | The 6G network shall support mechanisms (e.g. Al capabilities such as Al agent) to authorize the received intent(s) from user. | PR 6.44.6-1 | Network AI agent, Intent authorization |
| CPR Y.1.8-2-9 | The 6G network shall support a mechanism (e.g. AI capabilities such as AI agent) to provide mechanisms to monitor and evaluate the quality of the provided 3GPP service and perform adaptations if needed based on the evaluation. | PR 6.21.6-4 | Network AI agent, quality optimizations |
| CPR Y.1.8-2-10 | Subject to user consent and operator policy, the 6G system shall be able to support mechanisms (e.g. AI capabilities such as AI agent) in the 6G system to enable real-time call quality analytics and dynamic optimizations. | PR 6.54.6-1 | Network AI agent, quality optimizations |
| CPR Y.1.8-2-11 | Subject to user consent, and operator policy, the 6G network shall be able to receive and analyse aggregated call quality data, and apply enhancements using mechanisms (e.g. AI capabilities such as AI agent) in the 6G network. | PR 6.54.6-2 | Network AI agent, quality optimizations |
| CPR Y.1.8-2-12 | Subject to user consent and operator policy, the 6G system shall be able to support continuous enhancement of mechanisms (e.g. AI capabilities such as AI agent) in 6G network, from user experience feedback, to improve policies. | PR 6.54.6-3 | Network AI agent, quality optimizations |

Table Y.1.8-3 – 3rd party AI Agent

Editor’s Note: This includes requirements related to 3rd party AI agent.

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.8-3-1 | Based on operators’ policy and user consent, the 6G system shall support hosting of AI applications (e.g. 3rd party AI agent applications) managed and controlled by the 6G network and/or multiple AI applications (e.g. 3rd AI agent applications) on a UE. | PR 6.9.6-1 | Hosting AI applications (e.g. 3rd party AI agent), General |
| CPR Y.1.8-3-1 TBD | Based on regulatory requirements, operators’ policy and user consent, 6G network shall support trusted network access for 3rd party AI agent and support a mechanism to expose 3rd party AI agent’s attributes (e.g. related users, sensing capabilities, AI capabilities, service features) to other 3rd party AI agents. | PR 6.7.6-1 PR 6.23.6-3 | Proposed merged CPR on 3rd party AI agent, exposure between AI agent |
| - | Based on regulatory requirements, operators’ policy and user consent, 6G network shall support trusted network access for 3rd party AI agent and support a mechanism to expose 3rd party AI agent’s attributes (e.g. related users, sensing capabilities, AI capabilities, service features) to other 3rd party AI agents. | PR 6.7.6-1 | 3rd party AI agent, access, exposure between AI agent |
| - | Based on user consent and operator's policy, the 6G network shall support a secure mechanism to expose information of AI application (e.g. AI agent application) on UE (e.g. related user) to AI application (e.g. AI agent application) on other UE. | PR 6.23.6-3 | 3rd party AI agent, exposure between AI agent |
| CPR Y.1.8-3-2 | Based on regulatory requirements, user consent, operators’ policy and agreement with authorized 3rd party, the 6G network shall be able to support mechanisms to enable secure security and dynamic identification for 3rd party AI agents provided by authorized 3rd party associated with a user (e.g. AI agents belonging to a customer). NOTE: Dynamic means the identification of this AI agent can be temporary assigned based on task, and this identification can be assigned by different operators based on different tasks. | PR 6.7.6-2 PR 6.55.6-1 | Proposed merged CPR on identification for 3rd party AI agent |
| - | Based on regulatory requirements, user consent, operators’ policy and agreement with authorized 3rd party, the 6G network shall be able to support security identification for 3rd party AI agents provided by authorized 3rd party associated with a user (e.g. AI agents belonging to a customer). | PR 6.7.6-2 | 3rd party AI agent, identification |
| - | Based on regulatory requirements and operators’ policy, 6G network shall support dynamic identification of 3rd party AI agents. NOTE: Dynamic means the identification of this AI agent can be temporary assigned based on task, and this identification can be assigned by different operators based on different tasks. | PR 6.55.6-1 | 3rd party AI agent, identification |
| CPR Y.1.8-3-3 | Based on regulatory requirements, operators’ policy and user consent preference, 6G network shall support secure discovery mechanisms for authorized AI applications (e.g. 3rd party AI agents) on a UE 3rd party AI agents to provide/register their attributes (e.g. sensing capabilities, AI capabilities, service features, associated authorized users) to 6G network, and to discover other authorized 3rd party AI applications (e.g. 3rd party AI agents). to achieve collaborative task. | PR 6.7.6-3 PR 6.46.6-1 | Proposed merged CPR on 3rd party AI agent, register and discover |
| - | Based on regulatory requirements, operators’ policy and user consent, 6G network shall support mechanisms for 3rd party AI agents to provide/register their attributes (e.g. sensing capabilities, AI capabilities, service features, associated authorized users) to 6G network, and discover other authorized 3rd party AI agents to achieve collaborative task. | PR 6.7.6-3 | 3rd party AI agent, register and discover, collaboration |
| - | Based on the user consent and operator’s policy, the 6G network shall be able to support the discovery of 3rd party AI agent (application) on the UE. | PR 6.46.6-1 | discovery of 3rd party AI agent, |
| CPR Y.1.8-3-4 | Based on operator's policy and user preference, the 6G network shall support a mechanism to manage the information of AI application (e.g. 3rd party AI agent application) on the UE (e.g. related user). | PR 6.23.6-1 | 3rd party AI application agent, Information management |
| CPR Y.1.8-3-5 TBD | Based on user consent and operator's policy, the 6G system shall support secure interoperability and efficient mechanism between AI applications (e.g. 3rd party AI agents) on multiple UEs and between AI Agents and applications to achieve a collaborative task. NOTE 1: Interoperability between AI agents refers to the ability to discover, authenticate and authorize AI Agents to communicate, exchange data, and work together seamlessly. NOTE 2: Collaborative task refers to an activity, action requiring the involvement of two or more AI agents. | PR 6.9.6-2 PR 6.23.6-4 PR 6.30.6-2 PR 6.40.6-1 | Proposed merged CPR on 3rd party AI agent, collaboration |
| - | The 6G system shall support secure interoperability between AI Agents and between AI Agents and applications to achieve a collaborative task. NOTE 1: Interoperability between AI agents refers to the ability to discover, authenticate and authorize AI Agents to communicate, exchange data, and work together seamlessly. NOTE 2: Collaborative task refers to an activity, action requiring the involvement of two or more AI agents. | PR 6.9.6-2 | 3rd party AI agent, collaboration |
| - | Based on user consent and operator's policy, the 6G network shall support a secure mechanism to provide communication service between AI applications (e.g., AI agent applications) on multiple UEs for a collaborative task. | PR 6.23.6-4 | 3rd party AI agent, collaboration |
| - | Subject to operator’s policy, the 6G network shall be able to provide AI service to enable collaborative task for AI applications running on multiple UEs. | PR 6.30.6-2 | 3rd party AI agent, collaboration tasks by AI applications |
| - | Subject to user consent and operator’s policy, the 6G network shall provide efficient mechanisms to support the collaboration of UEs. | PR 6.40.6-1 | 3rd party AI agent, collaboration |
| CPR Y.1.8-3-56 | Based on regulatory requirements and operators’ policy and user preference, the 6G network shall provide means to support efficient and secure communication (including multi-modality exchange) between multiple AI applications (e.g. 3rd party AI agents) on UEs over a target area considering data characteristics. NOTE: This requirement can apply to 3rd party AI agents of same users or different users. It is expected that the required communication service would be provisioned in the range of minutes to days, depending on use case. Lower for temporary task and higher for long term task. | PR 6.7.6-4 PR 6.8.6-3 PR 6.23.6-4 | Proposed merged CPR on Communication between 3rd party AI Applications (e.g. AI agent,) communication |
| - | Based on regulatory requirements and operators’ policy, the 6G network shall provide means to support efficient and secure communication (including multi-modality exchange) between multiple 3rd party AI agents on UEs over a target area. NOTE: This requirement can apply to 3rd party AI agents of same users or different users. It is expected that the required communication service would be provisioned in the range of minutes to days, depending on use case. Lower for temporary task and higher for long term task. | PR 6.7.6-4 | Communication between 3rd party AI agent on UE, communication |
| - | Based on user consent, operator policy and regulatory requirements, the 6G network shall be able to provide a suitable means to support the multi-modal data exchange between the AI agent applications considering data characteristics. | PR 6.8.6-3 | Communication between 3rd party AI agent applications communication |
| - | Based on user consent and operator's policy, the 6G network shall support a secure mechanism to provide communication service between AI applications (e.g., AI agent applications) on multiple UEs for a collaborative task. | PR 6.23.6-4 | Communication between AI applications on UE |
| CPR Y.1.8-3-7 | The 6G system shall support hosting of large amounts of AI agent applications managed and controlled by the 6G network and/or multiple AI Agent applications on a UE. | PR 6.9.6-1 | 3rd party AI agent, general |
| CPR Y.1.8-3-8 TBD | Based on operator policy, the 6G network shall be able to support secure means to expose its different services (e.g. computing offloading service in Service Hosting Environment) to the authorised third-party AI agent based on its intent. | PR 6.6.6-1 PR 6.7.6-5 | Proposed merged CPR on 3rd party AI agent, service exposure |
| - | Based on operator policy, the 6G network shall be able to support secure means to expose its services to the authorised third-party AI agent based on its intent. | PR 6.6.6-1 | 3rd party AI agent, service exposure |
| - | Based on operator policy, the 6G network shall be able to support secure means to expose different services, e.g. computing offloading service in Service Hosting Environment, to the authorized third-party AI agent. | PR 6.7.6-5 | 3rd party AI agent, service exposure |
| CPR Y.1.8-3-69 | Based on user consent and operator policy, the 6G system shall provide a suitable means for an AI applications (e.g. 3rd party agent) application on UE to invoke some 3GPP services upon request (e.g. IMS service, AI service). | PR 6.8.6-1 PR 6.14.6-2 PR 6.30.6-1 | Proposed merged CPR on 3rd party AI agent, service invokeing by AI applications on UE |
| - | Based on user consent and operator policy, the 6G system shall provide a suitable means for an AI agent application on UE to invoke some 3GPP services (e.g. IMS service). | PR 6.8.6-1 | 3rd party AI agent, service invokeing by 3rd party AI agent |
| - | Subject to operator’s policy, the 6G network shall support mechanisms for a 3rd party AI-based application on UE (e.g. UAV) to invoke an AI service upon request. | PR 6.14.6-2 | 3rd party AI agent, service invokeing by AI-based application on UE |
| - | Subject to operator’s policy and user consent, the 6G network shall be able to support mechanism for AI application on UE to invoke AI services provided by 6G network. | PR 6.30.6-1 | 3rd party AI agent, service invokeing by AI application on UE |
| CPR Y.1.8-3-10 TBD | The 6G network shall be able to provide a suitable means to allocate network resources (e.g., network slice) to a group of trusted third parties (e.g., applications running on multiple UEs/robots or third party AI agents), based on its availability and capability, considering dynamic changes of traffic demand and QoS characteristics. | PR 6.52.6-2 | 3rd party AI agent, resource |
| CPR Y.1.8-3-711 | Based on user preference and operator policy, tThe 6G network shall support a mechanism to authenticate and authorize AI application (e.g. 3rd party AI agent) on the UE. | PR 6.23.6-2 PR 6.41.6-1 PR 6.41.6-2 | Proposed merged CPR on 3rd party AI agent, authentication and authorization for AI application (e.g. 3rd party AI agent) |
| - | The 6G network shall support a mechanism to authenticate and authorize 3rd party AI agent. | PR 6.41.6-1 | authentication and authorization for 3rd party AI agent, |
| - | The 6G network shall support a mechanism to authorize AI application (e.g. AI agent application) on UE to invoke 3GPP services. | PR 6.23.6-2 | service invoke, authorization for AI application |
| - | Based on the operator's policy, the 6G network shall support a secure mechanism for authenticated and authorized 3rd party AI agent to invoke 3GPP services. | PR 6.41.6-2 | service invoke, authentication and authorization for 3rd party AI agent, |
| CPR Y.1.8-3-12 TBD | Based on user consent, operator policy and regulatory requirements, the 6G system shall provide an efficient way to expose information (e.g. change of QoS) to authorized 3rd party AI agents (e.g. by means suitable for prompt augmentation). NOTE: Prompt augmentation is an approach of adding further information, or instructions, to a prompt in order to enhance the AI-generated response, e.g. in terms of quality or relevance. | PR 6.8.6-2 PR 6.13.6-1 | Proposed merged CPR on 3rd party AI agent, exposure to AI agent |
| - | Based on user consent, operator policy and regulatory requirements, the 6G system shall provide an efficient way to expose information (e.g. change of QoS) to the application on the UE. | PR 6.8.6-2 | 3rd party AI agent, exposure to AI agent |
| - | Subject to operator’s policy, the 6G network shall be able to expose information from the network to authorized 3rd party AI agents by means suitable for prompt augmentation. NOTE: Prompt augmentation is an approach of adding further information, or instructions, to a prompt in order to enhance the AI-generated response, e.g. in terms of quality or relevance. | PR 6.13.6-1 | 3rd party AI agent, exposure to AI agent |
| CPR Y.1.8-3-13 | Based on the user consent and operator’s policy, the 6G system shall be able to support means for the network to invoke the 3rd party AI agent (application) on the UE. | PR 6.46.6-2 | 3rd party AI agent, network invoke AI agent |
| CPR Y.1.8-3-14 TBD | Subject to operator’s policy, regulatory requirements and user consent, the 6G system shall support a mechanism for a user-authorized AI application (e.g. AI agent) in the Service Hosting Environment to autonomously initiate communication with emergency services on behalf of the user, i.e., the communication session is associated with the user's identity and location. | PR 6.47.6-1 | 3rd party AI agent, emergency service |
| CPR Y.1.8-3-15 TBD | Subject to operator’s policy, regulatory requirements and user consent, the 6G network shall support a mechanism to log the data (e.g., sensor data, context information) used by an AI application (e.g., AI agent) in the Service Hosting Environment, which autonomously decided to initiate communication with emergency services on behalf of the user. | PR 6.47.6-2 | 3rd party AI agent, emergency service |
| CPR Y.1.8-3-16 TBD | Subject to operator’s policy, regulatory requirements and user consent, the 6G system shall support a mechanism for a user to provide policies which define the autonomous actions that can be taken by their authorized AI applications (e.g. AI agents) in the Service Hosting Environment. NOTE: For example, a user could provide a policy authorizing their AI agent to share location data with a pre-defined family member. A separate policy could authorize their AI agent to initiate communication with emergency services on the user's behalf when the agent identifies a critical safety event | PR 6.47.6-3 | 3rd party AI agent, emergency service |

Table Y.1.8-4 – IMS based AI services / AI related IMS services

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.8-4-1 | Subject to operator policy and user’s consent, 6G system network (e.g. specifically core network or service hosting environment) shall be able to enable the IMS services ) shall be able to provide intelligent communication assistant service to users. NOTE: Intelligent Communication Assistant is the virtual intelligent communication assistant locates in operator network and interacts with the users through voice, video, text, gestures or other modalities. The assistant can be customized for each user by accessing user data and network data which are stored or collected in the network, with user’s consent. It can provide various communication services and support individual users based on user’s intention and requirement utilizing AI capability. One subscriber can have one or more Intelligent Communication Assistants. | PR 6.11.6-1 PR 6.17.6-1 | Proposed merged CPR on intelligent communication assistant service, general |
| - | Subject to operator policy and user’s consent, 6G network shall be able to enable the IMS services to provide intelligent communication assistant service to users. | PR 6.11.6-1 | intelligent communication assistant service, general |
| - | Subject to regulatory requirements and user consent, the 6G system shall provide suitable means for 6G network (e.g. specifically core network or service hosting environment) to provide AI services (e.g. the Intelligent Communication Assistant services) to the subscribers. | PR 6.17.6-1 | intelligent communication assistant service, general |
| CPR Y.1.8-4-2 | Subject to operator’s policy and user preference, 6G network shall be able to enable IMS services to support the communication between different (i.e. operator native or 3rd party) intelligent communication assistants, e.g. during an IMS calling service, both calling and callee parties are using intelligent communication services. | PR 6.11.6-3 PR 6.11.6-4 | Proposed merged CPR on intelligent communication assistant service, communication between assistants |
| CPR Y.1.8-4-2 | Subject to operator’s policy and user’s consent, 6G network shall be able to enable the IMS services to support the interaction between different intelligent communication assistants, e.g. during an IMS calling service, both calling and callee parties are using intelligent communication services. | PR 6.11.6-3 | intelligent communication assistant service, interaction between assistants |
| CPR Y.1.8-4-3 | Subject to operator’s policy and user’s consent, 6G network shall be able to support the communication between operator’s intelligent communication assistant and authorized third-party intelligent communication assistant during IMS communication between users utilizing the intelligent communication assistant. NOTE 1: It is not expected that intelligent communication assistants directly communicate with each other via IMS services. | PR 6.11.6-4 | intelligent communication assistant service, communication between native assistant and 3rd party assistant |
| CPR Y.1.8-4-3 | Subject to operator’s policy and user preference, the 6G network shall be able to enable the IMS services to support the intelligent communication assistant in invoking operator’s native capabilities or authorized third-party capabilities in order to meet user service requirements. NOTE 1: Examples of operator native capabilities include AR rendering, XR rendering in service hosting environment, SMS or voice, trigger QoS adjustment, Sensing. NOTE 2: Examples of 3rd party capabilities include weather inquiry, the language translation, the takeout services, etc | PR 6.11.6-5 PR 6.11.6-6 | Proposed merged CPR on intelligent communication assistant service, invoking operator native or 3rd party capabilites |
| CPR Y.1.8-4-4 - | Subject to operator’s policy and user’s consent, the 6G network shall be able to enable the IMS services to support the intelligent communication assistant to invoke operator’s native capabilities (e.g. AR rendering, XR rendering in service hosting environment, SMS or voice, trigger QoS adjustment, Sensing) to meet user service requirements dynamically). | PR 6.11.6-5 | intelligent communication assistant service, invoking 3GPP service operator capabilities |
| CPR Y.1.8-4-5 - | Subject to operator’s policy and user’s consent, the 6G network shall be able to enable the IMS services to support invoking authorized third-party capabilities (e.g. the weather inquiry, the language translation, the takeout services) and/or obtain various information from authorized third-party. NOTE 2: Intelligent Communication Assistant: The virtual intelligent communication assistant locates in operator network and interacts with the users through voice, video, text, gestures or other modalities. The assistant can be customized for each particular user by accessing user data and network data which are stored or collected in the network, with user’s consent. It can provide various communication services and support individual users based on user’s intention and requirement utilizing AI capability. One subscriber can have one or more Intelligent Communication Assistants. | PR 6.11.6-6 | intelligent communication assistant service, invoking 3rd party capabilities |
| CPR Y.1.8-4-6 | Subject to operator’s policy and user preference, the 6G system shall provide mechanism to collect the user consents for the secure exchanges of personal data with the Intelligent communication assistant inside the network or (third party) service hosting environment. | PR 6.17.6-2 | intelligent communication assistant service, user data security |
| CPR Y.1.8-4-7 | Subject to operator’s policy and user preference, Tthe 6G system shall provide mechanisms for registration and discovery of AI-based services provided by authorized 3rd party service providers (e.g. 3rd party providers of Intelligent Communication Assistant service). | PR 6.17.6-3 | intelligent communication assistant, registration and discovery of 3rd party service provider |
| CPR Y.1.8-4-8 | Subject to operator policy and user preference user’s consent, 6G network (e.g. in conjunction to IMS) shall be able to enable IMS services to provide intelligent calling service to users in two parties or multi-party call, when the user is unavailable or unwilling to take the calls, e.g. provide AI-based answering service with usage of AI capability in case of the user’s phone is in flight mode, powered-off or during busy time. | PR 6.22.6-1 | intelligent calling service, general |
| CPR Y.1.8-4-9 | Subject to regulatory requirements, operator policy and user preference user’s consent, the 6G network (e.g. in conjunction to IMS) shall be able to enable IMS services to support intelligent calling customization, e.g. the call-answering tone, speech rate customization based on user’s voice, the response can be customized based on different caller. | PR 6.22.6-3 | intelligent calling service, customization |
| CPR Y.1.8-4-10 | Subject to operator policy and user’s consent, the 6G network (e.g. in conjunction to IMS) shall be able to enable IMS services to support provideing the user with information related to the call (e.g. send the conversation record or summary to users after the intelligent calling service call has ended by SMS or voice mail). | PR 6.22.6-4 | intelligent calling service, call records |
| CPR Y.1.8-4-11 | Subject to operator policy, user preference and regulatory requirements, when an intelligent calling service is used on behalf of a user/subscriber, the 6G network (e.g. in conjunction with IMS) shall be able to enable IMS services to identify and associate the specific intelligent AI-based calling service being used and the user/subscriber on whose behalf the service is being used. | PR 6.22.6-5 | intelligent calling service, identification and association |
| CPR Y.1.8-4-12 | Based on operator policy and user consent preference, the 6G system should enable the IMS services enhanced by AI capability, to enhance the audio and video stream for disability support. NOTE 1: The disability support can be e.g. describing and explaining the surroundings, audio aid (modification of the audio stream i.e., enhancements, clarifications, translations, speech to text, text to speech, etc), visual aid (modification of the visual stream). | PR 6.38.6-1 | intelligent disability support service, IMS enhancement for communication service |
| CPR Y.1.8-4-13 | Based on operator policy, user preference consent and regulatory requirements, the 6G system may be able to enhance the IMS emergency communication service with AI capability, to enhance the audio and video stream for disability support. | PR 6.38.6-2 | intelligent disability support service,IMS enhancement for emergency communication service |
| CPR Y.1.8-4-14 | Subject to operator policy and user preference user’s consent, the 6G system (including IMS) shall be able to enable IMS services to support media transformations in multi-modal communication services, including image to video, 2D video to 3D video/avatar media, text to video and vice-versa. NOTE 1: The media transformations could be provided via AI capabilities of 3rd party or 6G network (including IMS). NOTE 2: For multi-modal communication service, refer to [14]. | PR 6.42.6-1 | media transformations |

Table Y.1.8-5 – AI model training and inferencing

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.8-5-1 | Subject to operator policy and user preferences the 6G network shall be able to provide AI service (e.g. AI/ML model training and inference in the SHE and AI model management) to subscriber/user (via a UE) or authorized third parties. | PR 6.10.6-1 PR 6.25.6-1 PR 6.34.6-1 | AI Services, General |
| CPR Y.1.8-5-1 - | Subject to operator’s policy, the 6G network shall be able to provide AI service (e.g. AI model inference) to a UE. | PR 6.10.6-1 | Support for AI Service |
| CPR Y.1.8-5-2 - | Subject to operator’s policy, the 6G network shall be able to provide AI/ML model training and inference within service hosting environment, and AI model management (e.g. model monitoring, retraining, validation) for authorized third parties. | PR 6.25.6-1 | Support for AI/ML model training and inference and AI model |
| CPR Y.1.8-5-3 | Subject to operator policy and user consent, the 6G network shall be able to authorize a user to offload task from a 3rd party application (e.g. a AI inference workload) to the Service Hosting Environment. | PR 6.34.6-1 | Support for offloading AI task from 3rd party application AI services, general |
| CPR Y.1.8-5-4 | Subject to operator’s policy, the 6G network shall provide a means to expose the communication QoS information (e.g. latency) between a given Service Hosting Environment and a UE to an authorized 3rd party for providing AI services to the application on the UE. | PR 6.18.6-1 | AI services, exposure |
| CPR Y.1.8-5-2 | Subject to operator’s policy and user preference, the 6G network shall be able to securely expose the result of AI service (e.g. AI analysis or prediction, AI model or AI inference, etc) to user/subscriber via application on UE(s) or authorized third parties. | PR 6.35.6-1 PR 6.51.6-4 | Service Exposure of AI service |
| CPR Y.1.8-5-5 - | Subject to regulatory requirements, operator’s policy and user-consent, 6G network shall provide the result of AI task (e.g. AI analysis or prediction) to certain application on UE(s) or authorized third parties in certain area. | PR 6.35.6-1 | AI task results exposure |
| - | Subject to operator policy, the 6G network shall be able to provide and expose AI inference service to the user/subscriber, and ensure required joint (inference, communication) latency. | PR 6.51.6-4 | AI inference Performance service exposure |
| CPR Y.1.8-5-6 TBD | Based on the request from authorized 3rd party and user consent, the 6G network shall be able to support secure coordination with one or more UEs in order to fulfill an AI service request for a user. NOTE: It is assumed that the UEs belong to the requesting user. | PR 6.29.6-1 | AI service, secure |
| CPR Y.1.8-5-37 | Based on operator's policy and agreement with 3rd party, the 6G network shall support monitoring energy consumption for an AI service (e.g. inference) requested by 3rd party | PR 6.37.6-1 | AI services, energy consumption |
| CPR Y.1.8-5-48 | Based on operator's policy and agreement with 3rd party, the 6G network shall support exposing energy consumption information of an AI service to 3rd party. | PR 6.37.6-2 | AI services, energy consumption, exposure |
| CPR Y.1.8-5-9 | Subject to operator policy and agreement with the third party, 6G network shall support means for an authorized third party to provide information about expected communication performance requirement and AI-related service requirements (e.g. related to AI inference accuracy and latency), for the 6G network to provide data transfer accordingly for the application to enhance user experience. NOTE: For example, for AI inference application based on video, such information can refer to max allowed packet error rate and average packet error rate on a per video frame basis. The max allowed packet error rate characterizes the upper bound of packer error rate of each frame to achieve successful source decoding. The average packet error rate is averaged over a number of video frames related to providing acceptable user experience. | PR 6.28.6-1 PR 6.39.6-1 | Proposed merged CPR on AI service, performance requirements |
| - | Subject to operator policy and agreement with the third party, 6G network shall support means for an authorized third party to provide information about expected communication performance requirement, for the 6G network to provide data transfer accordingly for the application to enhance user experience. NOTE: For example, for AI inference application based on video, such information can refer to max allowed packet error rate and average packet error rate on a per video frame basis. The max allowed packet error rate characterizes the upper bound of packer error rate of each frame to achieve successful source decoding. The average packet error rate is averaged over a number of video frames related to providing acceptable user experience. | PR 6.28.6-1 | performance |
| CPR Y.1.8-5-5 | Subject to the operator’s policy and regulatory requirements, the 6G network shall be able to receive AI-related service requirements (e.g. AI inference accuracy and latency) from a service consumer (e.g. 3rd party) as part of a request for an AI service., for example related to AI inference accuracy and latency. | PR 6.39.6-1 | AI service performance requirements reception |
| CPR Y.1.8-5-69 | Subject to operator’s policy, the 6G system shall be able to support negotiation of the service performance (e.g. latency, inference accuracy), between UE and 6G network, when providing AI service (e.g. AI model inference). | PR 6.10.6-2 | AI service performance requirements negotiation |
| CPR Y.1.8-5-710 | Subject to operator’s policy, the 6G network shall be able to support mechanism(s) to ensure a guaranteed the service performance (e.g. joint inference and communication) latency and inference accuracy) when providing combined 3GPP service (e.g., combines 6G AI service (e.g. AI inference service) and communication service, distributed AI inference service). NOTE: The UE is not involved in distributed AI inference service. | PR 6.10.6-3 PR 6.10.6-5 PR 6.50.6-2 PR 6.51.6-4 | Proposed merged CPR on AI service, Performance guarantee |
| - | Subject to operator’s policy, the 6G network shall be able to support mechanism to guarantee the service performance (e.g. latency, inference accuracy) when providing AI service (e.g. AI model inference). | PR 6.10.6-3 | AI service, Performance quarantees for AI service |
| - | Subject to operator policy, the 6G network shall support mechanisms to ensure a guaranteed service performance of the distributed AI inference service. NOTE: The UE is not involved in distributed AI inference service. | PR 6.10.6-5 | AI inference, Performance quarantees for AI inference |
| - | Subject to operator policy, the 6G network shall be able to provide and expose AI inference service to the user/subscriber, and ensure required joint (inference, communication) latency. | PR 6.51.6-4 | AI inference, Performance |
| - | Subject to operator policy, the 6G network shall be able to support a mechanism to guarantee the user experience when providing combined 3GPP service (e.g., combines 6G AI service and communication service). | PR 6.50.6-2 | AI service, performance |
| CPR Y.1.8-5-811 | The 6G network shall be able to provide mechanisms (e.g. by interacting with NDT) to ensure the reliability and the validity of the AI service inference results (e.g. by verifying decisions made by the network). | PR 6.32.6-2 | AI service performance guarantee using NDT inference, Performance |
| CPR Y.1.8-5-9 | Subject to operator policy, the 6G network shall be able to monitor AI service performance (e.g., AI model inference accuracy) and expose the service performance (e.g. the communication QoS information such as between a given Service Hosting Environment and a UE) to an authorized 3rd party. | PR 6.50.6-3 PR 6.18.6-1 | AI service performance monitoring and exposure |
| CPR Y.1.8-5-3 - | Subject to operator policy, the 6G network shall be able to monitor the performance (e.g., AI model inference accuracy) and report them to the 3rd party. | PR 6.50.6-3 | AI service, performance monitoring |
| CPR Y.1.8-5-4 - | Subject to operator’s policy, the 6G network shall provide a means to expose the communication QoS information (e.g. latency) between a given Service Hosting Environment and a UE to an authorized 3rd party for providing AI services to the application on the UE. | PR 6.18.6-1 | AI services, exposure |
| CPR Y.1.8-5-12 TBD | Subject to operator policy, the 6G network shall be able to manage and coordinate various network operations (e.g. AI model training/selection, computing resource selection, communication performance monitoring) upon receiving a request (e.g. combined 3GPP service that combines services such as 6G AI service and communication service) with the requested service requirement. | PR 6.50.6-1 | AI service, performance |
| CPR Y.1.8-5-13 | Subject to operator policy, the 6G network shall be able to monitor the performance (e.g., AI model inference accuracy) and report them to the 3rd party. | PR 6.50.6-3 | AI service, performance monitor |
| Y.1.8-5-10 | Subject to operator’s policy and user preference, the 6G network shall be able to train and store AI/ML Models in the Service Hosting Environment or re-select and retrain AI/ML model (s) from the stored AI/ML models inside Service Hosting Environment upon 3rd party’s request. |  | Proposed merged CPR on AI/ML model training/retraining based 3rd party request |
| - | Subject to operator’s policy, the 6G network shall be able to store and train authorized 3rd party’s AI/ML models inside the Service Hosting Environment. | PR 6.36.6-1 | AI/ML model training/storage based on 3rd part request |
| - | Subject to operator’s policy, the 6G network shall be able to select or generate AI/ML model(s) from the stored AI/ML models inside Service Hosting Environment upon 3rd party application’s request (e.g. model type, target area, requested AI capabilities for deployment, etc.) for the application’s use. NOTE: The algorithms used to generate a new AI/ML model are out of 3GPP scope, which may include model training, model aggregation, model pruning, etc. | PR 6.36.6-2 | AI/ML model retraining of stored AI/ML in SHE for 3rd party application |
| - | Based on operator’s policy the 6G network shall be able to securely provide the trained AI/ML model between the Service Hosting Environment and the authorized 3rd party. NOTE: It is up to the 6G network to determine the AI/ML model training method, e.g. centralized or distributed. | PR 6.12.6-1 | AI/ML model training provision to 3rd party |
| CPR Y.1.8-5-114 | Based on operator’s policy the 6G network shall be able to securely store and train authorized 3rd party’s AI/ML models for authorized 3rd party inside the Service Hosting Environment based on a dataset - based on a dataset provided by the authorized 3rd party requesting AI/ML model training. - using 6G system data (e.g. 6G network data (e.g., (such as configuration data of the deployed 6G network/services, network analytics data) and 3GPP sensing data collected from wide areas, if needed.) NOTE 1: this requirement considers among others the availability of some of the above-mentioned 6G network data and 3GPP sensing data collected from wide area, when available in the Service Hosting Environment, or/and in the 6G network, necessary for training the intelligent assistance service-related AI/ML models. NOTE 2: It is up to the 6G network to determine the AI/ML model training method, e.g. centralized or distributed. | PR 6.12.6-1 PR 6.12.6-2 PR 6.36.6-1 PR 6.51.6-2 | Proposed merged CPR on AI/ML model training data set |
| - | Based on operator’s policy the 6G network shall be able to securely provide the trained AI/ML model between the Service Hosting Environment and the authorized 3rd party. NOTE: It is up to the 6G network to determine the AI/ML model training method, e.g. centralized or distributed. | PR 6.12.6-1 | AI/ML model training |
| - | Based on operator’s policy the 6G network shall support requested training for an AI/ML Model provided by an authorized 3rd party in the Service Host Environment based on a dataset provided by the authorized 3rd party requesting AI/ML model training. | PR 6.12.6-2 | AI/ML model training data set provided by 3rd party |
| - | Subject to operator’s policy, the 6G network shall be able to store and train authorized 3rd party’s AI/ML models inside the Service Hosting Environment. | PR 6.36.6-1 | AI/ML model training |
| - | The 6G network shall support suitable means to support the training of AI/ML models in the Service Hosting Environment, using 6G network data (e.g., configuration data of the deployed 6G network/services, network analytics data) and 3GPP sensing data collected from wide areas, if needed. NOTE 1: The intelligent assistance service depends on the context. For autonomous driving, the provided 6G services (e.g. communication, sensing, AI inference) support the autonomous driving application, e.g. collision avoidance, parking assistance, emergency trajectory alignment, automated intersection-crossing, etc. NOTE 3: this requirement considers among others the availability of some of the above-mentioned 6G network data and 3GPP sensing data collected from wide area, when available in the Service Hosting Environment, or/and in the 6G network, necessary for training the intelligent assistance service-related AI/ML models. | PR 6.51.6-2 | AI/ML model training data using 6G system data |
| CPR Y.1.8-5-15 | Subject to operator’s policy, the 6G network shall be able to select or generate AI/ML model(s) from the stored AI/ML models inside Service Hosting Environment upon 3rd party application’s request (e.g. model type, target area, requested AI capabilities for deployment, etc.) for the application’s use. NOTE: The algorithms used to generate a new AI/ML model are out of 3GPP scope, which may include model training, model aggregation, model pruning, etc. | PR 6.36.6-2 | AI/ML model training |
| CPR Y.1.8-5-128 | Based on operator’s policy the 6G network shall ensure required privacy protection on the training dataset used in the Service Host Environment e.g. whether the dataset is from either the 6G network or a training dataset provided by the 3rd party requesting AI/ML Model training. | PR 6.12.6-3 | Privacy protection for AI/ML model training data set, |
| CPR Y.1.8-5-136 | Subject to operator’s policy, the 6G network shall support to distribute AI/ML model to multiple Service Hosting Environments to support distributed AI/ML model training. | PR 6.25.6-4 | AI/ML model training in multiple SHE, distributed |
| CPR Y.1.8-5-147 | Subject to operator policy and regulatory requirements, the 6G network shall be able to enable a federation with one or more other 6G networks (without involving the 6G radio network) in order to enable the collaborative execution of AI/ML tasks, e.g. model training and testing. | PR 6.27.6-1 | Federated AI/ML model training using multiple 6G networks, federate |
| CPR Y.1.8-5-18 | Based on operator’s policy the 6G network shall ensure required privacy protection on the training dataset used in the Service Host Environment e.g. whether the dataset is from either the 6G network or a training dataset provided by the 3rd party requesting AI/ML Model training. | PR 6.12.6-3 | AI/ML model training, privacy |
| CPR Y.1.8-5-19 TBD | Based on operator’s policy the 6G network shall be able to expose training validation performance parameters to the authorized 3rd party to enable selection of validation performance parameters for the 6G network to use to validate the requested 3rd party AI/ML model training. | PR 6.12.6-4 | AI/ML model training, Performance |
| CPR Y.1.8-5-20 TBD | Based on operator’s policy the 6G network shall be able to expose to the 3rd party the ML model training validation performance report, for the requested 3rd party AI/ML model training. | PR 6.12.6-5 | AI/ML model training, Performance |
| CPR Y.1.8-5-21 TBD | Subject to operator’s policy, the 6G network shall be able to support to coordinate with Service Hosting Environment to minimize the latency of distributed AI/ML model training. | PR 6.25.6-5 | AI/ML model training, performance |

Table Y.1.8-6 – AI traffic characteristics

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.8-6-1 | Subject to operator’s policy, agreement with authorized 3rd party and user consent, 6G system shall be to support network shall be able to be awareness of the burst characteristics (e.g. Burst Data Rate) in traffic and provide mechanisms to optimize resource efficiency and assure user experience when handling such traffic. NOTE: Improved coordination between application on the UE and the 6G network is expected, considering GenAI traffic is usually encrypted end-to-end. For example, the application on the UE could inform the 6G network of the type of GenAI traffic (e.g., image-based, video-based, chatbot) and/or characteristics of the traffic (e.g., burst), for the 6G network to consider appropriate mechanisms to provide communication service. | PR 6.26.6-2 | Communication Service Optimization based on Traffic characteristics Awareness in the Network |
| CPR Y.1.8-6-2 | Subject to operator’s policy, agreement with authorized 3rd party and user consent, 6G network shall be able to dynamically adjust the QoS configurations to changes in traffic characteristics (e.g dynamically when the payload type changes (e.g. from text to image), or the pay load type is predicted/notified to be changed during a session) | PR 6.26.6-3 | Dynamic Communication Service Optimization based on Traffic characteristics Changes |
| CPR Y.1.8-6-3 | Subject to operator’s policy, agreement with authorized 3rd party and user consent, 6G network shall be able to accommodate the dynamic changes of traffic demand and QoS characteristics in a group of trusted third parties (e.g., multiple applications running in multiple robots that are UEs) | PR 6.52.6-1 PR 6.52.6-3 | Proposed merged CPR on Communication QoS handling for group of 3rd party AI application (e.g. 3rd AI agents) |
| CPR Y.1.8-6-3 - | The 6G network shall be able provide a suitable means with a very high efficiency and reliability, required by trusted third party (e.g., application server), to accommodate the dynamic changes of traffic demand and QoS characteristics in a group of trusted third parties (e.g., multiple applications running in multiple robots that are UEs). NOTE 1: The dynamic changes of traffic demand and QoS characteristics might be caused e.g., by using different levels of data/sensor fusion within each robot of the robot group. The changes of traffic demand include changes of AI-related traffic type (e.g., raw data, pre-processed data, AI/ML model, inference result). NOTE 2: Each robot can have multiple applications. There can be multiple intra-robot AI-related operation sessions between robot’s applications layer (e.g., “robot sensing part”, “robot processing part”, “robot actuating part”) and communications layer (e.g., “robot communication part”). Also, there can be many more inter-robot AI-related communication sessions, each of which might have different QoS characteristics and traffic demand characteristics. | PR 6.52.6-1 | 3rd party AI agent, communication, QoS handling for group of 3rd party AI application (e.g. 3rd AI agents) |
| CPR Y.1.8-6-4 - | Subject to operator’s policy, the 6G network shall be able to support dynamic QoS needed for a group of UEs (e.g., multiple robots or third party AI agents) when traffic characteristics change is predicted to occur or has occurred (e.g. change between AI/ML model transfer and AI/ML inference result transfer). | PR 6.52.6-3 | 3rd party AI agent, Ccommunication, QoS handling for group of 3rd party AI application (e.g. 3rd AI agents) |
| PR 6.28.6-1 | Subject to operator policy and agreement with the third party, 6G network shall support means for an authorized third party to provide information about expected communication performance requirement, for the 6G network to provide data transfer accordingly for the application to enhance user experience. NOTE: For example, for AI inference application based on video, such information can refer to max allowed packet error rate and average packet error rate on a per video frame basis. The max allowed packet error rate characterizes the upper bound of packer error rate of each frame to achieve successful source decoding. The average packet error rate is averaged over a number of video frames related to providing acceptable user experience. | PR 6.28.6-1 | performance |

Table Y.1.8-7 – Other

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.8-7-1 | Subject to operator’s policy and user consent, the 6G system shall provide mechanisms to efficiently collect, transfer, and process multiple types of data when providing multiple 3GPP services in response to an intent from a subscriber. | PR 6.32.6-3 | Data Could be considered as part of Y.1.5 |
| CPR Y.1.8-7-2 | Subject to operator policy and user consent, the 6G network shall be able to optimize the user experience by actively informing UE about availability of related network services based on information such as device type, device location, mobility behaviour, connection status, subscription information, etc. | PR 6.45.6-1 | performance |
| CPR Y.1.8-7-3 | Subject to operator policy and user consent, the 6G system shall support dynamic configuration of UEs and the associated 3GPP services (e.g., communication service, sensing service, computing service, AI service) to adapt to varying conditions (e.g., network conditions, application needs, resource availability) to provide optimal service operation. NOTE: The optimal service operation aims to deliver network services efficiently and effectively, maximizing user satisfaction while minimizing resource utilization. | PR 6.45.6-2 |  |