3GPP TSG SA WG 1 Meeting #112 	S1-254262r1

17-21 November 2025, Dallas, TX, USA	(revision of S1-254262)

Source:	LG Electronics

Title:	Proposed Clarification on sensing-based positioning

Draft TS/TR:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.4.2 ISAC

Document for:	Approval

Contact:	Ki-Dong Lee (kidong.lee (at) lge.com)

Abstract: In many practical cases, the meaning of relative distance estimated via 3GPP sensing-based positioning and that via conventional 3GPP-based positioning are different. The reference point of the former case is the closest point on surface (reflection point) whereas the reference point of the latter case is the antenna reference point (APR). If the object is relatively large compared to the distance (e.g., road vehicle sensing for autonomous driving), the related KPIs need to be explicitly described reflecting such distinctions. It is proposed for discussion to put a suitable clarifying note in ISAC clauses (both in the main clause and CPR clause).

Summary of Changes (into 4262r1):

- Only keep the First Part and point out which sections to put the Note in

- Simplify the Note text

1. Introduction

In V2X, UAV, service robots, and cobots scenarios utilizing 3GPP Sensing for relative distance estimation, it's essential to recognize that the measurement output differs fundamentally from traditional 3GPP Positioning. Sensing estimates the distance from a Reference Point of Sensing (RPS) on the reference vehicle to the closest surface point of the neighboring object (e.g., vehicle). Conversely, traditional Positioning estimates the distance between the Antenna Reference Points (ARP) of the respective UEs. While this distinction is negligible for distant objects (e.g., in case of distant UAV detection), it becomes critically important for high-precision, safety-critical use cases such as autonomous driving and robotic operations such as in warehouse, dysinfection site, and residential areas where robots and humans (and companion animals / service dogs) are present. This crucial difference must be clarified and reflected in the applicable requirements proposed in the Sensing aspect of this 6G study. We propose that the relative distance estimated by 3GPP Sensing is explicitly defined as the distance from the RPS to the sensed object's surface.

The requirements for Sensing accuracy and resolution (e.g., range resolution in centimeters) are significantly tighter than traditional positioning. This high precision is meaningless if the target point (surface vs. antenna) is not explicitly defined.

For proximity-sensitive, safety-critical use cases such as autonomous driving V2X, the inherent difference in measurement output between 3GPP Sensing and 3GPP Positioning is vital. 3GPP Sensing aims to estimate the distance to the physical surface of a neighboring object, which is crucial for collision avoidance and planning. In contrast, traditional 3GPP Positioning estimates the distance to the neighboring UE's Antenna Reference Point (ARP). When the relative distance is small, the offset between the object's surface and the ARP is not negligible and must be accounted for by the Sensing requirements.

2. Reason for Change

See below.

3. Conclusions

See below.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.4.1.

* * * First Change (#1/3) * * * *

### 7.14.6	Potential New Requirements needed to support the use case

New functional requirements for this use case are:

[PR 7.14.6-1] Subject to operator policies, the 6G Network shall provide mechanisms for configuring a sensing operation with a single or multiple sensing modes from all sensing modes supported (e.g. bistatic, monostatic, multistatic).

NOTE:	Mechanisms for sensing operation configuration are in scope of ongoing 5GA work, without explicit 5GA service requirements.

[PR 7.14.6-2] The 6G Network shall provide suitable mechanisms for the exposure of sensing results in a synchronised manner with other types of traffic (e.g. audio, video, haptics) to the sensing service consumer.

New performance requirements for this use case are:

[PR 7.14.6-3]	The 6G system shall be able to support the following KPIs:

Table 7.14.6-1: Proposed KPIs for collaborative robots using digital twinning

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Collaborative Robots (NOTE1) (Note X) | Indoor/Outdoor | [95] | [≤ 0.1] | [≤ 0.1] |  |  |  |  | N/A |  |  |  |
| NOTE Related use cases are captured in clause 7.2 of TR 22.837 [9] and clause 6.2 of TS 22.137 [6]. NOTE X: The relative distance measurement obtained through 3GPP Sensing is referenced to the surface of the sensed object (e.g., the closest physical boundary of a neighboring vehicle, robot, or some obstacle) from the Reference Point of Sensing (RPS) on the reference entity (e.g., regular UE, vehicle, and robot). |  |  |  |  |  |  |  |  |  |  |  |  |

* * * First Change (#23) * * * *

### 7.16.6	Potential New Requirements needed to support the use case

[PR 7.16.6-1] Subject to regulatory requirements, operator’s policy, and user consent, the 6G network should support suitable means to collect non-3GPP sensing data from third party.

[PR 7.16.6-2] The 6G system shall be able to provide sensing service by enabling multi-sensor fusion based sensing as described in the following Table 7.16.6-1.

Table 7.16.6-1: Performance requirements for Multi-Sensor Fusion based sensing for UAV takeoff and landing

| Scenario | Object Type | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Spatial resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| UAV takeoff and landing (Note Y) | Outdoor | [95] | [0.1] (note 1) | [0.1] (note 1) | [1] | [1] | [0.5] | [1] | [100-500] | [0.2] | [5] | [5] |
| NOTE 1: The KPI values for accuracy of positioning are centimetre-level sourced from reference [199], [200] NOTE 2: The KPI values of this use case are effective for sensing in specific area for UAV takeoff and landing. NOTE Y: The relative distance measurement obtained through 3GPP Sensing is referenced to the surface of the sensed object (e.g., the closest physical boundary of a neighboring vehicle, robot, or some obstacle) from the Reference Point of Sensing (RPS) on the reference entity (e.g., regular UE, vehicle, and robot). |  |  |  |  |  |  |  |  |  |  |  |  |

* * * First Change (#3/3) * * * *

### 7.24.6	Potential New Requirements needed to support the use case

The following requirements are necessary abilities for recognizing relevant body language in industrial environments.

[PR 7.24.6-1]	Subject to regulatory requirements and user consent, the 6G network shall be able to use the 6G sensing service to monitor and recognize gestures of a worker such as hand gestures, body and limb positions, head positions, and facial expressions of a worker.

NOTE:	The sensing resolution is proportional to the accuracy of recognition. No angular resolution for the sensor is given, since it depends on the distance from the subject. A combination of different sensors could be implemented to meet these requirements.

[PR 7.24.6-2]	The 6G System shall support the sensing KPIs for gesture recognition in industrial environments as provided in Table 7.24.6-1.

Table 7.24.6-1: KPIs for gesture recognition in industrial environments [94]

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] (note 1) | False alarm [%] (note 1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Hand Gestures Recognition (Note Z) | Indoor (Factory Environment) | 99 | [0.1-0.3] | [0.1] | n/a | n/a | [≤0.02] | n/a | ≤20 | [≤0.02] | ≤ 1% | ≤ 10% |
| Facial Gestures Recognition (Note Z) | Indoor (Factory Environment) | 99 | [0.1] | [0.1] | n/a | n/a | [≤0.002] | n/a | ≤20 | [≤0.02] | ≤ 1% | ≤ 10% |
| Head Gestures Recognition (Note Z) | Indoor (Factory Environment) | 99 | [0.1] | [0.1] | n/a | n/a | [≤0.05] | n/a | ≤20 | [≤0.02] | ≤ 1% | ≤ 10% |
| Body and Limb Gestures Recognition (Note Z) | Indoor (Factory Environment) | 99 | [0.1] | [0.1] | n/a | n/a | [≤0.1] | n/a | ≤20 | [≤0.02] | ≤ 1% | ≤ 10% |
| NOTE 1: Depending on the actual application to be controlled by the gestures, Missed detection and False alarm may have different KPI values from < 1% to 10%. NOTE 2: References for hand gesture recognition are [331][332], for facial and head gesture recognition [333], and for body and limb gesture recognition [334][335]. NOTE Z: The relative distance measurement obtained through 3GPP Sensing is referenced to the surface of the sensed object (e.g., the closest physical boundary of a neighboring vehicle, robot, or some obstacle) from the Reference Point of Sensing (RPS) on the reference entity (e.g., regular UE, vehicle, and robot). |  |  |  |  |  |  |  |  |  |  |  |  |

* * * Second Change * * * *

(Moderator’s text from S1-254254)

/* none */

* * * Next Change * * * *

/* none */