3GPP TSG SA WG 1 Meeting #112 	S1-254266r1

17-21 November 2025, Dallas, USA	(merge of S1-254122)

Source:	Nokia (Moderator), CATT

pCR Title:	Pseudo-CR on consolidated performance requirements for Ubiqiutous Connectivity

Draft Spec:	3GPP 22.870

Agenda item:	8.1.10

Document for:	Approval

Contact:	feifei.lou@nokia.com

Abstract: this contribution provides the consolidation of performance requirements in clause 8 ubiquitous connectivity.

1. Introduction

The consolidated performance requirements for clause 8 are categorised into two parts.

The consolidated performance requirements for satellite-based positioning services covers the following use cases and PRs. The scenarios in the KPI table are sorted from less demanding to more demanding cases.

- [PR 8.5.6-1] from use case on resilient positioning in satellite networks

- [PR 8.7.6-2] from use case on low-energy positioning in satellite networks

- [PR 8.10.6-1] from use case on hybrid TN and NTN positioning

- [PR 8.11.6-2] from use case on hybrid NTN and GNSS positioning

The consolidated performance requirements for satellite access covers the following use cases and PRs.

- [PR 8.4.6-2] from use case on service continuity for wearable mobile devices

- [PR 8.6.6-2] from use case on disaster relief

- [PR 8.8.6-2] from use case on global mobile video

- [PR 8.9.6-2] from use case on low-altitude logistics supported by NTN

The following PR is not consolidated due to FFS.

- [PR 8.2.6-1] from use case on ubiquitous and resilient network

2. Reason for Change

To provide the consolidation of performance requirements in clause 8 ubiquitous connectivity.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.4.0.

* * * First Change * * * *

All new texts

## 8.x	Consolidated performance requirements for Ubiquitous Connectivity

### 8.x.1	Consolidated performance requirements for satellite-based positioning services

Table 8.x.1-1: Consolidated performance requirements for satellite-based positioning services

| Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |  |
| Containers | [100] m | N/A | [95] % | N/A | Outdoor | Up to [37] km/h | Container mounted | Battery life expectancy (note 1) 12 years |
| Pallets | [100] m | N/A | [95] % | N/A | Outdoor | Up to [100] km/h | Vehicle mounted | Battery life expectancy (note 1) 7 years |
| Wagons | [100] m | N/A | [95] % | N/A | Outdoor | Up to [350] km/h | Train mounted | Battery life expectancy (note 1) 20 years |
| Airplane en-route (note 2) | [50] m | [50] m | [99] % | [1] s | Outdoor | Up to [1500] km/h | Airplane mounted | - |
| Airplane landing (note 2) | [10] m | [10] m | [99] % | [1] s | Outdoor | Up to [350] km/h | Airplane mounted | - |
| UE with partly obstructed sky | [10] m | [3] m | [95] % | [1] s | Outdoor rural and urban | Up to [250] km/h | Handheld, vehicle mounted or IoT | - |
| Maritime | [3] m | [3] m | [99] % | [1] s | Outdoor rural | Up to [500] km/h | Maritime mounted | - |
| UAV facility monitoring (note 3) | [3] m | [3] m | [99] % | [0,1 – 0,5] s | Outdoor | Up to [160] km/h | UAV mounted | - |
| UAV positioning (note 2) | [1] m | [1] m | [99] % | [1] s | Outdoor | Up to [160] km/h | UAV mounted | - |
| Airplane taxiway (note 3, 4) | [1] m | N/A | [99] % | [0,1 – 0,5] s | Outdoor | Up to [100] km/h | Airplane mounted | - |
| NOTE 1: Battery life expectancy is to be assumed in all coverage conditions and is based on typical message size value and typical frequency, e.g. the number of messages per day by a typical user. NOTE 2: Positioning services are provided with 3GPP technologies, independently of non-3GPP positioning technologies (e.g. GNSS). Multiple satellites can be used to support 3GPP positioning technologies. NOTE 3: Requirements for Airplane taxiway and UAV facility monitoring are in [25]. NOTE 4: Airplane taxiway refers to an airplane on ground during taxi operations. |  |  |  |  |  |  |  |  |

Editor’s Note: maritime mounted UE speed is FFS.

### 8.x.2	Consolidated performance requirements for satellite access

Table 8.x.2-1: Consolidated performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Peak data rate (DL) | Peak data rate (UL) | Area traffic capacity (DL) | Density | Reliability | Service availability | End-to-end latency | UE speed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Disaster relief (note 1) | N/A | N/A | Up to [20] Mbit/s | Up to [2] Mbit/s | N/A | Up to [100] devices/km2 for SMS only | [99,9 –99,999] % | Up to 99,9 % | Up to [600] ms | 3 km/h |
| Remote area (note 2) | [5] Mbit/s | [1] Mbit/s | N/A | N/A | Up to [50] kbit/s/km2 (note 3) | [< 1] person/km2 | N/A | N/A | N/A | N/A |
| Deep Rural area (note 2) | [10] Mbit/s | [2] Mbit/s | N/A | N/A | [1] Mbit/s/km2 (note 4) | [1-10] person/km2 | N/A | N/A | N/A | N/A |
| NOTE 1: Positioning accuracy is low (≈ [100] m). Positioning service availability is up to [99] %. Positioning service latency is [1] s. NOTE 2: Reduced video quality is acceptable in the remote areas and deep rural area, see chapter 3.9.1.3 and 3.9.1.5 in [31]. NOTE 3: The DL area traffic capacity corresponds to an activity factor of 2 % with an average user density of 0.5 person/km2. NOTE 4: The DL area traffic capacity corresponds to an activity factor of 4 % with an average user density of 5 persons/km2 equally shared by two operators. The area traffic is here split between two operators, with [2] Mbit/s/km2 in total. NOTE 5: UE type is handheld. |  |  |  |  |  |  |  |  |  |  |

[CPR 8.x.2-1]: The 6G system with satellite access, shall support SMS delivery to a high density of UEs, i.e. up to [1000] devices/km2.

[CPR 8.x.2-2] The 6G system with satellite access shall be able to support communication service for UEs (e.g. UAV) at the altitudes from 0 to 3 km via satellite access and/or terrestrial access.

* * * End of changes * * * *