3GPP TSG SA WG1 Meeting #112                                                	   S1-254274r1

17-21 November 2025, Dallas, USA	 (revision of S1-253319)

Source:	Philips International B.V., Verizon

pCR Title:	Resubmission of update clause 5.7.1 on FWA

Draft Spec:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.2.2

Document for:	Approval

Contact:	Walter Dees (walter.dees@philips.com)

Abstract: This pCR proposes to update clause 5.7.1 in TR 22.870 with additional aspects related to having multiple FWA devices and adding service flows, pre- and postconditions to fit the use case template.

1. Introduction

Clause 5.7.1 describes the benefits of Fixed Wireless Access. This use case describes the benefits of supporting multiple FWA devices. This is a resubmission of use case S1-253319, wherein we provide additional details on the merits of supporting multiple FWA devices to address comments that were made during last meeting.

2. Reason for Change

In order to have good wireless network performance within people’s homes a single wireless access point built into or otherwise attached to a Fixed Wireless Access device, i.e. FWA Consumer Premise Equipment (CPE), is typically not sufficient, since good wireless coverage across all areas of people’s homes is difficult to achieve.  Thus, it would be beneficial for a user to employ multiple FWA CPEs to increase coverage within people’s homes and e.g. backyard or in a multi-floor house, but also to provide better performance in terms of capacity, reliability, or Quality of Experience since the load can be shared amongst the multiple FWA devices. This would also align with recent Wi-Fi developments that provide support for multi-AP wherein multiple AP cooperate when serving a wireless device.

3. Proposal

Update clause 5.7.1  with additional aspects related to having multiple FWA CPEs, for inclusion in 3GPP TR 22.870.

* * * First Change * * * *

### 5.7.1	Fixed wireless access (FWA)

#### 5.7.1.1	Description

Fixed Wireless Access (FWA) [359] in 5G NR has transformed the Home Internet business and has become a significant revenue generator for MNOs. It is expected that FWA services will continue to grow in 6G. While FWA has been supported since LTE, 5G's speed and flexibility has enabled it to emerge as a transformative and successful 5G service. FWA has become a key strategy in expanding high-speed broadband services to previously unserved or underserved areas (i.e. a tool to bridge the Digital Divide), for in-building services, and enterprise (i.e. private network) deployments.

Some initial deployments of FWA used "fallow capacity" in markets where eMBB usage was not in high demand, but the underlying network protocols, procedures, and resource management schemes of the macro networks are designed for eMBB services and Smartphone devices that require mobility support. FWA devices (i.e. FWA Consumer Premise Equipment (CPE)) have a very different mobility profile, traffic usage pattern, and are able to take advantage of higher output power improving their in-building link performance.

FWA should provide a quantitative improvement in service by using 6G services and capabilities (e.g. AI/ML).

A Customer Premise Network (CPN) may benefit of access through multiple FWA CPEs as means to improve coverage, capacity, and reliability. This is of particular relevance for larger deployments, such as apartment buildings with a shared wireless infrastructure, multi-floor concrete homes, family compounds, homes with a garden/summer house, condos, multi-dwelling homes, and also for office buildings, shops or factories, etc. It may also be an opportunity for operators to develop new multi-FWA subscription models (e.g. using multiple SIMs). For example:

•	Additional FWA CPEs will improve coverage in the whole CPN.

•	Additional FWA CPEs allows increasing the capability and QoS of UL/DL traffic as required in the CPN. By using load balancing between FWA CPEs, data traffic to/from multiple devices connected to the FWA CPE, in particular devices that are in coverage of both FWA CPEs, higher and more consistent throughput can be achieved. Also traffic overload situations e.g. by one of the FWA CPEs can be managed better in this way.

This is illustrated by means of Fig. 5.7.1.1-1 wherein the CPN is represented by means of a home network comprising five devices D1 to D5. The five devices are served by two FWA CPEs, namely FWA CPE1 and FWA CPE2. The two FWA CPEs cooperate to provide connectivity to the CPN. For example, FWA CPE1 could serve devices D1 and D2. FWA CPE2 could serve devices D5. Device D3 may connect via both FWA CPE1 and FWA CPE2 whereby FWA CPE1 and FWA CPE2 could coordinate between themselves (e.g. by sending via the network to the other FWA CPE coordination information about D3, e.g. how to divide the traffic load of D3 between FWA CPE1 and FWA CPE2). Similarly, device D4 may connect to FWA CPE2 which may be overloaded with traffic from device D5, so FWA CPE2 could communicate via the network with FWA CPE1 that it is better to serve D4 by FWA CPE1 even though FWA CPE2 is closer.

Hence, if FWA CPE2 were not present, the connection to, e.g., D3 or D4 or D5, would be potentially much worse (e.g., in terms of capacity or latency), However, the usage of multiple residential gateway units allows improving coverage, increasing capacity, improving Quality of Service, and improving the reliability of the system. This use case assumes that both FWA CPEs connect to the same PLMN.

Fig. 5.7.1.1-1 – Usage of multiple FWA CPEs for increased coverage, capacity, and reliability in a customer premises network

#### 5.7.1.2	Pre-conditions

Operator A offers FWA services to provide connectivity in homes.

User A subscribes to a Fixed Wireless Access of Operator A and receives a FWA CPE1 from Operator A.

#### 5.7.1.3	Service Flows

User A installs and registers his FWA CPE (FWA CPE1) with Operator A. If not yet preconfigured, this may involve installing a USIM (USIM1) to FWA CPE1.

User A places FWA CPE1 in/near the living room of the ground floor near the garden. FWA CPE1 provides connectivity to multiple smart devices in the home network. User A and his family enjoy great connectivity when making video calls or watching movies in the smart TV. FWA CPE1 provides an optimized high-speed broadband connection with the network.

User A decides to go to work in his office on the first floor above the garage. User A uses the office for working purposes and online computer gaming after work, where gaming requires very low latency. Regrettably, connectivity speed to devices D4 and D5 in the first floor is slower, latency is higher, and QoS is variable.

User A orders a second FWA CPE (FWA CPE2) from Operator A and the corresponding USIM, USIM2.

User A installs the USIM (USIM2) to FWA CPE2, if the FWA CPE2 was not yet preconfigured. User A installs FWA CPE2 in his office.

#### 5.7.1.4 	Post-conditions

User A and his family enjoy of great FWA connectivity in the whole house and their garden.

#### 5.7.1.5	Existing features partly or fully covering the use case functionality

TS 22.261 [14] includes a number of requirements related to residential gateways, e.g. in clause 6.3.2.4:

The 5G system shall be able to efficiently support connectivity using fixed broadband access.

NOTE: The specification of fixed broadband access network is outside the scope of 3GPP.

The 5G system shall support use of a relay UE that supports multiple access types (e.g. 5G RAT, WLAN access, fixed broadband access).

and clause 6.38:

Subject to regulatory requirements and operator policy, the 5G system shall support an efficient data path within the CPN for intra-CPN communications.

NOTE 1:	For services an operator deploys in the 5G network (i.e. not in the CPN), local data routed via eRG does not apply.

The 5G system shall minimize service disruption for a UE that is moving between CPN access and operator provided mobile access.

NOTE 3:	CPN access can imply access via a PRAS or can imply access directly via an eRG. Operator provided mobile access implies access via an operator owned base station.

The 5G system shall be able to support access to the 5G network and its services via at least one gateway (i.e. PIN Element with Gateway Capability or eRG) for authorised UEs and authorised non-3GPP devices in a PIN or a CPN.

The 5G system shall be able to support IP traffic offload to data network for a CPN.

NOTE 1: The priority of offload can be from default configuration, network or user.

The 5G system shall support real time E2E QoS monitoring and control for any intra-CPN data traffic to or from a UE (i.e. via eRG or via PRAS and eRG).

The 5G system shall support real time E2E QoS monitoring and control for any data traffic between a UE within a CPN and the 5G network (i.e. via eRG or via PRAS and eRG).

These requirements do not address optimizing connectivity for FWA, support for multiple FWA CPEs and load balancing.

#### 5.7.1.6	Potential New Requirements

[PR 5.7.1.6-1] The 6G system shall provide optimized network capabilities for FWA (e.g. support stationary devices).

[PR 5.7.1.6-2] The 6G system shall support FWA in relevant bands taking into consideration the regulatory requirements for each specific band.

[PR 5.7.1.6-3] The 6G system shall enable the means to provide awareness of user service characteristics (e.g. data rate, latency) to support the RAN and CN in making real time resource allocation for FWA.

[PR 5.7.1.6-4] The 6G system shall provide FWA a comparable level of security and privacy protection to other 6G services.

[PR 5.7.1.6-5] The 6G system shall support efficient FWA connectivity.

Editor's Note: this requirement is FFS.

NOTE:	For FWA, an FWA Customer Premises Equipment (CPE) is used to connect to the network, like any other UE, using a 3GPP access.

[PR 5.7.1.6-6] The 6G system shall be able to support the use of multiple FWA CPEs in a large CPN for the same PLMN in order to enhance coverage, service availability, and service reliability.

[PR 5.7.1.6-7] The 6G system shall be able to support a device to seamlessly switch between multiple FWA CPEs offering FWA connectivity within a CPN for the same PLMN.

* * * End of Change * * * *