3GPP TSG SA WG 1 Meeting #112 	S1-254281r1

Dallas, USA, 17 -21 November 2025	(revision of S1-254034)

Source:	TNO, Airbus, ESA, Fraunhofer, Thales, Novamint

Title:	Resubmission of a use case on NTN based Sensing of space debris

Draft Spec:	3GPP TR 22.870 V0.4.1

Agenda item:	8.1.4.2 Resubmission of Use Cases and others

Document type:	pCR

Document for:	Discussion/Approval

Contact:	relja.djapic@tno.nl

Abstract: This document resubmits a use case on non-terrestrial network based integrated sensing and communication (NTN ISAC) for sensing of space debris.

1. Introduction

Integrated Sensing and Communication (ISAC) is an important element of 6G as reflected in Chapter 7 of TR 22.870. However, the use cases so far only refer to the use of terrestrial networks (TN) for ISAC. Authors in ([X1], [X2], [X3]) prove the feasibility of ISAC based on non-terrestrial networks (NTN). In additoin, [X4] and [X5] develop satellite processing/computing payload to support NTN based ISAC. Accordingly, it becomes apparent that the field of ISAC from space is rapidly emerging  and that non-terrestrial network (NTN) can support integrated sensing application.

This contribution proposes a new use case on Sensing of space debris with NTN, i.e., NTN based space debris sensing using ISAC.

2. Reason for Change

Ensure that the capabilities of NTN based Integrated Sensing and Communications (NTN ISAC), will be supported by 6G networks.

3. Conclusions

Add a new use case on NTN based Integrated Sensing and Communications (NTN ISAC) in TR 22.870.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 V0.4.1.

* * * 1st Change * * * *

# 2	References

[X1]	Matera et al, “Terrestrial and Non-Terrestrial Networks for Integrated Sensing and Communication”, International Symposium on Systems Engineering ISSSE 2024, Oct 2024.

[X2]	Y. Gu et al, "ISAC towards 6G Satellite–Terrestrial Communications: Principles, Status, and Prospects", 2024, Electronics 13, no. 7: 1369, https://doi.org/10.3390/electronics13071369.

[X3]	L. You et al, “Ubiquitous Integrated Sensing and Communications for Massive MIMO LEO Satellite Systems”. Integrating sensing and communication for ubiquitous internet of things, IEEE Internet of Things Magazine, Volume: 7, Issue: 4, July 2024

[X4]	M. Corici et al. "Repurposable payloads: concept, design, and initial results," 41st International Communications Satellite Systems Conference (ICSSC 2024), Seattle, USA, 2024, pp. 120-124, doi: 10.1049/icp.2024.4622.

[X5]	ESA Project 5GEOSiS – 5G Earth Observation Server in Space: https://connectivity.esa.int/projects/5geosis-–-5g-earth-observation-server-space

[X6]	C. K. Sheemar et al., "Joint Communication and Sensing for 6G Satellite Networks: Use Cases and Challenges"    https://arxiv.org/abs/2501.05243v2

[X7]	L. Yin, Z. Liu, M. R. B. Shankar, M. Alaee-Kerahroodi and B. Clerckx, "Integrated Sensing and Communications Enabled Low Earth Orbit Satellite Systems," in IEEE Network, vol. 38, no. 6, pp. 252-258, Nov. 2024, doi: 10.1109/MNET.2024.3371172

[X8]	Antti Anttonen, Markku Kiviranta, Marko Höyhtyä, ”Space debris detection over intersatellite communication signals”, Acta Astronautica, Volume 187, 2021, Pages 156-166, https://doi.org/10.1016/j.actaastro.2021.06.023.

[X9]	 H. Dong and O. B. Akan, "DebriSense: THz-based Integrated Sensing and Communications (ISAC) for Debris Detection and Classification in the Internet of Space (IoS)," in IEEE Transactions on Wireless Communications, doi: 10.1109/TWC.2025.3572276

* * * 2nd Change * * * *

## 7.x	Use case on Sensing of space debris with NTN

### 7.x.1	Description

Minimization of the effects created by space debris as well as reduction thereof are critical aspects in achieving sustainability of the space economy and also for deployment and operation of Non-Terrestrial Networks. NTN based Integrated Sensing and Communication (NTN ISAC) as described in [X1, X2, X3, X4, X5] has the potential to detect and track space debris to ensure the continued safe operation of all space assets [X6]. Exploitation of NTN signals for debris detection is essential to prevent destruction/damage to satellites and the corresponding effect operation of non-terrestrial networks.

Monitoring the downlink NTN channels (from satellite towards the end users) for spurious reflections or disturbances represents the basis of NTN ISAC. This approach allows for detection of space debris in the area below the satellite in which NTN radio signal exists. Accordingly, NTN ISAC for space debris detection is applicable for all orbital altitudes, i.e., LEO, MEO and GEO where NTN signal exists with the detection range (few up to tens of kilometers [X6]) being limited to the vicinity of the individual satellite. In addition, sensing of space debris could also exploit NTN signals between a ground station and the satellite. Depending on the signal used (originating from satellite and/or ground station) the sensing can exploit monostatic or bistatic passive radar principles. An optimized ISAC approaches that focuses to achieve a satisfactory trade-off between space debris sensing and communications is proposed in [X7].

Regarding tracking of the detected object, it is critical to have on-board intelligence for signal processing and detection of target. Depending on detailed measurement scheme, parameters of target movement such as velocity and flight direction should be estimated and shared with a satellite following in the same NGSO orbit for tracking. Therefore, such use case will require high-speed processing and on-board intelligence possibly exploiting AI machine learning. In line with this, to support space debris sensing, 6G system should support: satellite onboard data collection, satellite onboard computing and data fusion (as proposed in [X8]). Keeping sensing and data processing in space allows endangered satellites to react promptly (e.g., slightly adapt their paths) in avoiding the sensed space debris. Using different carrier frequencies and bandwidths for non-terrestrial network communications will enable increased precision of the space debris sensing from the perspective of the range of sensing (using lower frequencies) and its resolution (broader spectral bandwidths). Mobility of the satellites used for sensing can be compensated for, based on the known predetermined ephemeris. The collected sensed data might be used by the end users in building a digital twin of the objects in the space. Feeder links should be designed in such a way to support (transfer of the sensed/processed data) towards a data center on the ground.

Estimation of debris position in orbit can be improved by involving multiple satellites in LEO, e.g., through triangulation measurements. Use of high frequency signal source like Ka-band is necessary for the detection of small debris (< 5cm) with better results, i.e. high probabilities of detection,  being achieved by even higher frequencies [X8] up into the THz-regime [X9]. Besides, increased performance of space debris sensing is possible to achieve by combining NTN ISAC with other sensing techniques (e.g., infrared radar, detection of electromagnetic signature of space debris, optical sensors). A combined sensing approach reduces the probability of missed detection and increases reliability and accuracy of the sensed data (e.g., position, velocity).

### 7.x4.2	Pre-conditions

Satellite Network Operator B (SNOB) constantly collects sensing data on space debris from its own NTN assets and can possibly also use other satellites for bistatic monitoring.

SNOB exploits collected sensing data to provide 6G NTN based “smart space debris detection and tracking” to other satellite operators, service providers or authorities engaged in Space Traffic Surveillance and Management.

Earth Observation Constellation Operator (EOCO) subscribes to smart space debris detection and tracking service provided by SNOB.

### 7.x4.3	Service Flows

- Company EOCO operates a constellation of Earth observation satellites. EOCO possesses several satellites with hi-tech equipment on-board that is sensitive to kinetic impacts. To protect the assets EOCO satellites are equipped with thrust engines to avoid space debris.

- Satellite network operator B (SNOB) exploits its 6G NTN to provide communication and sensing services. EOCO is subscribed to SNOB’s space debris sensing services. SNOB alerts EOCO on any space debris that could potentially collide with and damage EOCO satellites.

- Based on the acquired information on potentially dangerous space debris EOCO evaluates the threat and commands an avoidance maneuver if/when needed.

### 7.x4.4	Post-conditions

Collision between space debris and EOCO satellites as well as potential follow-up impacts (like a Kessler syndrome for this orbit) has been avoided. EOCO constellation continues safe operation and keeps providing observation data to its customers.

### 7.x4.5	Existing features partly or fully covering the use case functionality

None.

### 7.x4.6	Potential New Requirements needed to support the use case

[PR 7.X.6-1] Subject to operator’s policy and agreement with 3rd party, the 6G system shall support detection and tracking of space debris at altitudes between 300 and 2000km..

[PR 7.X.6-2] The 6G system shall support sensing services with KPIs as given in following table:

Table 7.x.6-1	Performance requirements sensing of space debris using NTN [X8]

| Scenario |  | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[s] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Sensing of space debris by NTN | Large satellite debris (4 m x 1 m x 1 m) | Space | 99.9 | 10 NOTE 1 | 10 NOTE 1 | 10 NOTE 1 | 10 NOTE 1 | 10 | 0.5x0.5 | 1 | 1 application configurable | <1 | <1 |
|  | Cube sat debris (30 cm x 10 cm x 10 cm) NOTE 2 | Space | 95 | 10 NOTE 1 | 10 NOTE 1 | 10 NOTE 1 | 10 NOTE 1 | 10 | 0.5x0.5 | 1 | 1 application configurable | <5 | <5 |
|  | Small space debris (2 cm x 2 cm x 1 cm) NOTE 2 | Space | 90 | 10 | 10 | 10 | 10 | 10 | 0.5x0.5 | 1 | 1 application configurable | <5 | <5 |
| NOTE 1 : The objective of space debris measurements is a precise determination of the orbit, therefore position and velocity are correlated and accuracies can be traded off against each other. Multiple measurements on the same piece of space debris can significantly enhance precision. NOTE 2: Detecting and tracking small space debris is challenging, but nevertheless necessary. Missed detection percentage could be reduced by combining NTN ISAC with other sensing techniques (e.g., radar, detection of debris’ electromagnetic signature, optical sensors). Note that the Cube sat and Small space debris can potentially create damage to a satellites while Large satellite debris can cause destruction of operational satellites. Accordingly, the percentage of missed detection is smaller in case of Large satellite debris. |  |  |  |  |  |  |  |  |  |  |  |  |  |